3GPP TSG-RAN WG4 Meeting #97-e 	R4-2015092

Electronic Meeting, 2-13 Nov., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 HST BS demodulation PUSCH simulation results

Agenda item:	7.15.3.2.2

Document for:	Discussion

Introduction

All previously known open issues on the topic of NR Rel-16 HST BS demodulation PRACH requirements have been resolved at the RAN4#96e meeting. Results are captured in the email discussion summary [1], and WF [2].

Since the last meeting, we have not identified further topics that still need discussion.

This contribution contains the results of our remaining simulation campaigns (long format restricted sets).

Simulation results

In RAN4#96-e it was decided to include requirements for format 0 with restricted sets under multi-path fading [2]:

- TDLC300-100 propagation conditions for long preamble formats - Introduce TDLC300-100 FO= 0Hz for PRACH with restricted set type A and B. - In Rel-16, no additional test cases for PRACH besides what we already agreed test case. - The test cases applied for BS if BS declared supporting corresponding restricted set A/B

Following this we update the previously delivered simulation results.

Parameters

Unless otherwise stated in the following tables, the simulation setup follows the HST PRACH baseline as captured in [4] and [5], with the update from [2]:

Table 1: PRACH HST baseline configuration

| Parameter | Value |  |
| --- | --- | --- |
|  | v = 350km/h |  |
| Frequency range | FR1 |  |
| Number of Tx | 1 |  |
| Number of Rx | 2Rx, 4Rx, 8Rx |  |
| Formats | 0 |  |
| SCS | 1.25kHz |  |
| Restricted set format 0 | {Set A, Set B} |  |
| (Ncs, logical sequence index, v) | Set A: (15, 384, 0) Set B: (15, 30, 30) |  |
| Max timing error | AWGN: 1.04us TDLC300: 2.55us |  |
| Maximum Doppler shift | Set A: {1340 Hz, 625 Hz} Set B: {2334 Hz, 625Hz} |  |
| Carrier frequency (GHz) | Set A: 2.1 GHz Set B: 3.6 GHz |  |
| Propagation condition | {AWGN with max Doppler shift from above, TDLC300-100 FO= 0Hz} |  |
| Testing metric | SNR @ 0.1% false alarm probability and 99% detection probability. |  |

The simulation results collection template [3], erroneously captures the carrier frequency for set B as 2.1GHz (instead of 3.6GHz). This can be checked as a source of error in case of observed misalignment but is currently not expected to be a cause of concern.

  - PRACH 1.25kHz SCS Type A

Table 2: PRACH 1.25kHz SCS, restricted set Type A

| Format | SCS | Ncs | Logical sequence index | v | Tx/Rx | fc (GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impaired |
| Format 0 | 1.25kHz | 15 | 384 | 0 | 1T2R | 2.1 | TDLC300-100 | 0 | 2.55us | -7.87 | -5.37 |
| Format 0 | 1.25kHz | 15 | 384 | 0 | 1T4R | 2.1 | TDLC300-100 | 0 | 2.55us | -13.14 | -10.64 |
| Format 0 | 1.25kHz | 15 | 384 | 0 | 1T8R | 2.1 | TDLC300-100 | 0 | 2.55us | -17.05 | -14.55 |

  - PRACH 1.25kHz SCS Type B

Table 1: PRACH 1.25kHz SCS restricted set Type B

| Format | SCS | Ncs | Logical sequence index | v | Tx/Rx | fc (GHz) | Propagation condition | Frequency offset (Hz) | Timing error tolerance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impaired |
| Format 0 | 1.25kHz | 15 | 30 | 30 | 1T2R | 2.1 | TDLC300-100 | 0 | 2.55us | -7.36 | -4.86 |
| Format 0 | 1.25kHz | 15 | 30 | 30 | 1T4R | 2.1 | TDLC300-100 | 0 | 2.55us | -12.35 | -9.85 |
| Format 0 | 1.25kHz | 15 | 30 | 30 | 1T8R | 2.1 | TDLC300-100 | 0 | 2.55us | -16.33 | -13.83 |

Conclusion

In this contribution we have provided our simulation results for HST PRACH restricted sets under fading propagation conditions.
We have made one observation:

- The simulation results collection template [3], erroneously captures the carrier frequency for set B as 2.1GHz (instead of 3.6GHz). This can be checked as a source of error in case of observed misalignment but is currently not expected to be a cause of concern.

- References

- R4-2012739, Email discussion summary for [96e][323] NR_HST_Demod_BS, Moderator (Nokia, Nokia Shanghai Bell), RAN4#96-e.

- R4-2012676, WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- R4-2012749, Summary of ideal and impairment results for NR HST demodulation requirements, CATT, RAN4#96-e.

- R4-1915871, Way forward on NR HST PRACH requirements, Samsung, RAN4#93.

- R4-1912729, Way forward on NR HST PRACH demodulation, Samsung, RAN4#92-Bis.