3GPP TSG-RAN WG4 Meeting #97-e              	R4-2015120

Electronic Meeting, 2nd Nov– 13th Nov, 2020

Agenda item:	7.15.3.2.2

Source:	Samsung

Title:	Simulation results for NR HST PRACH

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issues for NR HST BS PRACH demodulation requirement were discussed and test parameters for related requirement were captured in the WF [1].

In this contribution, the simulation results for HST PRACH are provided for performance derivation.

# 2	Simulation Results

In the last meeting, it was agreed to introduce TDLC300-100 FO=0Hz for PRACH with restricted type A and type B. requirement. In this section, the initial results for NR HST PRACH is provided as table 1.

Table 1: Simulation results for NR HST PRACH with restricted type A and type B (to be updated)

| Format | Restricted set type | Ncs | Ant | Logical sequence index | v | Propagation on condition | FO | SNR (ideal) | SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A | 15 | 1T2R | 384 | 0 | TDLC300-100 | 0Hz | -8.38 | -6.38 |
| 0 | A | 15 | 1T4R | 384 | 0 | TDLC300-100 | 0Hz | -13.84 | -11.84 |
| 0 | A | 15 | 1T8R | 384 | 0 | TDLC300-100 | 0Hz | -18.18 | -16.18 |
| 0 | B | 15 | 1T2R | 30 | 30 | TDLC300-100 | 0Hz | -8.25 | -6.25 |
| 0 | B | 15 | 1T4R | 30 | 30 | TDLC300-100 | 0Hz | -13.74 | -11.74 |
| 0 | B | 15 | 1T8R | 30 | 30 | TDLC300-100 | 0Hz | -18.08 | -16.08 |

# 3	Conclusion

In this contribution, the simulation results for NR HST PRACH are provided for requirement derivation.

# References

R4-2012676 WF on Rel-16 NR HST BS demodulation requirements, Nokia