3GPP TSG-RAN WG4 Meeting #97-e              	R4-2015125

Electronic meeting, 2nd Nov – 13th Nov, 2020

Agenda item:	7.18.3

Source:	Samsung

Title:	Discussion and simulation results for BS 2-step RACH requirement

Document for:	Discussion

# 1	Introduction

In the last RAN4#96-e meeting, the general issues with demodulation requirement for NR 2-step RACH was discussed and the related agreement was captured in the WF [1]. Meanwhile, the initial assumption was proposed for requirement.

In this contribution, the view on remaining issue for NR 2-step RACH is provided.  Also, the initial simulation are provided for alignment.

# 2	Discussion

Remaining test parameters

Regarding the DMRS configuration, the following options are considered

- Option 1: 1+1+1 - Option 2: 1+1

The motivation is to verify the impact of timing offset on PUSCH MsgA performance. Generally, the timing offset estimation and compensated are performed per symbol by symbol. Different with high Doppler scenario, network will configure more DMRS symbols for Doppler or frequency offset tracking, where the tracking capability depends on the distance between DMRS symbols. Since the current scenario is not targeting with high speed scenario, therefore, either configure 2 DMRS or 3 DMRS configuration can fulfil the requirement. As indicated in our initial results, the similar performance can be achieved with two different DMRS configurations.

In the last meeting, the following combination set was agreed as

- Number of RB: 2 - Number of OFDM symbol: 14 - MCS: 1

For MsgA PUSCH transmission, the data payload size is 72 bits. The following is the feasibility checking for each combination set.

| Combination set | TBS | Coding Rate | Effective coding rate | Padding bits compared with 72 bits |
| --- | --- | --- | --- | --- |
| (14 OS, MCS1, 2 DMRS) | 88 | 0.1533 | 0.1806 | 16 |
| (14 OS, MCS1, 3 DMRS) | 80 | 0.1533 | 0.1818 | 8 |

Observation 1: In terms of padding bit, the combination set (14 OS, MCS1 and 3 DMRS) has less padding bits compared with combination set (14 OS, MCS 1 and 2 DMRS).

Observation 2: From the effective coding rate can be achieved for combination sets of (14 OS, MCS1 and 2 DMRS) and (14 OS, MCS 1, 3 DMRS)

The motivation of 2-step RACH is to guarantee the center UE can reduce the access time, meanwhile, the overhead of signaling can be reduced. In that sense, configured 3 DMRS can benefit for Doppler or frequency offset tracking, as well as the channel estimation performance

Meanwhile, based on the agreement about the DMRS configuration in RAN1 for MsgA PUSCH. In case there is no RRC parameters configured, the UE will apply the value with “pos2”.

Base on above analysis, we prefer to configure 3 DMRS for specifying MsgA PUSCH requirement.

Proposal 1: 3 DMRS can be configured for MsgA PUSCH requirement

Different level TO cycling

During the last meeting, it was agreed to introduce medium level TO cycling for MsgA PUSCH requirement. As for high level TO cycling, whether to introduce should be further discussion. As agreed, RAN4 will introduce TO cycling (X, ∆t, Y) in defining core requirement.

Generally, the value Y should be related with cell size and CP length. Since there is no limitation for preamble format, the timing offset should be applied with all the preamble format. The CP length, Ncs, maximal delay of fading channel, and derive the maximum timing offset can be calculated for all the preamble and SCS as indicated as follows：

| Format | SCS | CP | Ncs | (a) min {CP, Ncs} | (b) Maximum delay of fading channel (Note) | (a-b) Maximum configured timing offset |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 kHz | 288 | 339 | 288 | 80 | 208 | 6.77 us |
|  | 30 kHz | 144 | 339 | 144 | 80 | 64 | 2.08 us |
|  | 60 kHz | 72 | 254 | 72 | 9 | 63 | 2.05 us |
|  | 120 kHz | 36 | 127 | 36 | 9 | 27 | 0.88 us |
| A2 | 15 kHz | 576 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 288 | 339 | 288 | 80 | 208 | 6.77 us |
|  | 60 kHz | 144 | 254 | 144 | 9 | 135 | 4.39 us |
|  | 120 kHz | 72 | 127 | 72 | 9 | 63 | 2.05 us |
| A3 | 15 kHz | 864 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 432 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 216 | 254 | 216 | 9 | 207 | 6.74 us |
|  | 120 kHz | 108 | 127 | 108 | 9 | 99 | 3.22 us |
| B4 | 15 kHz | 936 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 468 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 234 | 254 | 234 | 9 | 225 | 7.32 us |
|  | 120 kHz | 117 | 127 | 117 | 9 | 108 | 3.52 us |
| C0 | 15 kHz | 1240 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 620 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 310 | 254 | 254 | 9 | 245 | 7.98 us |
|  | 120 kHz | 155 | 127 | 127 | 9 | 118 | 3.84 us |
| C2 | 15 kHz | 2048 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 30 kHz | 1024 | 339 | 339 | 80 | 259 | 8.43 us |
|  | 60 kHz | 512 | 254 | 254 | 9 | 245 | 7.98 us |
|  | 120 kHz | 256 | 127 | 127 | 9 | 118 | 3.84 us |
| Note: TDLC300 for FR1, TDLA30 for FR2 |  |  |  |  |  |  |  |

In order to simply the test, a unified timing offset should be considered for all the preamble formats. And it is seen from Table 1 that format A1 with 120 kHz SCS has the smallest maximum timing offset (due to the short CP length), which is 0.88us. During the Rel-15 PRACH test, the maximum timing offset is 0.8us.

As agreed in the last meeting

- The TO error is assumed to be same for a MsgA-PRACH preamble and MsgA-PUSCH pair - The TO error is updated per each occasion of PRACH preamble+ MsgA during the test

Based on this agreement, we think the TO should be set appropriately to guarantee both MsgA-PRACH and MsgA-PUSCH can be detected correctly. As shown in the table, for 30 KHz SCS, the smallest maximum timing offset for PRACH is 2.08 us. The current value for high level TO value is 3.8 us, which has already larger than the CP length.

As indicated our initial results, with the TO larger than CP, the BLER performance is still worse even with TO compensation operation, where the PUSCH detection error comes from the ISI.

From the demodulation performance perspective, the current high level TO setting is not feasible at least for 30KHz SCS.

From the typical scenario of NR 2-step RACH perspective, whether 2-step RACH or 4-step RACH applied is depending on RSRP threshold, according to 38.321 spec. The UE selected 2-step random access type to perform random access on this threshold. If Random Access Resource and the RSRP of the downlink pathloss reference is above Msg A-RSRP-Threshold, UE will apply the 2-step RACH. In that sense, 2-step RACH is targeting with cell-centers UE, cell-edge UEs will be fallback to 4 step RACH procedure.

From the BS receiver perspective, the motivation of introduce TO is to discriminate the practical BS implementation. As shown in our initial results, the performance gap between with TO compensation and without TO compensation for medium level TO is enough large, which can discriminate the proper BS receiver.

In that sense, it is not a typical scenario to specify the high level TO for MsgA PUSCH requirement.

Proposal 2:  Specify the MsgA PUSCH demodulation requirement only with medium level TO set.

Test Coverage

Regarding the test coverage, the following options are considered

- Discuss further the implications of adopting medium only or medium/high TO on test coverage - Option 1: only test the requirements for medium level TO cycling, i,e., lack of test coverage for large cell operation is OK - Option 2: test the requirements for both medium and high level TO cycling - Option 3: test either the medium TO or the high TO (but never both) depending on vendor declaration - Option 4: Only test the requirements for high level TO cycling, considering no performance difference after TO compensation and high level TO

Regarding the test coverage, as mentioned above, it is not a useful scenario to specify the high level TO for 2-step RACH. We do not think it is necessary to define the requirement with high level TO cycling.

Proposal 3: Only test the requirements for medium level TO cycling.

Test metric

Regarding the test metric, the following options are considered

- BLER of MsgA when preamble are correctly detected - Option 1: 0.01 - Option 2: 0.1

Since MsgA for 2-step RACH includes both preamble and PUSCH detections, the MsgA PUSCH detection depends on the preamble detection, where 1% missing detection is the target metric of preamble detection. In that sense, we are fine to use 1% for BLER of MsgA PUSCH.

Proposal 4: Using BLER 0.01 as the test metric for requirement of MsgA PUSCH.

Other test parameters

Regarding other test parameters not mentioned, we prefer to reuse the test parameters defined in Rel-15 for BS demodulation requirement.

Proposal 5: Use the following test parameters for requirement of MsgA PUSCH

PT-RS: not configured for both FR1 and FR2

SCS/BW: 15 kHZ SCS/5 MHz BW, 30 kHz SCS/10 MHz

60 kHz SCS/50 MHz BW, 120 kHz SCS/50 MHz

# 3	Initial Analysis

In this section, the initial simulation are provided to analysis the impact of timing offset on the MsgA PUSCH.

Simulation assumption for MsgA PUSCH with timing offset

| Parameter | Values for FR1 |  | Values for FR2 |  |
| --- | --- | --- | --- | --- |
| Preamble Format | Free Choice |  |  |  |
| Waveform | CP-OFDM |  |  |  |
| Power offset between preamble and MsgA | Free choice |  |  |  |
| Subcarrier Spacing for PUSCH | 15KHz, 30KHz |  | 60KHz, 120KHz |  |
| PUSCH mapping type | Type A and Type B |  | Type B |  |
| MCS level | 1 |  | 2 |  |
| Number of Symbols | 14 |  | 10 |  |
| Number of PRBs | 2 |  | 2 |  |
| TBS | 72 bits |  |  |  |
| DMRS | Option 1: 1+1+1 Option 2: 1+1 |  | 1+1 |  |
| Antenna configuration | 1T2R |  | 1T2R |  |
| Propagation channel | TDLC300-100 |  | TDLA30-300 |  |
| TO values | Medium Level | SCS 15KHz: 0:0.4:2 | Medium Level | SCS 60KHz: 0:0.1:0.5 |
|  |  | SCS 30KHz: 0:0.2:1 |  | SCS 120KHz: 0:0.025:0.25 |
|  | High Level | SCS 15KHz: 0:0.1:3.8 | High level | SCS 60KHz: 0:0.1:0.6 |
|  |  | SCS 30KHz: 0:0.1:3.8 |  | SCS 120KHz: 0:0.1:0.6 |
| Test metric (BLER of MsgA when preambles are correctly detected) | Baseline: 0.01 |  |  |  |
| Re-transmission | No retransmission considered |  |  |  |

Figure 1: BLER requirement with different timing offset under 2 DMRS configuration, 15KHz SCS

Figure 2: BLER requirement with different timing offset under 2 DMRS configuration, 30KHz SCS

Figure 3: BLER requirement with different timing offset under 3 DMRS configuration, 15KHz SCS

Figure 4: BLER requirement with different timing offset under 3 DMRS configuration, 30KHz SCS

Based on our initial results, we have several observations:

Observation 4: Similar BLER performance can be achieved with 2 DMRS and 3 DMRS configuration

Observation 5: With small value of TO, the impact on BLER performance of MsgA is minor without TO compensation.

Observation 6: With TO larger than CP, even with TO compensation operation, large performance degradation is still existed.

# 4 Simulation results

Table 1:  FR1 requirement for medium level TO cycling value (to be updated)

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | To range | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.4:2] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.4:2] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.2:1] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.2:1] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.4:2] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.4:2] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.2:1] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.2:1] | 1% |  |

Table 2:  FR1 requirement for high level TO cycling value (to be updated)

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | To range | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(2,7,11) | 14 | Type A | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(2,11) | 14 | Type A | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 15KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1+1(0,5,10) | 14 | Type B | TDLC 300-100 | [0:0.1:3.8] | 1% |  |
| 30KHz, 2PRB | MCS 1 from table 1 | 1T2R | 1+1(0,10) | 14 | Type B | TDLC 300-100 | [0:0.1:3.8] | 1% |  |

Table 3:  FR2 requirement for medium level TO cycling value (to be updated)

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | To range | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.1:0.5] | 1% |  |
| 120 KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.05:0.25] | 1% |  |

Table 4:  FR2 requirement for high level TO cycling value (to be updated)

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | To range | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.1:0.5] | 1% |  |
| 120 KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.05:0.25] | 1% |  |
| 60KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.1:0.5] | 10% |  |
| 120 KHz, 2PRB | MCS 2 from table 1 | 1T2R | 1+1(0,8) | 10 | Type B | TDLA 30-300 | [0:0.05:0.25] | 10% |  |

# 5 Conclusion

In this contribution, the view on the remaining issue of requirement is provided. Meanwhile, the initial simulation results are provided to investigate the timing offset impact on the MsgA PUSCH requirement.

Observation 1: In terms of padding bit, the combination set (14 OS, MCS1 and 3 DMRS) has less padding bits compared with combination set (14 OS, MCS 1 and 2 DMRS).

Observation 2: From the effective coding rate can be achieved for combination sets of (14 OS, MCS1 and 2 DMRS) and (14 OS, MCS 1, 3 DMRS)

Proposal 1: 3 DMRS can be configured for MsgA PUSCH requirement

Proposal 2:  Specify the MsgA PUSCH demodulation requirement only with medium level TO set.

Proposal 3: Only test the requirements for medium level TO cycling.

Proposal 4: Using BLER 0.01 as the test metric for requirement of MsgA PUSCH.

Proposal 5: Use the following test parameters for requirement of MsgA PUSCH

PT-RS: not configured for both FR1 and FR2

SCS/BW: 15 kHz SCS/5 MHz BW, 30 kHz SCS/10 MHz

60 kHz SCS/50 MHz BW, 120 kHz SCS/50 MHz

Observation 4: Similar BLER performance can be achieved with 2 DMRS and 3 DMRS configuration

Observation 5: With small value of TO, the impact on BLER performance of MsgA is minor without TO compensation.

Observation 6: With TO larger than CP, even with TO compensation operation, large performance degradation is still existed.

# References

- [1] R4-2008864 WF on BS demodulation requirements for 2-step RACH, ZTE