3GPP TSG-RAN WG4 #97-e	R4-2015129

Electronic Meeting, November 2nd – November 13th, 2020

Agenda item:	7.8.1.2.1

Source:	MediaTek Inc.

Title:	Discussion on eMBB UE performance requirement with pre-emption

Document for:	Discussion

- Introduction

In RAN4#95-e, the simulation assumption for URLLC UE was agreed. Besides, RAN4 agreed the assumption of UE behaviours for buffer flushing and decoding and URLLC interference modelling in RAN4#96-e [1].

- The assumption of UE behaviours for buffer flushing and decoding - If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. - URLLC interference modelling - SNR: Same as for eMBB transmission - Structure: Some random data

Based on the above simulation assumption and agreements, we provide our simulation results and proposal for pre-emption probability.

- Simulation results

In Fig. 1 and Fig. 2, we provide the simulation results for MCS4 and MCS 13 based on the agreed simulation assumptions. Besides, Table 1 show the SNR values to achieve the 70% of maximum throughput.

| FDD |  |
| --- | --- |
| Figure 1. PDSCH performance for pre-emption with MCS4 | Figure 2. PDSCH performance for pre-emption with MCS13 |

Table 1. SNR values at 70% throughput for FDD

| MCS | Pre-emption probability | SNR @ 70% throughput [dB] |  |
| --- | --- | --- | --- |
|  |  | Without HARQ buffer Flushing | With HARQ buffer Flushing |
| 4 | 10% | -2.09 | -2.09 |
|  | 20% | -1.36 | -1.38 |
| 13 | 10% | 6.19 | 6.15 |
|  | 20% | 8.86 | 8.31 |

We think that 20% probability is too aggressive in practice. It could be more realistic to schedule type B transmission rather than using the pre-emption feature if pre-emption probability is more than 10% during a radio frame. Hence, we think pre-emption periodicity of 10% is enough.

Proposal: Only configure 10% pre-emption probability for pre-emption tests.

- Summary

Our proposal is summarized as below:

Proposal: Only configure 10% pre-emption probability for pre-emption tests.

- Reference

- R4-2008808, “Simulation assumptions for NR URLLC UE performance requirements test cases”, Intel

- R4-2012648, “Way forward for NR UE URLLC performance requirements”, Intel