3GPP TSG-RAN WG4 Meeting #97-e				  											R4-2015225 Online, 2nd Nov. 2020 – 13th Nov. 2020

Source: 	Nokia

Title: 		TP for 37.717-21-11 to introduce DC_5A-7A_n7A

Agenda Item: 			10.4.2 [DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_5A-7A_n7A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_5-7_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations.

Co-existence analysis for DC_5_n7 UL shows that 3rd and 5th IMD may fall in DL band 5.

Co-existence analysis for DC_7_n7 UL is not needed since only switched UL is supported.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_5-7_n7 | 5 | 0.5 |
|  | 7 | 0.3 |
|  | n7 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_5-7_n7 | 5 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.x.4	Reference sensitivity exceptions

Based on co-existence studies additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 6. x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-7A_n7A | 5 | 834 | 5 | 25 | 879 | 12 | IMD34 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

*********************** End of the TP ***************************************************

# 3	References