3GPP TSG-RAN WG4 Meeting #97-e				  											R4-2015227 Online, 2nd Nov. 2020 – 13th Nov. 2020

Source: 	Nokia, ZTE

Title: 		TP for 37.717-21-11 to introduce DC_28A-66A_n7A

Agenda Item: 			10.4.2 [DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_28A-66A_n7A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_28-66_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_28A_n7A and DC_66A_n7A, where:

- No IMD product caused by DC_28A_n7A fall into own Rx of band 66.

- No IMD product caused by DC_66A_n7A fall into own Rx of band n28.

### 5.x.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands as given in 36.101.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-66_n7 | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-66_n7 | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

No further REFSENS exceptions needed.

*********************** End of the TP ***************************************************

# 3	References