3GPP TSG-RAN WG4 #97-e	R4-2015268

Online, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-21-11 DC_1A-40C_n78A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-21-11 for EN-DC band combinations in Rel-17 to introduce DC_1A-40C_n78A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 5.25	DC_1-40_n78

### 5.25.1	Operating bands for DC

Table 5.25.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-40_n78 | CA_1-40 | n78 | No |

### 5.25.2	Configurations for DC

Table 5.25.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A | CA_1A-40A CA_1A-40C | n78 |

### 5.25.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n78 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD of the two bands may fall into Rx frequencies of band 1 and band 40.

And based on co-existence studies of Band 40 + Band n78 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 1.

- 	harmonic mixing generated by 2nd harmonic of n78 Tx and 3rd harmonic of band 40 Rx.

### 5.25.4	∆TIB and ∆RIB values

Table 5.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-40-n78 | 1 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-40-n78 | 1 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.25.5	REFSENS requirements

As stated in 5.25.3, for MSD requirement caused by IMDs is specified below accordingly. To note that the MSD requirements are already defined for DC_1A_n40A-n78A in TS 38.101-3, among which the one for IMD4 on band n40 can be followed here for band 40 IMD4 impact. The requirements apply both to DC_1A-40A_n78A and DC_1A-40C_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 40 | 2340 | 5 | 25 | 2340 | 10.6 | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |

Other issues mentioned in 5.25.3 (i.e., harmonic mixing between band 40 Rx and n78 Tx, and MSD for band 1 caused by 4th IMD of band 1 and n78) had already been specified in the corresponding specs.

----- Unchanged sections omitted -----

# Reference