3GPP TSG-RAN WG4 #97-e	R4-2015275

Online, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-31-11 DC_3A-7A-40C_n78A

Agenda item:			10.5.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-31-11 for EN-DC band combinations in Rel-17 to introduce DC_3A-7A-40C_n78A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 5.1.x		DC_3-7-40_n78

### 5.1.x.1	Configuration for EN-DC

Table 5.1.x.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |

### 5.1.x.2	∆TIB and ∆RIB values

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.x.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.x.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

----- Unchanged sections omitted -----

# Reference