3GPP TSG-RAN WG4 #97-e	R4-2015282

Online, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717-51-11 DC_1A-3A-7A-8A-40C_n78A

Agenda item:			10.16.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-51-11 for EN-DC band combinations in Rel-17 to introduce DC_1A-3A-7A-8A-40C_n78A.

# 2. Text Proposal

----- Unchanged sections omitted -----

### 5.1.x	DC_1-3-7-8-40-n78

#### 5.1.x.1	Operating bands for DC_ 1-3-7-8-40-n78

Table 5.1.x.1-1: Band combinations EN-DC (six bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-7-8-40_n78 | CA_1-3-7-8-40 | n78 | no |

#### 5.1.x.2	Configurations for DC_ a-b-c-d-e-nW

Table 5.1.x.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-8A-40A_n78A DC_1A-3A-7A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A | CA_1A-3A-7A-8A-40A CA_1A-3A-7A-8A-40C | n78 |

#### 5.1.x.3	∆TIB and ∆RIB values

Table 5.1.x.3-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8-40_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.x.3-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8-40_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

----- Unchanged sections omitted -----

# Reference