3GPP TSG RAN4 Meeting #97e	R4-2015289

# Electronic Meeting, November 2 - 13, 2020

Title: 	Discussion on the new SUL band n97 BS requirements

Source: 	Huawei, HiSilicon

Agenda item:	10.27.2

Document for:	Discussion

# Introduction

In the RANP#88 plenary meeting, a new WI to introduce a new SUL band for 2300 – 2400MHz is approved in [1]. The new proposed SUL band overlaps with the exact frequencies of TDD band n40. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is straightforward that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n97.

For the BS requirements, introducing SUL band n97 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

In this paper, we discuss the necessary BS RF requirements to be specified in order to accomplish introduction of the new SUL band n97. In our parallel paper in [2], UE RF requirements are discussed correspondingly.

# BS RF Requirements

Several BS specs need to be considered to introduce a new band. Taking TS 38.104 as an example, the corresponding parts shall be introduced for spurious emissions as below and in every other spec.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n97 | 2300 – 2400 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n40 since it is already covered in clause 6.6.5.2.2. |

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |  |
| NR band n97 | 2300 – 2400MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

The related BS specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, which are the same specs for introducing band n96 at the same time.

Proposal:  Specify BS spurious emissions requirements for the new SUL band n97 in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

# Conclusions

In this paper, we discuss the necessary BS RF requirements to be specified in order to accomplish introduction of the new SUL band n97. We propose:

Proposal:  Specify BS spurious emissions requirements for the new SUL band n97 in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

# References

[1] RP-201363, New WID on introduction of 1880-1920MHz SUL (supplemental uplink) band for NR, CMCC

[2] RP-2015289, Discussion on the new SUL band n97 UE requirements, Huawei, HiSilicon