3GPP TSG RAN4 Meeting #97e	R4-2015290

# Electronic Meeting, November 2 - 13, 2020

Title: 	Discussion on the new SUL band n98 UE requirements

Source: 	Huawei, HiSilicon

Agenda item:	10.26.1

Document for:	Discussion

# Introduction

In the June plenary meeting, a new WI to introduce a new SUL band for 1880 – 1920MHz is approved in [1]. The new proposed SUL band overlaps with the exact frequencies of TDD band n39. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is straightforward that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n98.

For UE RF requirements, introducing SUL band n98 involves the items listed in the below box which are all specified in TS 38.101-1:

UE RF requirements - Operating bands - UE channel bandwidth - Channel raster - UE MOP and MPR/AMPR - Spurious emissions

For the BS requirements, introducing SUL band n98 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2. A parallel discussion paper in [2] discussed the BS requirements.

In this paper, we discuss the necessary UE RF requirements to be specified in order to accomplish introduction of the new SUL band n98. The only remaining issue after discussions carried out in the last meeting now is how to put a note in the operating band table to clarify that the new SUL band n98 is not interfering corresponding TDD bands 39/n39 in terms of cross UL/DL interferences.

# UE RF Requirements

Operating band number

It is consensus in the last meeting that the SUL band for 1880 – 1920MHz is named with n98.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n98 | 1880 MHz – 1920 MHz | N/A | SUL |

Proposal 1:  Introduce the new SUL band for 1880 – 1920MHz as band n98.

UE channel bandwidth

It is natural that for the new SUL band n98, the UE supporting this band should support all the channel bandwidths specified for band n39 as listed below, since n39 and n98 have the exact same frequencies for UL.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n98 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

Channel raster

The channel raster for n98 should follow exactly the one for band n39 UL.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n98 | 100 | 376000 – <20> – 384000 | N/A |

MOP and A-MPR

The MOP requirements of PC3 for SUL band n98 should follow the ones for band n39 and a few others.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n98 |  |  |  |  | 23 | ±2 |

For MPR requirements, the generic ones apply to n98.

For A-MPR requirements, it is reasonable that band n98 should follow band n39 at least with a default NS_01. Regarding NS_50, since it is to protect the 1805 – 1880MHz frequencies, we also propose to add NS_50 for band n98 since the protected frequencies are widely used globally.

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_50 | 6.5.3.3.16 | n39, n98 | 25, 30, 40 |  | Clause 6.2.3.19 |

Spurious emissions

Regarding spurious emissions, band n98 should have the same requirements as those for band n39 as below.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n39, n98 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 15, 26, 33 |

Proposal 2:  Specify UE RF requirements for the new SUL band n98 following band n39.

# Conclusions

In this paper, we discuss the necessary UE RF requirements to be specified in order to accomplish introduction of the new SUL band n98. We propose:

Proposal 1:  Introduce the new SUL band for 1880 – 1920MHz as band n98.

Proposal 2:  Specify UE RF requirements for the new SUL band n98 following band n39.

# References

[1] RP-201363, New WID on introduction of 1880-1920MHz SUL (supplemental uplink) band for NR, CMCC

[2] R4-2015291, Discussion on the new SUL band n98 BS requirements, Huawei, HiSilicon