3GPP TSG RAN4 Meeting #97e	R4-2015292

# Online, 2nd – 13th November, 2020

Title: 	Adding 40M bandwidth for band n80 and n83

Source: 	Huawei, HiSilicon

Agenda item:	10.21.2

Document for:	Discussion

# Introduction

In the last plenary meeting, proposals for adding new existing BWs are approved in the revised WID RP-202062 [1]. It is copied below that for every proposal, RAN4 considers to specify the corresponding RF requirements:

- Analyze and specify requirements: - Reference sensitivity and associated RB allocation. - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - NS signalling. - Any other RF requirement which might be relevant.

In this paper, we discuss the corresponding RF requirements for the approved proposals: adding 40MHz for n80 and n83 (only BS). The generic idea is to reuse as much as possible the requirements that had been defined for n3 and n28, respectively.

# Requirements for adding 40MHz to band n83 BS

System parameter: BW

To add 40MHz to band n83, for BS spec the 40MHz channel bandwidth needs to be added to the channel bandwidth tables in 38.104. Below is an example of TP to specify the 40MHz BW for n83 BS for TS 38.104.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
| n83 | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

There is a parallel CR to implement the above TP into TS 38.104 in [2].

# Requirements for adding 40MHz to band n80

System parameter: BW

Similar to 40MHz addition for n83, adding 40MHz for n80 requires the addition of the corresponding system parameters for both UE and BS specs. The below TP shows an example of that specification. To note that the reference architecture of a UE operating in n80 resembles that of one operating in n3.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| n80 | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

It is also the same that for band combinations, adding the proposed channel bandwidth can be covered by BC basket WI. In parallel to this discussion paper, there are two CRs to implement the proposed TP above for band n80 in [3] and [4].

Proposal:  RAN4 agrees on the CRs proposed in this meeting in [2] [3] and [4] to add 40MHz for band n83 and n80, respectively.

# Conclusions

In this paper, we discuss the corresponding RF requirements for the approved proposals: adding 40MHz for n80 and n83 (only BS). The generic idea is to reuse as much as possible the requirements that had been defined for n3 and n28, respectively.

Together we propose:

Proposal:  RAN4 agrees on the CRs proposed in this meeting in [2] [3] and [4] to add 40MHz for band n83 and n80, respectively.

# References

[1] RP-202062, Revised Basket WID on Rel-17 adding channel bandwidth support to existing NR bands, Ericsson

[2] R4-2015295, draftCR to 38104 to add 30MHz BW for band n83, Huawei, HiSilicon

[3] R4-2015293, draftCR to 38101-1 to add 40MHz BW for band n80, Huawei, HiSilicon

[4] R4-2015294, draftCR to 38104 to add 40MHz BW for band n80, Huawei, HiSilicon