3GPP TSG RAN4 Meeting #97e	R4-2015296

# Online, 2nd – 13th November, 2020

Title: 	Adding 90 and 100MHz bandwidth for band n40

Source: 	Huawei, HiSilicon

Agenda item:	10.21.2

Document for:	Discussion

# Introduction

In the last plenary meeting, proposals for adding new existing BWs are approved in the revised WID RP-202062 [1]. It is copied below that for every proposal, RAN4 considers to specify the corresponding RF requirements:

- Analyze and specify requirements: - Reference sensitivity and associated RB allocation. - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - NS signalling. - Any other RF requirement which might be relevant.

In this paper, we discuss the corresponding RF requirements for the approved proposals: adding 90 and 100MHz for band n40. We mainly propose to have 1dB deltaMPR specified for 100MHz UE channel bandwidth on band n40. To implement the BW addition of 90 and 100MHz for band n40, draftCRs are provided in [3] and [4].

# Introducing 90 and 100MHz UE channel bandwidth for band n40

To add 90 and 100MHz UE channel bandwidth to n40 requires thorough review on the general UE RF requirements specified for band n40 in TS 38.101-1. In the below table, we list the corresponding UE RF requirements that belong to band n40 in the current spec, and we also identify the potential specification needed to accommodate 90 and 100MHz UE channel bandwidth.

| Item | Sub-item | Appliance | Note |
| --- | --- | --- | --- |
| Tx |  |  |  |
| MOP | PC3 23dBm; PC2 26dBm | Yes to both 90 and 100MHz |  |
| MPR | General MPR | Yes |  |
|  | Delta MPR | 0dB for 90 1dB for 100 | 1dB deltaMPR specification is needed according to our measurements |
| AMPR | None | NA |  |
| Emissions | SEM | Generally applied |  |
|  | ACLR |  |  |
|  | Spurious |  |  |
| Others |  | Generally applied |  |
| Rx |  |  |  |
| REFSENS | 30k SCS REFSENS | Calculation based on smaller bandwidth, value scaled with bandwidth/RB allocations | FFS if relaxation is needed. |
|  | 60k SCS REFSENS | Same logic as 30k SCS |  |
| Others |  | Generally applied |  |

# 1dB deltaMPR for 100MHz UE channel bandwidth

In the above table, we propose 1dB delta MPR specification on n40 for 100MHz UE channel bandwidth.

According to the agreement in [2], the limitation for maximum UE channel bandwidth is the relative bandwidth, which is the key to ensure the MPR performance (Memory effects, bandwidth limitation, and output matching Q). The agreement for maximum channel bandwidth criteria in [2] is as follows:

In order to introduce new channel bandwidths in the future we propose to introduce a criteria to decide if the general MPR applies or additional MPR should be evaluated:

- If max UL CH BW rel ≤3% for FDD bands and ≤4% for TDD bands general MPR requirement applies

- If max UL CH BW rel >3% for FDD bands and >4% for TDD bands additional band and channel bandwidth specific MPR should be evaluated

According to equation of maximum relative channel bandwidth as following,

max UL CH BW rel = (2*max CH BW) / (FUL low + FUL high)

we can calculate the maximum channel bandwidth for NR band n40, which is 94MHz and close to 100MHz.

According to the agreement above, we see the possibility to define 100MHz UE channel bandwidth, if the proper deltaMPR is defined. Thus we would like to propose 1dB deltaMPR according to the results of series of measurements carried out upon PA, RFIC and filters. It would be OK when 100MHz channel bandwidth is utilized if the transmission power is reduced slightly and properly, while when the higher transmission power is needed the smaller BWP or per-UE channel bandwidth can be configured.

And it seems feasible to specify 90MHz channel bandwidth even without bandwidth specific deltaMPR.

Proposal 1: Specify 1dB deltaMPR for 100MHz UE channel bandwidth for n40.

# REFSENS for 90 and 100MHz UE channel bandwidths

In the below table it is listed that the current REFSENS specified in TS 38.101-1 for band n40 and n41 respectively.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n40 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 | -89.6 |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 | -89.7 | -88.9 |  | -87.6 |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 | -89.8 | -89.1 |  | -87.6 |  |  |  |
| n411 | 15 |  | -94.8 | -93.0 | -91.8 |  | -89.9 | -88.6 | -87.6 |  |  |  |  |  | TDD |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  | -90.0 | -88.7 | -87.7 | -86.9 |  | -85.6 | -85.1 | -84.7 |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  | -90.1 | -88.9 | -87.8 | -87.1 |  | -85.6 | -85.1 | -84.7 |  |

Correspondingly, RB allocations are specified as in the below table.

|  | Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n40 |  | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  |  | 30 |  | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 |  | 216 |  |  |  |
|  |  | 60 |  | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |
| n41 |  | 15 |  | 50 | 75 | 100 |  | 160 | 216 | 270 |  |  |  |  |  | TDD |
|  |  | 30 |  | 24 | 36 | 50 |  | 75 | 100 | 128 | 162 |  | 216 | 243 | 270 |  |
|  |  | 60 |  | 10 | 18 | 24 |  | 36 | 50 | 64 | 75 |  | 100 | 120 | 135 |  |

For 30KHz SCS, the values for REFSENS on band n41 with 90 and 100MHz are derived by scaling the values with bandwidths. Thus for band n40 it can also be derived that the REFSENS for 90 and 100MHz are ideally -87.1 and -84.7dBm. Meanwhile the RB allocations for band n40 follows the ones specified for n41: 243RBs for 90M and 270RBs for 100MHz.

For 60KHz SCS, the logic of deriving the REFSENS values should resemble the one used for 30KHz SCS. This means that the REFSENS are the same with 30KHz while RB allocations are 120RB for 90M and 135RB for 100MHz.

In general, we should specify the REFSENS according to the above elaboration or at least use the values as baseline. We also welcome vendors provide more detailed analysis based on the true characteristics of popular RF parts to see if further relaxations are needed on n40. Guaranteeing prosperity of 2.3GHz NR industry requires thorough considerations as we see it.

We propose:

Proposal 2: Use values of REFSENS and RB allocation in the below table as the baseline when specifying requirements for 90 and 100MHz UE channel bandwidth for band n40.

| Bandwidth | SCS | REFSENS Value (dBm) | RB allocation |
| --- | --- | --- | --- |
| 90MHz | 30KHz | -87.1 | 243 |
|  | 60KHz | -87.1 | 120 |
| 100MHz | 30KHz | -84.7 | 270 |
|  | 60KHz | -84.7 | 135 |

# Conclusions

In this paper, we discuss the corresponding RF requirements for the approved proposals: adding 90 and 100MHz for band n40. We mainly propose to have 1dB deltaMPR specified for 100MHz UE channel bandwidth on band n40. To implement the BW addition of 90 and 100MHz for band n40, draftCRs are provided in [3] and [4].

Together we propose:

Proposal 1: Specify 1dB deltaMPR for 100MHz UE channel bandwidth for n40.

Proposal 2: Use values of REFSENS and RB allocation in the below table as the baseline when specifying requirements for 90 and 100MHz UE channel bandwidth for band n40.

| Bandwidth | SCS | REFSENS Value (dBm) | RB allocation |
| --- | --- | --- | --- |
| 90MHz | 30KHz | -87.1 | 243 |
|  | 60KHz | -87.1 | 120 |
| 100MHz | 30KHz | -84.7 | 270 |
|  | 60KHz | -84.7 | 135 |

# References

[1] RP-202062, Revised Basket WID on Rel-17 adding channel bandwidth support to existing NR bands, Ericsson

[2] Skyworks, Huawei, Qorvo, Broadcom, R4-1801008, “WF on maximum channel bandwidth criteria”, 3GPP TSG RAN4 Meeting #94e.

[3] R4-2015297, draftCR to 38101-1 to add 90 and 100MHz BW for band n40, Huawei, HiSilicon

[4] R4-2015298, draftCR to 38104 to add 90MHz BW for band n40, Huawei, HiSilicon