3GPP TSG-RAN WG4 Meeting #97-e           	R4-2015304

E-Meeting, 02 November 2020 – 13 November 2020

Title:	Discussion on cross carrier BWP switch delay requirements for single and multiple CC

Source:	NEC

Agenda item:	7.13.1.3

Document for:	Approval

Introduction

In Rel-15 and Rel-16, RAN4 defined delay requirements for DCI based BWP switch request received on the same carrier. However, RAN4 did not discuss and define BWP switch delay requirements when the DCI is received using cross-carrier scheduling mechanism. In this contribution, we provide our views on delay requirements for DCI based BWP switching when the DCI indication through cross carrier scheduling.

Discussion

BWP switching delay for DCI based BWP switching was discussed for many meetings for same carrier scheduling. Since most of the steps involved in cross-carrier scheduling also same, to derive requirements for cross-carrier scheduling, we mainly look at difference of steps involved between same carrier scheduling and cross carrier scheduling.

Compared to same carrier DCI scheduling, when the DCI is scheduled on cross-carrier, we may need to account for additional delay arising due to following components

- Delay for cross carrier communication

- MRTD between spCell and SCell; and

- Difference of SCS between the two carriers (between spCell and SCell).

Since some of these issues are discussed during SCell dormancy study, to make progress, we can use the agreements made in SCell dormancy requirements study.

Delay required to indicate BWP switch command received on one carrier to other carrier involves PDCCH decoding delay, delay between cross-carrier communication and MRTD between spCell and SCell. From the previous discussion of BWP switch delay requirements, PDCCH processing delay may be in the order of 100us (though it depends on numerology, we can assume it in the order of 100us). Maximum MRTD value specified in TS 38.133 is 33us. To compute worst case delay we can take it as 33us. Delay for cross carrier communication can be assumed as 100us.

Considering above factors, BWP switch trigger received on spCell may be received at SCell after 233us (approx.).

For further analysis, let us consider two scenarios shown in figure 1 and figure 2, that is spCell SCS is larger than SCell SCS (figure 1) and spCell SCS is smaller than SCell SCS (figure 2).

Figure 1: Scenario when spCell SCS is larger than SCell SCS

As shown in figure 1 and 2, DCI received on spCell at the start of the slot may be received on SCell after 3 OFDM symbol or in the second half of the slot based on the numerology. As shown in both figures, since DCI is received on SCell after 230us, there may not much duration left in the slot for processing. This results in BWP switching spreading over to next slot.

Figure 2: Scenario when spCell SCS is smaller than SCS

Based on the above analysis, since UE may not have full slot for processing of BWP switching, we propose to add one slot relaxation on top of current BWP switching delay requirement for both single CC and multiple CC BWP switching. We make following proposal from the above analysis.

Proposal 1: DCI based BWP switching delay is relaxed by one slot w.r.t single and multiple CC BWP switching delay, when DCI is received/scheduled using cross-carrier scheduling.

- Conclusion

In this contribution, we have analysed the delay requirements for DCI based BWP switching when the DCI indication through cross carrier scheduling BWP switching on multiple CC and made the following proposals:

Proposal 1: DCI based BWP switching delay is relaxed by one slot w.r.t single and multiple CC BWP switching delay, when DCI is received/scheduled using cross-carrier scheduling.

- References

- R4-2012278