3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2015314

Electronic Meeting, 2-13 Nov., 2020

Agenda item:		7.10.1.1

Source:				NTT DOCOMO, INC.

Title:		Views on 256QAM UE requirements for FR2

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed 256QAM UE requirements and RAN4 agreed a WF [1]. This contribution presents our views on 256QAM UE requirements.

2. Discussion

  - Simulation results

Table 1 shows our simulation results based on the simulation parameters [1].

Observation 1

Table 1. Summary of ideal simulation results

| Rank | Channel Model | Antenna configuration | CBW (MHz) | SNR point (dB) @70%TP Using MCS index: |
| --- | --- | --- | --- | --- |
|  |  |  |  | 20 |
| 1 | TDLD 30-75 | 2x2 | 50 | 16.5 dB |
| 1 | TDLA 30-300 | 2x2 | 50 | 17.4 dB |

Observation 2: Considering around 3dB impairment margin, TDL-D 30-75 and TDL-A 30-300 are testable under 50MHz CBW with full PRB allocation.

  - Propagation condition

Propagation condition

Propagation condition for FR2 256QAM was also discussed at the last meeting, and RAN4 has not reached the consensus. The possible options are TDL-D 30-75 and TDL-A 30-300 [1]. Since NLOS channel model (TDL-A) is more typical for UE demodulation tests than LOS channel model (TDL-D), we prefer TDLA30-300. However, we are fine to discuss further based on simulation results.

Proposal 1: Define FR2 DL 256QAM demodulation requirements with TDLA30-300

3. Conclusion

In this contribution, we present our views on 256QAM UE requirements for FR2. Our proposals and observations are summarized below.

Observation 1

Table 1. Summary of ideal simulation results

| Rank | Channel Model | Antenna configuration | CBW (MHz) | SNR point (dB) @70%TP Using MCS index: |
| --- | --- | --- | --- | --- |
|  |  |  |  | 20 |
| 1 | TDLD 30-75 | 2x2 | 50 | 16.5 dB |
| 1 | TDLA 30-300 | 2x2 | 50 | 17.4 dB |

Observation 2: Considering around 3dB impairment margin, TDL-D 30-75 and TDL-A 30-300 are testable under 50MHz CBW with full PRB allocation.

Proposal 1: Define FR2 DL 256QAM demodulation requirements with TDLA30-300

References

- R4-2012666, China Telecom, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4 #96e