3GPP TSG RAN4 Meeting #97e	R4-2015356

# Electronic Meeting, November 02 - 13, 2020

Title: 	Discussion on the new SUL band for 1.6GHz

Source: 	Huawei, HiSilicon

Agenda item:	10.30

Document for:	Discussion

Introduction

At RAN#88e, a new work item related to introduction of SUL band in 1.6 GHz was approved in [1]. In the last RAN4 meeting, a work plan for the WI was also approved in [2], which categorized up the work into three areas as follows:

Common part - System paramters: - Operating band - Channel bandwidth - Channel raster - Synchronization raster BS Part - Co-existence - Spurious emissions - Co-location with other base stations UE Part - Given the justification provided in the WID, determine if requirements are required to meet the transmit power limits in the UL frequency range, 1627.5 – 1632.5 MHz - UE maximum output power - Out of band emissions - Spurious emissions

The new proposed SUL band overlaps with the exact frequencies of FDD band n24. With the previous experiences in introducing the SUL band n98 which overlaps with the TDD band n39, it is straightforward that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n99.

For UE RF requirements, introducing SUL band n99 involves the items listed in the below box which are all specified in TS 38.101-1:

UE RF requirements - Operating bands - UE channel bandwidth - Channel raster - UE MOP and MPR/AMPR - Spurious emissions

For the BS requirements, introducing SUL band n99 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n99. Companion draftCRs are in [3] – [13].

UE RF Requirements

Operating band number

In success to n98, the new SUL band to be introduced for 1626.5-1660.5 MHz can be named with n99.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n99 | 1626.5 MHz –1660.5 MHz | N/A | SUL |

Proposal 1:  Introduce the new SUL band for 1626.5-1660.5 MHz as band n99.

UE channel bandwidth

It is natural that for the new SUL band n99, the UE supporting this band should support all the channel bandwidths specified for band n24 as listed below, since n24 and n99 have the exact same frequencies for UL.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n99 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |

Channel raster

The channel raster for n99 should follow exactly the one for band n24 UL.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n99 | 100 | 325300 – <20> – 332100 | N/A |

MOP and A-MPR

The MOP requirements of PC3 for SUL band n99 should follow the ones for band n24 and a few others.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n99 |  |  |  |  | 23 | ±2 |

For MPR requirements, the generic ones apply to n99.

For A-MPR requirements, it is reasonable that band n99 should follow band n24 at least with a default NS_01.

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n99 | NS_01 |  |  |  |  |  |  |  |

Spurious emissions

Regarding spurious emissions, band n99 should have the same requirements as those for band n24 as below.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band |  | Spurious emission for UE co-existence |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| n24, n99 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |  |

Proposal 2:  Specify UE RF requirements for the new SUL band for 1626.5-1660.5 MHz following band n24.

BS RF Requirements

Several BS specs need to be considered to introduce a new band. Taking TS 38.104 as an example, the corresponding parts shall be introduced for spurious emissions as below and in every other spec.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n99 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz |  |

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |  |
| NR band n99 | 1626.5 – 1660.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

The related BS specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

Proposal 3:  Specify BS spurious emissions requirements for the new SUL band for 1626.5-1660.5MHz in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

Conclusions

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n99. Companion draftCRs are in [3] – [13]. We propose:

Proposal 1:  Introduce the new SUL band for 1626.5-1660.5MHz as band n99.

Proposal 2:  Specify UE RF requirements for the new SUL band for 1626.5-1660.5MHz following band n24.

Proposal 3:  Specify BS spurious emissions requirements for the new SUL band for 1626.5-1660.5MHz in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

References

[1] RP-201251, New WID proposal on introduction of 1.6 GHz NR SUL band with same uplink frequency range of Band 24, Ligado Networks

[2] R4-2010745, Work plan for Introduction of 1.6 GHz NR SUL Band with same uplink frequency range of Band 24, Ligado Networks

[3] R4-2015357, draftCR to 38101-1 on introducing new SUL band n99, Huawei, HiSilicon

[4] R4-2015358, draftCR to 38104 on introducing new SUL band n99, Huawei, HiSilicon

[5] R4-2015359, draftCR to 36104 on introducing new SUL band n99, Huawei, HiSilicon

[6] R4-2015360, draftCR to 38141-1 on introducing new SUL band n99, Huawei, HiSilicon

[7] R4-2015361, draftCR to 38141-2 on introducing new SUL band n99, Huawei, HiSilicon

[8] R4-2015362, draftCR to 36141 on introducing new SUL band n99, Huawei, HiSilicon

[9] R4-2015363, draftCR to 37104 on introducing new SUL band n99, Huawei, HiSilicon

[10] R4-2015364, draftCR to 37141 on introducing new SUL band n99, Huawei, HiSilicon

[11] R4-2015365, draftCR to 37105 on introducing new SUL band n99, Huawei, HiSilicon

[12] R4-2015366, draftCR to 38145-1 on introducing new SUL band n99, Huawei, HiSilicon

[13] R4-2015367, draftCR to 38145-2 on introducing new SUL band n99, Huawei, HiSilicon