3GPP TSG-RAN WG4 Meeting # 97-e                                  		 R4-2015397

Electronic Meeting, 2nd – 13th November, 2020

Source:	Huawei, HiSilicon

Title:	TP for TR 36.717-04-01 CA_3A-8A-20A-38A

Agenda item:	14.3.2

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_3A-8A-20A-38A as defined in the revised WID [1].

Reference

[1]	RP-201911, “Revised WID: LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL”, Nokia, Nokia Shanghai Bell

Text Proposal

---Start of changes---

5.x	CA_3-8-20-38

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-20A-38A | - | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	∆TIB and ∆RIB values

Table 5.x.2-1: IB,c

| CA_3-8-20-38 | 3 | 0.5 |
| --- | --- | --- |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |

Table 5.x.2-2: R IB,c

| CA_3-8-20-38 | 3 | 0 |
| --- | --- | --- |
|  | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |

5.x.3 	REFSENS requirements

There is no MSD requirements for CA_3A-8A-20A-38A.

---End of changes---