3GPP TSG-RAN WG4 Meeting #97-e		                               R4-2015432

Electronic Meeting, 2 Nov – 13 Nov, 2020

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n3, n8, n25 and n71 for new channel bandwidth

Agenda Item:	10.22.3	UE RF requirements	[NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

Introduction

In the last RAN4 e-meeting, WF[1] was approved which about introduce 35MHz channel bandwidth (CBW) for bands n3, n7, n8, n25, n66, n71 and 45MHz CBW for bands n3, n25, n66. For n7 and n66, the REFSENS can be specified by channel scaling method because these bands have a large Tx/Rx duplex distance. On the other hand, the REFSENS of n3, n8, n25 and n71 should be studied.

In this contribution, we provide the PA Rx noise measurement results, MSD analysis and REFSENS for n3, n8, n25 and n71 at 35MHz/45MHz CBW operation.

Experimental Measurements

Power Amplifier Calibration and Test Waveforms

PA conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test conditions:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Experimental Results

The Rx band Noise (RxBN) of PA was measured by setting with RB specified in the current specifications of the bandwidth closest to the target bandwidth. PA output power is set 27dBm since the front-loss is assumed 4 dB.

In experiments, we noticed that IMD had a significant effect to RxBN in n8, n25, and n71. As example, the PA output spectrum of n8 at 30MHz and n25 at 45MHz is shown in Figure 2-1 and Figure 2-2, respectively.

Figure 2-1 the PA output spectrum of n8 at 27dBm for 35MHz CBW operation (20RBs)

Figure 2-2 the PA output spectrum of n25 at 27dBm for 45MHz CBW operation (40RBs)

The measurement results is shown in table 2-1.

Table 2-1 PA RxBN measurement results for each band

| Band | BW | NRB | PA RxBN |
| --- | --- | --- | --- |
| (-) | (MHz) | (-) | (dBm/Hz) |
| n3 | 35 | 50 | -39.8 |
|  | 45 | 50 | -37 |
| n8 | 35 | 20 | -12.2 |
| n25 | 35 | 40 | -38.5 |
|  | 45 | 40 | -20.8 |
| n71 | 35 | 20 | -12.6 |

Observation 1:  For n8, n71, the RxBN is significantly degraded due to the influence of IMD when the CBW is wider.

Observation 2:  For n25, the RxBN is degraded for 45MHz operation however less than n8 and n71.

Observation 3:  For n3, the RxBN is slightly degraded.

MSD Estimation

The link-budget calculation assumptions are listed in table 3-1 and MSD analysis is in table 3-2.

Table 3-1 Typical receiver performance parameters for MSD analysis

| Parameter | Band / Value |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | n3 | n8 | n25 | n71 |  |
| Antenna isolation | 10 | 10 | 10 | 10 | dB |
| DPX isolation from TX to RX | 50 | 50 | 50 | 55 | dB |
| DPX attenuation at RX band | 40 | 30 | 40 | 45 | dB |
| Diversity receiver filter TX rejection | 40 | 45 | 40 | 40 | dB |
| Front-end loss | 4 | 4 | 4 | 4 | dB |
| Thermal noise at RX ANT port | -164 | -164 | -164 | -164 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -144 | -144 | -144 | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | -1 | -1 | -1 | dB |

Table 3-2 MSD analysis for each band

| Parameter | Unit | n3 |  |  |  | n8 |  | n25 |  |  |  | n71 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW | MHz | 35.0 |  | 45.0 |  | 35.0 |  | 35.0 |  | 45.0 |  | 35.0 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -39.8 |  | -37.0 |  | -12.2 |  | -38.5 |  | -20.8 |  | -12.6 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  | 50.0 |  | 50.0 |  | 50.0 |  | 55.0 |  |
| DPX ATT RX band | dB | 40.0 |  | 40.0 |  | 30.0 |  | 40.0 |  | 40.0 |  | 45.0 |  |
| Div_filter_Tx rejection | dB | 40.0 |  | 40.0 |  | 45.0 |  | 40.0 |  | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -85.8 | -85.8 | -83.0 | -83.0 | -58.2 | -58.2 | -84.5 | -84.5 | -66.8 | -66.8 | -63.6 | -63.6 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -95.0 | -87.0 | -95.0 | -87.0 | -105.0 | -87.0 | -95.0 | -87.0 | -95.0 | -97.0 | -95.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -85.7 | -85.7 | -84.6 | -84.6 | -85.7 | -85.7 | -85.7 | -85.7 | -84.6 | -84.6 | -85.7 | -85.7 |
| Total noise level at PRx/DRx ANT Port | dBm | -82.4 | -83.5 | -80.8 | -81.5 | -59.2 | -59.2 | -81.8 | -82.8 | -67.7 | -67.7 | -64.6 | -64.6 |
| De-sens. | dB | 2.7 |  | 3.4 |  | 26.5 |  | 3.3 |  | 16.9 |  | 21.1 |  |

Observation 4:  For n8, n71 at 35MHz CBW and n25 at 45MHz CBW, there are large de-sens since the significant RxBN degradation.

Proposal 1:  REFSENS for 35/45MHz CBW operation are proposed highlighted as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n3 | 15 | -97 | -93.8 | -92 | -90.8 | -89.7 | -88.9 | -86.0 | -82.3 | -84.2 |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -91 | -89.8 | -89 | -86.1 | -82.4 | -84.3 |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90 | -89.1 | -86.2 | -82.6 | -84.4 |  |  |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -62.2 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -62.3 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -85.4 | -79.5 | -70.7 |  | FDD |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -85.5 | -79.6 | -70.8 |  |  |
|  | 60 |  | -94 | -91.9 | -90.7 | -89.6 | -82.4 | -85.6 | -79.7 | -70.9 |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -67.6 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -67.7 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n3 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25 | 50 | 50 | 50 | 50 | 48 | 40 | 40 | 40 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 20 | 20 | 20 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, we present PA Rx band noise in NR band n3, n8, n25, n71 at 35MHz, 45MHz CBW and propose the following reference sensitivity levels and UL configurations.

Observation 1:  For n8, n71, the RxBN is significantly degraded due to the influence of IMD when the CBW is wider.

Observation 2:  For n25, the RxBN is degraded for 45MHz operation however less than n8 and n71.

Observation 3:  For n3, the RxBN is slightly degraded.

Observation 4:  For n8, n71 at 35MHz CBW and n25 at 45MHz CBW, there are large de-sens since the significant RxBN deterioration.

Proposal 1:  REFSENS for 35/45MHz CBW operation are proposed highlighted as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n3 | 15 | -97 | -93.8 | -92 | -90.8 | -89.7 | -88.9 | -86.0 | -82.3 | -84.2 |  | FDD |
|  | 30 |  | -94.1 | -92.1 | -91 | -89.8 | -89 | -86.1 | -82.4 | -84.3 |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90 | -89.1 | -86.2 | -82.6 | -84.4 |  |  |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -62.2 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -62.3 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -85.4 | -79.5 | -70.7 |  | FDD |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -85.5 | -79.6 | -70.8 |  |  |
|  | 60 |  | -94 | -91.9 | -90.7 | -89.6 | -82.4 | -85.6 | -79.7 | -70.9 |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -67.6 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -67.7 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n3 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | 25 | 50 | 50 | 50 | 50 | 48 | 40 | 40 | 40 |  | FDD |
|  | 30 |  | 24 | 24 | 24 | 24 | 24 | 20 | 20 | 20 |  |  |
|  | 60 |  | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

References

[1]		R4-2011793 WF on UE RF Requirement, Skyworks Solutions Inc., ZTE, Huawei, [Qualcomm]