3GPP TSG-RAN WG4 Meeting #97-e	R4-2015481

# Online, 2nd– 13th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on RRM performance requirements for FR2 new FWA UE in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the agreements in [R4-2012199] in RAN4#96-e meeting, it has been obseved that the side condition, UE gain range and test directions for FR2 RRM tests need to be introduced for FR2 new FWA UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To introduce the test direction configuration for FR2 new FWA UE - To introduce the corresponding condition requirements for FR2 new FWA UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test direction configuration and condition requirements for new FR2 FWA UE are not specified in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.15.3, B.1.2, B.2.1.3, B.2.1.4, B.2.1.5, B.2.2, B.2.3, B.2.4, B.2.5, B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

Editor Note: If RAN5 finds the changing of angular offset between the directions (AoAs) of the 2 active probes per test iteration to be infeasible from the perspectives of EIS spherical coverage and other impacts, e.g.: testing time, then the test setup will be revised.

Table 3.15.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |
| [TBD] | FFS |

<End of Change 1>

<Start of Change 2>

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |  |
|  |  |  | 1 | 2 | 3 | 4 | [TBD] | 1, 2, 3, 4, [TBD] |  |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | [TBD]+Y5 | (Value for SCSSSB = 120 kHz) +3dB |  | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | [TBD]+Y5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 | [TBD]+Y5 |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | [TBD]+Y5 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | [TBD]+Z5 | (Value for SCSSSB = 120 kHz) +3dB |  | ≥-4 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | [TBD]+Z5 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 | [TBD]+Z5 |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | [TBD]+Z5 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.1.2-2:

- The value of Y for Power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1, 4 and [TBD] respectively

- The value of Z for Power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 4 and [TBD] respectively

<End of Change 2>

<Start of Change 3>

B.2.1.3	Derivation of Minimum SSB_RP values for FR2

Editor’s note:

-	The Assumption for UE beams (fine or rough) in Annex A RRM test cases is defined based on power class 3, and unless otherwise stated also applies for other UE power classes

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on reference sensitivity for the Operating band and for the UE power class, taking a baseline of UE power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n

where:

Reference sensitivity PC3, n260, 50MHz is the reference sensitivity value in dBm specified for power class 3 in Band n260 for 50 MHz Channel bandwidth in Table 7.3.2.3-1 of TS 38.101-2 [19];

Y is the gain difference between fine and rough beams, which is defined in Table B.2.1.3.1-1;

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value “Y” in dB, for each UE power class |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | [TBD] |
| FFS | 9.0 | 7.0 | FFS | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE power class 3 in Band n260 is (-109.5+∆MBP,n) dBm/120kHz for intra-frequency measurements and (-107.5+∆MBP,n ) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -109.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +∆MBP,n,

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -107.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +∆MBP,n .

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage

Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on EIS spherical coverage for the Operating band and for the UE power class, taking a baseline of UE power class 3 in Band n260 with 50 MHz channel bandwidth.

Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBS,n,

where:

EIS spherical coverage PC3, n260, 50MHz is the EIS spherical coverage value in dBm specified for power class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.4.3-1;

Z is the gain difference between fine and rough beams, and is defined in Table B.2.1.3.2-1;

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value “Z” in dB, for each UE power class |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | [TBD] |
| FFS | 9.0 | 7.0 | FFS | FFS |

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-2 [19] Table 5.3.2-1, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBS,n is the UE multi-band relaxation factor value in dB specified in TS 38.101-2 [19] clause 6.2.1.

The calculated Minimum SSB_RP value for the baseline of UE power class 3 in Band n260 is (-96.9+∆MBS,n) dBm/120kHz for intra-frequency measurements and is (-94.9+∆MBS,n) dBm/120kHz for inter-frequency measurements.

The following methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is used:

For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -96.9 dBm/120kHz + EIS spherical coverage PC_X, Band_Y, 50MHz – EIS spherical coverage PC3, n260, 50MHz + Z PC_X – Z PC3 +∆MBS,n

For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -94.9 dBm/120kHz + EIS spherical coverage PC_X, Band_Y, 50MHz – EIS spherical coverage PC3, n260, 50MHz + Z PC_X – Z PC3 +∆MBS,n

B.2.1.4	Gain to SS-RSRP measurement point for FR1

In FR1 conducted requirements are specified at the UE antenna connector, which is also the SS-RSRP measurement point.

B.2.1.5	Gain to SS-RSRP measurement point for FR2

B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival

In clause 5.1.1 of TS 38.215 [4] SS-RSRP is defined to be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reference point for requirement parameters from the UE perspective is the input of the UE antenna array. The gain “G” relates the combined signal from antenna elements corresponding to a given receiver branch to the reference point for requirement parameters.

The gain “G” affects absolute signal level values reported by the UE.

Figure B.2.1.5.1-1: Gain and Reference point for requirement parameters

The gain range for each power class is specified in Table B.2.1.5.1-1.

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | [TBD] |
| Minimum, dBi | FFS | FFS | -10 | FFS | FFS |
| Maximum, dBi | FFS | FFS | +20 | FFS | FFS |

Gain range in spherical coverage directions may be lower than in Rx beam peak direction, according to the difference between the EIS spherical coverage value specified in TS 38.101-2 [19] clause 7.3.4 and the Reference sensitivity level specified in TS 38.101-2 [19] clause 7.3.2.

B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival |  |  | NR operating bands |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  |  | n257 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-6 |
|  |  |  |  | n258 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -125.3+Y1 |  |  | -109.5 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  |  | n257 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-6 |
|  |  |  |  | n258 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -117.3+Z1 |  |  | -96.9 |  | -113.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.2-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival |  |  | NR operating bands |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  |  | n257 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-4 |
|  |  |  |  | n258 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -123.3+Y1 |  |  | -107.5 |  | -123.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  |  | n257 |  | -118.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-4 |
|  |  |  |  | n258 |  | -118.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -115.3+Z1 |  |  | -94.9 |  | -111.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -118.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

B.2.4	Conditions for NR L1-RSRP reporting

B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival |  |  | NR operating bands |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  |  | n257 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -122.3+Y1 |  |  | -106.5 |  | -122.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  |  | n257 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -114.3+Z1 |  |  | -93.9 |  | -110.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, and 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival |  |  | NR operating bands |  | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSCSI-RS |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSCSI-RS = 60 kHz |  |  |  |  |  |  |  |  | SCSCSI-RS = 120 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  |  | n257 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSCSI-RS = 60 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -125.3+Y1 |  |  | -109.5 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -128.3+Y1 |  | -113.8 | -112.1 |  | -127.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  |  | n257 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSCSI-RS = 60 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -117.3+Z1 |  |  | -96.9 |  | -113.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -120.3+Z1 |  | -102.8 | -101.2 |  | -118.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.2-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

B.2.5	Conditions for RRC connection release with redirection to NR

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival |  | NR operating bands |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  | n257 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-4 |
|  |  |  | n258 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  | n260 |  | -123.3+Y1 |  |  | -107.5 |  | -123.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  | n261 |  | -126.3+Y1 |  | -111.8 | -110.1 |  | -125.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  | n257 |  | -118.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-4 |
|  |  |  | n258 |  | -118.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  | n260 |  | -115.3+Z1 |  |  | -94.9 |  | -111.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  | n261 |  | -114.3+Z1 |  | -100.8 | -99.2 |  | -116.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

B.2.6	Conditions for UE transmit timing

B.2.6.1	Conditions for SSB based UE transmit timing

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival |  |  | NR operating bands |  | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  |  |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | dBm / SCSSSB |  |  |  |  |  |  |  |  |  |  |  | dB |
|  |  |  |  |  |  | SCSSSB = 120 kHz |  |  |  |  |  |  |  |  | SCSSSB = 240 kHz |  |  |  |
|  |  |  |  |  |  | UE Power class |  |  |  |  |  |  |  |  | UE Power class |  |  |  |
|  |  |  |  |  |  | 1 |  | 2 | 3 |  | 4 |  | [TBD] |  | 1, 2, 3, 4, [TBD] |  |  |  |
| Conditions | Rx Beam Peak |  |  | n257 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -122.3+Y1 |  |  | -106.5 |  | -122.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -125.3+Y1 |  | -110.8 | -109.1 |  | -124.8+Y4 |  | [TBD]+Y5 |  |  |  |  |  |
|  | Spherical coverage Note 1 |  |  | n257 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  | (Value for SCSSSB = 120 kHz) +3dB |  |  | ≥-3 |
|  |  |  |  | n258 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n260 |  | -114.3+Z1 |  |  | -93.9 |  | -110.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
|  |  |  |  | n261 |  | -117.3+Z1 |  | -99.8 | -98.2 |  | -115.8+Z4 |  | [TBD]+Z5 |  |  |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.6.1-2:

- The value of Y for power classes 1, 4 and [TBD] is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and [TBD] respectively

- The value of Z for power classes 1, 4 and [TBD] is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and [TBD] respectively

<End of Change 3>