3GPP TSG-RAN WG4 Meeting # 97-e 	                                                            R4-2015525

Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RSSI and CO performance requirements for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-09-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RSSI measurement report mapping and accuracy requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RSSI measurement report mapping and accuracy requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.X and 10.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.X	RSSI measurement accuracy requirements under operation with CCA

#### 10.1.X.1	RSSI measurement report mapping

The reporting range of RSSI measurement is defined from -100 dBm to -25 dBm with 1 dBm resolution.

The mapping of the measured quantity is defined in table 10.1.X.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.X.1-1: RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSSI_00 | RSSI < -100 | dBm |
| RSSI_01 | -100 RSSI < -99 | dBm |
| RSSI_02 | -99 RSSI < -98 | dBm |
| … | … | … |
| RSSI_74 | -27 RSSI < -26 | dBm |
| RSSI_75 | -26 RSSI < -25 | dBm |
| RSSI_76 | -25 RSSI | dBm |

#### 10.1.X.2	RSSI measurement accuracy requirements

The RSSI measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.X.2.1-1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

Table 10.1.X.2.1-1: RSSI accuracy

| Accuracy |  | Conditions |  |  |
| --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io | Maximum Io |
| dB |  | dB |  |  |  | dBm/15kHz |  | dBm/BWChannel |  |
| 3.5 |  | 6.5 |  | TBD |  | TBD |  | -70 |  |
| 5.5 |  | 8.5 |  | Note 3 |  | Note 3 |  | -50 |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |

<End of Change 1>

<Start of Change 2>

10.1.Y	Channel occupancy measurement accuracy requirements under operation with CCA

The UE shall be able to correctly evaluate the configured according to 38.331 [2], provided that the following conditions are met:

-	RSSI at the UE receiver meets the following condition with respect to the configured channelOccupancyThreshold-r16 [2]:

RSSI at the UE receiver is below channelOccupancyThreshold-r16 -, or

RSSI at the UE receiver is above channelOccupancyThreshold-r16 +,

where  is the applicable RSSI measurement accuracy value from the RSSI measurement accuracy requirements specified in Section 10.1.X.2.

<End of Change 2>