3GPP TSG-RAN WG4 Meeting # 97-e                                  R4-2015555

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on asynchronous for DC_42_n79

Agenda Item:	7.19.3

Document for:	Approval

Introduction

In RAN4#96, companies found whether DC_42_n79 supports simultaneous Tx/Rx was ambiguous since Rel-15 [1]. It can’t be supported by solutions implemented with n77 or n78 filter without MSD. However, most of UEs have been designed to reuse the C-band RF chain to implement the band 42. This paper will discuss this issue.

Discussion

Based on the current specification, there is no additional clarification and note about simultaneous/non-simultaneous Tx/Rx operation for DC_42_n79. It means this feature is optional. UE can choose to support simultaneous Tx/Rx for DC_42_n79 and meet all the RF requirements or not to support simultaneous Tx/Rx. As RAN4 discussed this issue previously, it can’t be supported by solutions implemented with n77 or n78 filter without MSD. Therefore, there are two options to address this issue.

Option 1: Adding a note that the minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers.

Option 2: Evaluating the MSD due to cross band isolation between band 42 and n79.

Generally, it’s assumed that the separated antenna architecture is used for this DC band combinations. The architecture frame is shown below.

Figure 1 The architecture frame for DC_42_n79

The MSD evaluation is shown below in table 1 for aggressor band 42 Tx and victim band n79. The MSD evaluation is shown below in table 2 for aggressor band n79 Tx and victim band 42.

Table 1 The MSD evaluation for aggressor band 42 Tx and victim band n79.

| item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| B42 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| B42 PA output noise power at n79 freq | dBm/Hz | （20MHz UL） | -133 | -133 |
| Antenna isolation Primary <-> Diversity |  | 15 |  |  |
| n79 Rx bandwidth | MHz | 40 |  |  |
| B42 PA output noise power |  |  | -57.43697 | -57.436975 |
| B42 SAW TX to COM port on n79 RX freq | dB | 20 |  |  |
| B41 PA noise power at DUP- COM port |  |  | -92.43697 | -92.436975 |
| front end loss |  | 4 |  |  |
| B42 PA noise power at ant port |  |  | -88.43697 | -88.436975 |
| noise figure dB | dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -165 |  |  |
| Thermal noise, dBm |  |  | -88.9794 | -88.9794001 |
| TX IM2 noise dBm |  |  | -102 | -102 |
| totle noise level at ANT port, dBm |  |  | -85.58903 | -85.589033 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  |  | -86.58903 | -86.589033 |
| combined REFSENS(40MHz BW), dBm |  |  | -89.59933 |  |
| MSD | dB |  | 0.0006671 |  |

Table 2 The MSD evaluation for aggressor band n79 Tx and victim band 42.

| item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n79 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n79 PA output noise power at B42 freq | dBm/Hz | （100MHz UL） | -130 | -130 |
| Antenna isolation Primary <-> Diversity |  | 15 |  |  |
| B42 Rx bandwidth | MHz | 5 |  |  |
| n79 PA output noise power |  |  | -63.0103 | -63.0103 |
| n79 SAW TX to COM port on n42 RX freq | dB | 20 |  |  |
| n79 PA noise power at DUP- COM port |  |  | -98.0103 | -98.0103 |
| front end loss |  | 4 |  |  |
| n79 PA noise power at ant port |  |  | -94.0103 | -94.0103 |
| noise figure dB | dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -165 |  |  |
| Thermal noise, dBm |  |  | -98.0103 | -98.0103 |
| TX IM2 noise dBm |  |  | -102 | -102 |
| totle noise level at ANT port, dBm |  |  | -92.08749 | -92.0874897 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  |  | -93.08749 | -93.0874897 |
| combined REFSENS(5MHz BW), dBm |  |  | -96.09779 |  |
| MSD | dB |  | 2.9022104 |  |

Proposal 1: To introduce MSD values as below between band 42 and n79 if UE choose to support simultaneous Tx/Rx for DC_42_n79.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 42 | n79 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| n79 | 42 | 2.9 | 2.9 | 2.9 | 2.9 |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 42 | n79 | 15 |  |  |  |  |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |

Conclusion

Based on the analysis above, we present our proposal as below.

Proposal 1: To introduce MSD values as below between band 42 and n79 if UE choose to support simultaneous Tx/Rx for DC_42_n79.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 42 | n79 |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| n79 | 42 | 2.9 | 2.9 | 2.9 | 2.9 |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 42 | n79 | 15 |  |  |  |  |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |

- References

[1]	R4-2011460, “CR to 38.101-3 - Correction DC_42_n79 simultaneous Tx/Rx operation”, Skyworks Solutions Inc.