3GPP TSG-RAN WG4 Meeting #97-e	R4-2015581

Electronic Meeting, 2 Nov. – 13 Nov., 2020

Source:	ZTE

Title:	Test case list for SRS carrier based switching

Agenda item:	7.13.2.2.1

Document for:	Approval

1. Introduction

In the RAN4#95-e meeting, core part for SRS carrier based switching have been completed. In this contribution we provide test case list for SRS carrier based switching.

2. Discussion

Interruption requirements at NR SRS carrier based switching are specified for EN-DC, NE-DC, NR SA (Carrier aggregation) and NR –DC operation.

Interruption requirements at E-UTRA SRS carrier based switching are specified for EN-DC and NE-DC operation mode.

To reduce test number it would be better not to test interruption at SRS carrier based switching under each operation mode. For NR SRS carrier based switching it can be tested under EN-DC and NR SA (carrier aggregation) operation mode. For E-UTRA carrier based switching it only needs to be tested under EN-DC.

For NR SRS carrier based switching, the tests should be specified for both FR1 and FR2.

Proposal 1: For SRS carrier based switching, following test cases are specified.

| Test No. | Test | Comment |
| --- | --- | --- |
| TC1 | E-UTRAN – NR interruptions at NR SRS carrier based switching | PSCell in FR1 SCell in FR1 |
| TC2 | E-UTRAN – NR interruptions at NR SRS carrier based switching | PSCell in FR2 SCell in FR2 |
| TC3 | SA interruptions at NR SRS carrier based switching | PCell in FR1 SCell in FR1 |
| TC4 | SA interruptions at NR SRS carrier based switching | PCell in FR2 SCell in FR2 |
| TC5 | E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching | PSCell in FR1 E-UTRA SCell |
| TC6 | E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching | PSCell in FR2 E-UTRA SCell |

3. Conclusion

In this contribution we further provided our views on test case list for SRS carrier based switching. Following proposals are present.

Proposal 1: For SRS carrier based switching, following test cases are specified.

| Test No. | Test | Comment |
| --- | --- | --- |
| TC1 | E-UTRAN – NR interruptions at NR SRS carrier based switching | PSCell in FR1 SCell in FR1 |
| TC2 | E-UTRAN – NR interruptions at NR SRS carrier based switching | PSCell in FR2 SCell in FR2 |
| TC3 | SA interruptions at NR SRS carrier based switching | PCell in FR1 SCell in FR1 |
| TC4 | SA interruptions at NR SRS carrier based switching | PCell in FR2 SCell in FR2 |
| TC5 | E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching | PSCell in FR1 E-UTRA SCell |
| TC6 | E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching | PSCell in FR2 E-UTRA SCell |

4. References

- TS 38.133 V16.5.0