3GPP TSG-RAN WG4 Meeting #97-e	R4-2015582

Electronic Meeting, 2 Nov. – 13 Nov., 2020

Source:	ZTE

Title:	Test case list for mandatory gap pattern

Agenda item:	7.13.2.2.6

Document for:	Approval

1. Introduction

In the RAN4#95-e meeting, core part for mandatory gap patterns has been completed. Additional mandatory gap patterns with new UE capability were agreed. In this contribution we provide views on test cases for mandatory gap pattern.

2. Discussion

There is no specific test to verify UE’s functionality of measurement gap. It is verified implicitly in event triggered reporting tests. In existing test cases, for per-UE gap capable UE GP#0 is always used in the tests. For per-FR gap capable UE GP#4 is always used in the test if neighbor cell is in FR1 and GP#13 is always used in the test if neighbor is in FR2. All of the gap patterns used in the test have longest MGL. The main purpose of additional mandatory gap pattern is to have reduced MGL. So in order to verify that UE can function correctly with short MGL gap patterns, tests are needed for the new mandatory gap pattern.

Event triggered reporting test can still be used to verify new mandatory gap patterns. It would be enough to verify only one gap pattern per FR. Since this is mainly functional tests, the test step should be chosen to reduce test time as much as possible. GP#2 and GP#17 with 40ms MGRP can be selected for the test. In the test, no DRX is configured and no SSB index detection is needed.

Proposal 1: For additional mandatory gap patterns, following test cases are specified.

| Test No. | Test | Comment |
| --- | --- | --- |
| TC1 | SA event triggered reporting tests with additional mandatory gap pattern | PCell in FR1 Neighbor cell in FR1 |
| TC2 | SA event triggered reporting tests with additional mandatory gap pattern | PCell in FR2 Neighbor cell in FR2 |

3. Conclusion

In this contribution we further provided our views on test case list for mandatory gap pattern. Following proposals are present.

Proposal 1: For additional mandatory gap patterns, following test cases are specified.

| Test No. | Test | Comment |
| --- | --- | --- |
| TC1 | SA event triggered reporting tests with additional mandatory gap pattern | PCell in FR1 Neighbor cell in FR1 |
| TC2 | SA event triggered reporting tests with additional mandatory gap pattern | PCell in FR2 Neighbor cell in FR2 |

4. References

- TS 38.133 V16.5.0