3GPP TSG-RAN WG4 Meeting #97-e	R4-2015594

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Discussion on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements demodulation performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.5.4

Document for:	Discussion

Background

As per status report [1], demodulation performance for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements is started for discussion in this meeting. In this contribution, we share our views about Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements demodulation performance requirements.

Discussion

For Multi-RAT Dual-Connectivity and Carrier Aggregation enhancement, RAN1 physical layer design and corresponding analytics of the impact on demodulation performance is shown below:

Power control for dual connectivity

Semi-static power sharing and dynamic power sharing is supported between MCG and SCG. From perspective of RAN4, it has no impact on demodulation part.

Enhancements to single Tx switched uplink solution for EN-DC

A new reference HARQ configuration is introduced for EN-DC for LTE TDD PCell, as the following Figure 2-1 shows. Different HARQ-offsets for different UEs to avoid all HARQ-ACK traffic concentrate in a subset of uplink subframes. For UE that incapable of dynamic power sharing between NR & LTE, any LTE UL transmissions can only take place in UL subframes designated for HARQ-ACK feedback. For UE that capable of dynamic power sharing between NR & LTE, LTE UL transmission scheduled by DCI can take place in UL subframes not designated for HARQ-ACK feedback. Semi-statically LTE UL transmissions are allowed in all UL subframes and prioritized over NR UL transmission under collision scenario.

Figure 2-1 Enhancements to single Tx switched uplink solution for EN-DC

From perspective of RAN4, the limitation for uplink HARQ response position is not UE performance related change.

Efficient and low latency serving cell configuration/activation/setup

To reduce power consumption of UE in SCells but also keep the SCells configuration and can fast return to SCells utilization for data transfer, dormant SCells are introduced. UE in dormant SCells is still in the activated state, but UE behaviour is different from the normal activated state.

For physical layer design, RAN 1 introduce an explicit information field to the PDCCH WUS when UE is outside active time and an explicit information field to DCI formats 0-1/1-1 for the primary cell when UE is in active time. The explicit information field for SCell dormancy indication is a bitmap that the length is the number of configured SCell groups for the UE and up to 5 bit, and each bit in the bitmap corresponds to one configured SCell groups.

When UE is within active time, there are two cases specified as following:

- Case 1: PDCCH schedules data for primary cell and also indicates dormancy for SCells.

- Case2: PDCCH with DCI format 1-1 is used for indicating dormancy for SCells, and UE is indicated that the PDCCH is not used for PDSCH scheduling.

For Case 2, ACK is transmitted by the UE in response to detection of Case 2 PDCCH with SCell dormancy indication for Type 2 codebook while no HARQ response is supported in response to detection of Case 2 PDCCH with SCell dormancy indication for Type 1 codebook.

From perspective of RAN4, although new HARQ response mechanism in introduced for Type 2 codebook for Case 2 when UE is within active time, there is no new algorithms are introduced on PDCCH demodulation with only total number of payload bits increased.

Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers

The case that scheduling cell of lower SCS with scheduled cell of higher SCS and the case that scheduling cell of higher SCS with scheduled cell of lower SCS is introduced. From perspective of RAN4, it is a new feature for scheduling but has no impact on the final demodulation performance.

Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

The case that PDCCH of lower SCS with CSI-RS of higher SCS and the case that PDCCH of higher SCS with CSI-RS of lower SCS is introduced. From perspective of RAN4, it is a new feature for scheduling but has no impact on the demodulation performance.

Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA

Explicit RRC signalling of slot offset is configured to the UE for unaligned frame boundary for inter-band CA to ensure that there is a unique interpretation for timing in scheduled cell for cross-carrier operation. From perspective of RAN4, it is a functional change and no impact on demodulation performance.

As per above, we can make the conclusion that there is no need to introduce any performance requirements for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.

Proposal 1: Do not introduce any performance requirements for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.

Proposals

In this contribution, we discuss on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements demodulation performance requirements. Our observations and proposals are:

Proposal 1: Do not introduce any performance requirements for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements.

Reference

RP-201532, Status report for WI on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements, RAN#89-e, Ericsson