3GPP TSG-RAN WG4 Meeting #97-e	R4-2015597

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Discussion on PDSCH requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	7.10.1.1

Document for:	Discussion

Background

During RAN4#95-e meeting, way forward [1] for NR DL 256QAM for FR2 was approved. In this contribution, we share our views on PDSCH requirements for NR DL 256QAM for FR2.

Simulations

The following simulation results is based on the agreed simulation assumption in WF [1].

Table 2-1 Simulation results for PDSCH demodulation

| Case number | CBW/MHz | MIMO | Rank | Propagation condition | MCS | SNR@70%max throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 1 | 50 | 2x2 | 1 | TDL-D30-35 | 20 | 16.70 | 19.20 |
| 2 | 50 | 2x2 | 1 | TDL-A30-300 | 20 | 17.94 | 20.44 |

Observation 1: Both Propagation condition of TDLD30-35 and TDLA30-300 is feasible.

Discussion

- Propagation condition - Use fading channel - In the next meeting, companies are encouraged to provide ideal and impairment results for both option 1A and option 1B, and down select one of the two options based on simulation results. - Option 1A: TDLA30-300 - Option 1B: TDLD30-75 - Note: extra effort on TDLD channel model simplification is needed.

The only one remained issue is selection of the propagation condition. As per our simulation result in Section 2, also considering extra effort on TDL-D channel model simplification is needed, we prefer TDLA30-300 for requirements definition for NR DL 256QAM for FR2.

Proposal 1: Define requirements for NR DL 256QAM for FR2 with the propagation condition of TDLA30-300.

Proposals

In this contribution, we discuss on PUSCH requirements for NR DL 256QAM for FR2. Our observations and proposals are:

Observation 1: Both Propagation condition of TDLD30-35 and TDLA30-300 is feasible.

Proposal 1: Define requirements for NR DL 256QAM for FR2 with the propagation condition of TDLA30-300.

Reference

R4-2012666, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4#96-e, China Telecom