3GPP TSG-RAN WG4 Meeting #97-e	R4-2015599

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Discussion and simulation results on SDR requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	7.10.1.2

Document for:	Discussion

Background

During RAN4#96-e meeting, way forward [1] for NR DL 256QAM for FR2 was approved. The open issues for SDR requirements are shown below.

- Whether to define SDR requirements for FR2 256QAM - Option 1: Add 256QAM (modulation format of 8) to FR2 SDR requirements - Option 2: Not to define FR2 SDR requirements for 256QAM - If it is agreed to define SDR requirements, consider the following test parameters: - MCS and rank - Option 1: Add MCS indexes 26, 21, 20 and 11 in MCS table 2 for both 1 and 2 MIMO layers in the Table 7.5A.1-3. Run simulations to derive the required SNR at 85% throughput for MCS 20 to MCS 26 in MCS table 2, with both 1 layer and 2 layers. - Other options are not precluded.

In this contribution, we share our views on SDR requirements for NR DL 256QAM for FR2.

Discussion

For SDR test cases, the mainly factor of testability is the testable SNR, which is in a certain range due to the limitation of the transmitted signal power of TE. The maximum testable SNR can be derived as per the following equations referenced from TS 38.810 and the calculated results is shown as Table 2-1.

SNRmax = Ps,max / Pnoise,Σ Pnoise,Σ = NnoiseRF * BW Ptotal,max = Ps,max SNRmax = Ptotal,max / (NnoiseRF * BW)

Table 2-1: Maximum testable SNR for single band UE

| Channel Bandwidth/MHz | Maximum SNR/dB |  |  |
| --- | --- | --- | --- |
|  | DNF | DFF | IFF |
| 50 | 32.75 | 29.55 | 29.85 |
| 100 | 29.87 | 26.67 | 26.97 |
| 200 | 26.86 | 23.66 | 23.96 |
| 400 | 23.85 | 20.65 | 20.95 |
| 500 | 22.88 | 19.68 | 19.98 |
| 600 | 22.09 | 18.89 | 19.19 |
| 700 | 21.42 | 18.22 | 18.52 |
| 800 | 20.84 | 17.64 | 17.94 |
| 900 | 19.83 | 16.63 | 16.93 |
| 1000 | 19.37 | 16.17 | 16.47 |

Considering lower testable SNR can be observed by using DFF method comparing to other two methods, we can assume that DFF method is used.

As per slide 7 in WF [1], we provide the following simulation results:

Table 2-2 SNR required to achieve 85% of peak throughput under AWGN conditions

| MCS Index | Ideal simulation results |  | Impairment simulation results |  |
| --- | --- | --- | --- | --- |
|  | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 1 | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 2 | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 1 | SNRBB(dB) for maximum number of PDSCH MIMO Layers = 2 |
| 20 | 15.78 | 18.65 | 18.28 | 21.15 |
| 21 | 16.55 | 19.39 | 19.05 | 21.89 |
| 22 | 17.21 | 20.13 | 19.71 | 22.63 |
| 23 | 18.61 | 21.48 | 21.01 | 23.98 |
| 24 | 19.73 | 22.66 | 22.23 | 25.16 |
| 25 | 20.70 | 23.51 | 23.20 | 26.01 |
| 26 | 21.56 | 24.60 | 24.06 | 27.10 |

From the simulation results we can see the following observation:

Observation 1: Considering extra 0.8dB margin,

- For maximum supporting MIMO layer 1, MCS 26 can be test only for the bandwidth less than 200MHz, MCS 20 can be test for the bandwidth less than 500MHz.

- For maximum supporting MIMO layer 2, MCS 26 can be test only for the bandwidth of 50MHz, MCS 20 can be test for the bandwidth less than 200MHz.

In Rel-15, the MCS for SDR testing is mainly limited by UE capabilities. Only for some situations that large bandwidth combination with strong UE capability, the MCS for SDR testing is limited by TE maximum achievable SNR, e.g. MCS 27 of MCS Table 1 cannot be tested for UE supporting bandwidth combination of 1000MHz. For 256QAM, the MCS for SDR testing is mainly limited by TE maximum achievable SNR, and 256QAM will not be tested in most of cases. Therefore, we propose to not define SDR requirements for FR2 256QAM.

Proposal 1: Do not define SDR requirements for FR2 256QAM for Rel-16.

Proposals

In this contribution, we discuss on SDR requirements for NR DL 256QAM for FR2. Our observations and proposals are:

Observation 1: Considering extra 0.8dB margin,

- For maximum supporting MIMO layer 1, MCS 26 can be test only for the bandwidth less than 200MHz, MCS 20 can be test for the bandwidth less than 500MHz.

- For maximum supporting MIMO layer 2, MCS 26 can be test only for the bandwidth of 50MHz, MCS 20 can be test for the bandwidth less than 200MHz.

Proposal 1: Do not define SDR requirements for FR2 256QAM for Rel-16.

Reference

R4-2012666, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4#96-e, China Telecom