3GPP TSG-RAN WG4 Meeting #97-e	R4-2015604

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Discussion on UE performance requirements for DPS transmission scheme

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.1.1

Document for:	Discussion

Background

During RAN4#96-e, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the DPS transmission scheme for NR Rel-16 UE HST performance requirements.

Discussion

DPS 1a

Test setup

At last meeting, RAN4 agreed the following test setup for DPS 1a:

- Two RRH s of RRH#(2k) and RRH#(2k+1) are assumed, and SSB#0 is transmitted from both TRPs, where k is the RRH number with k=0,1, 2, … - UE is configured with TCI#(k mod 2) and TCI#(k+1 mod 2) that are associated with TRS#(k mod 2) and TRS#(k+1 mod 2) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signaling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signaling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP measurements are not tested) - TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - PDSCH associated with TCI #0 is transmitted during the slots from 0 to (n-1) + HARQ needed time + 3ms + first TRS + TRS processing time; - In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE”; - PDSCH associated with TCI #1 is transmitted in slots from n + HARQ needed time + 3ms + first TRS + TRS processing time to N. PDSCH associated with TCI #(k mod 2) (k=0,1,2,…) is transmitted in slot from max((2k-1)n, 0) to ((2k+1)n-1) + HARQ needed time + 3ms + first TRS + TRS processing, where n slots are equivalent to time that needed to pass middle point between two RRHs, N slots is equivalent to time that needed to pass second RRH. And k is the RRH number in the channel model.

There will exist some area in which both sides transmit signals to UE as per the generalized wording (yellow highlighted lines above), as shown in Figure 2.1.1-1 below. To remove the overlapping part, PDSCH associated with TCI #(k mod 2) (k=0,1,2,…) should be transmitted in slot from max((2k-1)n + HARQ needed time + 3ms + first TRS + TRS processing, 0) to ((2k+1)n-1) + HARQ needed time + 3ms + first TRS + TRS processing.

Figure 2.1.1-1 The overlapped area

Proposal 1: For test setup for DPS 1a, modify test setup as following:

- PDSCH associated with TCI #(k mod 2) (k=0,1,2,…) is transmitted in slot from max((2k-1)n + HARQ needed time + 3ms + first TRS + TRS processing, 0) to ((2k+1)n-1) + HARQ needed time + 3ms + first TRS + TRS processing, where n slots are equivalent to time that needed to pass middle point between two RRHs, N slots is equivalent to time that needed to pass second RRH. And k is the RRH number in the channel model.

Test parameters

- Transmission scheme 1a - Rank 2 - PDSCH is scheduled in DL, FFS for special slots - TRS processing time: 2ms - MCS - Option 1: MCS 13 based on 64QAM table (same as HST-SFN) - Option 2: MCS 17 based on 64QAM tables - HARQ timing - FDD 15kHz SCS: 2 slots - TDD 30kHz SCS with TDD pattern 7D1S2U: {8,7,6,5,5,4,3,2} slots

PDSCH scheduling in special slots

As per section 7 of TS 38.211 [2], we show the resource allocation for downlink slots and special slots as Figure 2.1.2-1 shows. In theory, the performance become worse in special slot for SFN or multi-path fading channel since that only one column of DMRS cannot handle large Doppler spread well when perform channel estimation and other procedures. However, in DPS scenario, there is no large Doppler spread, therefore the performance should not be deteriorated significantly.

Figure 2.1.2-1: Resource allocation for download slots and special slots

Also, as per our contribution [3], our simulation results for comparing that performance difference between the configuration that PDSCH is scheduled in special slots or not are shown as Figure 2.1.2-2 below.

Figure 2.1.2-2: Transmission in ‘D+S’ or ‘D’

From the simulation results we can see that there is negligible performance difference when PDSCH is scheduled in special slots or not. Therefore, it is unnecessary to disable PDSCH transmission in special slots which is same as single-tap scenario.

Proposal 2: For DPS 1a, PDSCH should be scheduled in special slots.

MCS

As per our contribution [3], the MCS 17 is feasible for DPS 1a scenario, so it is more appropriate to use MCS 17 to achieve higher throughput.

Proposal 3: Use MCS 17 for DPS 1a.

DPS 1b

Number of active TCI states

- Number of active TCI states in DPS transmission scheme 1b - Option 1: with 2 active TCI states. - Option 2: with 2 and more than 2 active TCI states.

For DPS 1b, there will not exist TCI state switching delay if UE supports more than two active TCI states tracking while switching delay of HARQ needed time + 3ms is needed if UE supports two active TCI states, but considering the UE capability to support more than 2 active TCI states and the WI completion date, it is fine for us to only define performance requirements for DPS 1b with 2 active TCI states.

Proposal 4: Only define PDSCH performance requirements with 2 active TCI states.

Test setup

- Test setup for transmission scheme 1b - Use following as baseline for DPS transmission scheme 1b test setup (agreement in last meeting) - for scenario with 2 active TCI states (total 2 active TCI states): PDCCH TCI state switching delay caused by MAC CE, but less than DPS 1a with pre-tracking of second TCI state and only HARQ needed time + 3ms delay is needed, UE tracks 2 active TCI states in advance so that UE can quickly get better Doppler shift estimation for the second TRP compared to DPS 1a. - UE is configured with two different TCI states (TCI #0 and TCI #1) associated with two different RRHs by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - TE activates TCI #0 and TCI #1 for PDSCH at the same time by “TCI States Activation/Deactivation for UE-specific PDSCH MAC CE” and activates TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE” command with the field of CORESET ID set to 0; - TE transmits PDCCH and PDSCH associated with TCI #0 from TRP#1 from slot 0 to n-1; - In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE”; - TE transmits PDCCH and PDSCH associated with TCI #1 from TRP#2 from slot n [+ HARQ needed time + 3ms] to N. where n slots are equivalent to time that needed to pass middle point between two RRHs, N slots is equivalent to time that needed to pass second RRH - for scenario with 3 active TCI states (Total 3 active TCI states): No PDCCH TCI state switching delay by using MAC CE, but 3 active TCI states to track, UE needs to report supporting of maxNumberActiveTCI-PerBWP = n4 - UE is configured with three different TCI states (TCI #0, TCI #1 and TCI #2) associated with two different RRHs by RRC signalling tci-StatesToAddModList in the PDSCH-Config; - TE activates TCI #0 and TCI #1 for PDSCH at the same time by “TCI States Activation/Deactivation for UE-specific PDSCH MAC CE” and activates TCI #2 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE” command with the field of CORESET ID set to 0; - TE transmits PDCCH associated with TCI#2 from TRP#1 and TRP#2 from slot 0 to N - DCI contains pointer to TCI#0 from slot 0 to n-1 and pointer to TCI#1 from slot n to N - TE transmits PDSCH associated with TCI #0 from TRP#0 from slot 0 to n-1 - TE transmits PDSCH associated with TCI #1 from TRP#1 from slot [n or n+1] to N where n slots are equivalent to time that needed to pass middle point between two RRHs, N slots is equivalent to time that needed to pass second RRH

For scenario with 2 active TCI states, the test setup described in the WF is feasible.

For scenario with 3 active TCI states, at the step 6, the remained issue is that in which slot TE transmits PDSCH associated with TCI #1 from TRP#1. In our view, slot n is suitable considering that UE is tracking active TCIs for both TRP#0 and TRP#1, no additional 1 slot is needed for UE to switch TCI state.

Proposal 5: For scenario with 3 active TCI states, TE transmits PDSCH associated with TCI #1 from TRP#1 from slot n to N

Statistics for both DPS 1a and 1b

- Statistics during the switching time for both DPS 1a and 1b - Option 1: The switch command is transmitted via MAC CE, the corresponding PDSCH carrying that MAC CE should be ensured to be decoded successfully and lower MCS should be used, such as MCS 4. - Option 2: PDCCH/PDSCH are DTXed from the time gNB indicates MAC CE TCI state switch to the time UE receive the first TRS from the new TRP. - Option 3: Use same SNR point for all DPS Tx schemes requirements definition: - Skip PDSCH allocation on slots with TRS transmission - Skip PDSCH allocation on slots from n to m, where n slots are equivalent to time that needed to pass middle point between two RRH and m is a slot which corresponds to HARQ needed time on MAC CE command in DPS scheme 1a. - Other options are not precluded.

The performance difference between different DPS transmission schemes is mainly caused by different UE behavior during the switching period. If same SNR point with only test metric of 70% maximum throughput is used for all DPS transmission schemes for requirements definition, the tests cannot distinguish the performance difference during the switching period and cannot figure out different UE processing capability, even cannot distinguish whether UE support only 1 active TCI or more than 1 active TCI states, it will result in the difference between DPS 1a and DPS 1b is only the test setup, it is not the test purpose that RAN4 define the performance requirements for both DPS 1a and DPS 1b.

To solve this issue, a different test metric should be investigated. In section 8.8.2 of TS 36.101 for LTE EPDCCH performance requirements definition, test metric “EPDCCH subframe monitoring is configured and the subframe monitoring requirement in EPDCCH restricted subframes is statDTX of 99.9%.” is used to check if UE correctly performs the EPDCCH subframe monitoring in EPDCCH restricted subframes. Similarly we can set the PDSCH subframe decoding requirements during the switching period besides the 70% maximum throughput with the checking of correct PDSCH decoding after TCI state switching, we propose:

Proposal 6: For DPS requirements definition, besides the 70% maximum throughput, define an extra test metric that,

- for DPS 1a, UE should meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n + HARQ needed time + 3ms + first TRS + TRS processing during the test

- for DPS 1b with 2 active TCI states, UE meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n + HARQ needed time + 3ms during the test

- for DPS 1b with more than 2 active TCI state, UE meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n+1 during the test

Proposals

In this contribution, we discuss on NR UE HST performance requirements for DPS transmission scheme. Our observations and proposals are:

Proposal 1: For test setup for DPS 1a, modify test setup as following:

- PDSCH associated with TCI #(k mod 2) (k=0,1,2,…) is transmitted in slot from max((2k-1)n + HARQ needed time + 3ms + first TRS + TRS processing, 0) to ((2k+1)n-1) + HARQ needed time + 3ms + first TRS + TRS processing, where n slots are equivalent to time that needed to pass middle point between two RRHs, N slots is equivalent to time that needed to pass second RRH. And k is the RRH number in the channel model.

Proposal 2: For DPS 1a, PDSCH should be scheduled in special slots.

Proposal 3: Use MCS 17 for DPS 1a.

Proposal 4: Only define PDSCH performance requirements with 2 active TCI states.

Proposal 5: For scenario with 3 active TCI states, TE transmits PDSCH associated with TCI #1 from TRP#1 from slot n to N

Proposal 6: For DPS requirements definition, besides the 70% maximum throughput, define an extra test metric that,

- for DPS 1a, UE should meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n + HARQ needed time + 3ms + first TRS + TRS processing during the test

- for DPS 1b with 2 active TCI states, UE meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n + HARQ needed time + 3ms during the test

- for DPS 1b with more than 2 active TCI state, UE meet probability of 99% (ACK and NACK) transmission for all PDSCH scheduled at each switching time point of (2k+1)n+1 during the test

Reference

R4-2012668, WF on UE demodulation for NR HST, RAN4#96-e, CMCC

TS 38.211, Physical channels and modulation

R4-2015605, Simulation results on UE performance requirements for DPS 1a transmission scheme, RAN4#97-e, Huawei, HiSilicon