3GPP TSG-RAN WG4 Meeting #97-e	R4-2015605

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Simulation results on UE performance requirements for DPS 1a transmission scheme

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.1.1

Document for:	Information

Background

During RAN4#96-e meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we provide our simulation results on NR Rel-16 UE HST demodulation requirements for SFN for alignment. The simulation assumption is derived from WF [1] as Table 1-1 below.

Table 1-1 Simulation assumption for NR UE HST DPS 1a scenario

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, FFS whether PDSCH is scheduled in special slots |  |
| MCS | Option 1: MCS 13 based on 64QAM table Option 2: MCS 17 based on 64QAM table |  |
| Propagation condition | HST-DPS (Assume two TRPs for HST-DPS transmission scenario) |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=700m, Dmin=150m |  |
| Rank | Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 870Hz | 1667Hz |
| HARQ needed time/slots | 2 | 8,7,6,5,5,4,3,2 |
| TRS processing time | 2ms |  |
| Testing metric | SNR @70% of maximum throughput |  |
| Note: The other parameters are same as Rel-15 |  |  |

Simulations

The simulation results is shown in the table 2-1 and Figure 2-1 below.

Table 2-1: Ideal Simulation results for NR UE HST DPS 1a scenario

| Case Number | Antenna configuration | CHBW/SCS | PDSCH scheduling in special slot | MCS | maximum Doppler shift(Hz) | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2x2 | 10MHz/15kHz | N/A | 13 | 870 | 7.56 |
| 2 | 2x4 | 10MHz/15kHz | N/A | 13 | 870 | 5.10 |
| 3 | 2x2 | 10MHz/15kHz | N/A | 17 | 870 | 11.47 |
| 4 | 2x4 | 10MHz/15kHz | N/A | 17 | 870 | 9.24 |
| 5 | 2x2 | 40MHz/30kHz | Enabled | 13 | 1667 | 8.11 |
| 6 | 2x4 | 40MHz/30kHz | Enabled | 13 | 1667 | 5.68 |
| 7 | 2x2 | 40MHz/30kHz | Enabled | 17 | 1667 | 11.90 |
| 8 | 2x4 | 40MHz/30kHz | Enabled | 17 | 1667 | 10.01 |
| 9 | 2x2 | 40MHz/30kHz | Disabled | 13 | 1667 | 8.10 |
| 10 | 2x4 | 40MHz/30kHz | Disabled | 13 | 1667 | 5.66 |
| 11 | 2x2 | 40MHz/30kHz | Disabled | 17 | 1667 | 11.89 |
| 12 | 2x4 | 40MHz/30kHz | Disabled | 17 | 1667 | 9.98 |

Figure 2-1: Ideal Simulation results for NR UE HST DPS 1a scenario

Observation 1: MCS 17 is feasible for HST DPS 1a scenario.

Observation 2: There is negligible performance difference between configuration that PDSCH is scheduled in special slot or not.

Conclusion

In this paper we provide our simulation results on Rel-16 NR UE HST performance requirements for DPS 1a for alignment. Our observations are:

Observation 1: MCS 17 is feasible for HST DPS 1a scenario.

Observation 2: There is negligible performance difference between configuration that PDSCH is scheduled in special slot or not.

Reference

R4-2012668, WF on UE demodulation for NR HST, RAN4#96-e, CMCC