3GPP TSG-RAN WG4 Meeting #97-e	R4-2015619

# Electronic Meeting, 2nd – 13th Nov, 2020

Title: 	Simulation results on URLLC BS demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.2

Document for:	Discussion

Background

In this paper, URLLC BS PUSCH demodulation requirements for high reliability and low latency are provided based on the agreed parameters.

Low latency for PUSCH

FR2 PUSCH mapping Type B

Simulation assumption

Some of the test parameters were agreed during the last meeting, the remained parameters are proposed and shown in table below:

Table 2-1 Simulation assumption for FR2 PUSCH mapping Type B

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR2 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, low |
| SCS and BW |  | 60 kHz for 50 MHz/100 MHz 120 kHz for 50 MHz/100 MHz |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | 1 |
| Time domain resource assignment | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 7 |
|  | PUSCH aggregation factor | 1 |
| TDD pattern |  | 3D1S1U, S=10:2:2 |
| MCS Table |  | Table 3, MCS 5 |
| Propagation condition |  | TDLA30-300 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth |
| Test metric |  | 70% TP |

Simulation results

The performance for FR2 PUSCH mapping Type B for TDD with symbol length of 7 is presented in figure below:

- TDD

Based on the agreed and proposed parameters listed in Table 2-1, the simulated performances for TDD are shown in Figure 2-1 for symbol length of 7 for 60 kHz with bandwidth of 50 MHz and 100 MHz..

Figure 2-1: Throughput vs SNR for FR2 PUSCH mapping Type B with symbol length of 7

The SNR values of 70% throughput for TDD are list in table below:

| Bandwidth | 50MHz |  | 100MHz |  |
| --- | --- | --- | --- | --- |
| Duplex mode | Real SNR (dB) | Implemented SNR (dB) | Real SNR (dB) | Implemented SNR (dB) |
| TDD | -7.1 | -5.1 | -7.8 | -5.8 |

Reference

[1] R4-2005528, Way forward on NR URLLC BS performance requirements, #94bis, Huawei, Hisilicon.