3GPP TSG-RAN WG4 Meeting #97-e	R4-2015641

# Electronic Meeting, 2-13 Nov., 2020

Title: 	Simulation results for NR V2X single-link test

Source: 	Huawei, HiSilicon

Agenda item:	7.3.7.2

Document for:	Discussion

Background

At the last meeting, RAN 4 agreed to define the performance requirements for PSSCH, PSCCH, PSFCH and PSBCH [1] and simulation assumptions [2] was approved. In this contribution, we provide our simulation results.

Simulation results

PSSCH

The simulation assumptions are shown in appendix. We have some comments for the simulation assumptions for PSSCH, if we set allocated resource blocks of PSSCH to 15, allocated resource blocks of PSCCH to 10 and DMRS pattern is #1, #4, #7, the first DMRS symbol will be FDM with PSCCH symbol. But according to the lastest agreement from RAN 1 #102-e meeting, the TX UE is not expected to choose a PSSCH DMRS pattern to be transmitted in the same OFDM symbol with PSCCH and related clarification is shown as follows:

Agreements: - When a subchannel size is less than 20 PRBs and the size of PSCCH is less than the subchannel size, a TX UE is not expected to choose a PSSCH DMRS pattern to be transmitted in the same OFDM symbol with PSCCH.

Based on the above analysis, we propose to change 15 RBs for PSSCH resource allocation to 20RBs, so TBS and SCI stage2 configuration must be also changed, we give our updated simulation assumptions on revised version in appendix.

Then we give our simulation results based on assumptions of revised version in Figure 2.1.1 and Table 2.1.1. For channel estimation algorithm: we use MMSE for frequency-domain interpolation and linear for time-domain interpolation. For CFO estimation algorithm, we use cross-symbol algorithm.

Figure 2.1.1 Simulation results of PSSCH

Table 2.1.1 Summary of PSSCH simulation results

| Test number | Ideal SNR @ 10% of BLER | Impariment SNR @ 10% of BLER |
| --- | --- | --- |
| Option 1: SCH Test 1 | 1.24 | 2.74 |
| Option 1: SCH Test 2 | 11.72 | 13.22 |
| Option 2: SCH Test 1 | 6.46 | 7.96 |
| Option 2: SCH Test 2 | 0.82 | 2.32 |

PSCCH

The simulation assumptions derived from [1] are shown in Table 2.2.1:

Table 2.2.1: Simulation assumptions for PSCCH

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180, TDLA30-1350] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 2 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | [28] |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 360 |
| PSCCH performance metric (Note 1) |  | SNRPSCCH@1%BLER |
| Note 1: The performance metric is used for PSCCH simulation evaluation. Note 2: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 3: Two propagation conditions are listed as options for alignment purpose, one of the option will be chosen for defining requirement |  |  |

Based on the above simulation assumptions, the simulation results of PSCCH with different propagation condition are provided in Figure 2.2.1. Corresponding SNR target at 1% BLER are shown in Table 2.2.2.

Figure 2.2.1: Performance curve of PSCCH

Table 2.2.2: SNR @1 % of BLER

|  | TDLA-30-180 | TDLA-30-1350 |
| --- | --- | --- |
| Ideal SNR@ %1 of BLER | 3.44 | 3.38 |
| Impairment SNR @ %1 of BLER | 4.94 | 4.88 |

PSBCH

The simulation assumptions based on contribution [1] were summarized in Table 2.3-1:

Table 2.3-1: Simulation assumptions for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | S-SSB |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period(NperiodS-SSB) |  | Option 1: 1 Option 2: 2 S-SSB with different contents |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | 0 |
| Frequency offset | Hz | 0 |
| The number of symbols (Note 1) |  | 8 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| PSBCH performance metric (Note 2) |  | SNRPSBCH@1%BLER |
| Note 1: The first symbol is for AGC. Note 2: The performance metric is used for PSBCH simulation assumption. |  |  |

Based on Table 2.3-1, additional simulation assumptions are added in Table 2.3-2:

Table 2.3-2: Additional simulation assumptions for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| CFO estimation algorithm |  | Cross -symbol |
| sl-TimeOffsetSSB-r16 | slot | 0 |
| sl-TimeInterval-r16(Only for 2 S-SSB transmission) | slot | 300 |

The curve of simulation results is shown in Figure 2.3-1 and the SNR target at 1% BLER are shown in Table 2.3-3:

Figure 2.3.1: Performance of PSBCH

Table 2.3-3 Summary of PSBCH performance

| Tests | Ideal SNR @ 1% of BLER(dB) | Impairment SNR @ 1% of BLER (dB) |
| --- | --- | --- |
| 1 S-SSB transmission within one period | -2.55 | -1.05 |
| 2 S-SSB transmissions within one period | -2.51 | -1.01 |

PSFCH

Simulation assumptions derived from contribution [2] are summarized in Table 2.4.1:

Table 2.4.1: Simulation assumptions for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | [TDLA30-180] |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH period | Slot | [1] |
| Cyclic shift pair index |  | 1 or 2 |
| PSFCH performance metric (Note 1) |  | [Pr(ACK miss) < 1%], [Pr(NACK to ACK) < 0.1%], [Pr(DTX to ACK)<1%] |
| Note 1: The performance metric is used for PSFCH simulation assumption. [Note 2: The 1st PSFCH which is used for AGC doesn’t need to be considered in performance evaluation.] |  |  |

Based on the simulation assumptions, we provide our simulation results in Figure 2.4.1(Only 1 CS pair is allocated):

Figure 2.4.1 Simulation results of PSFCH

Table 2.4.2: Summary of PSFCH performance requirements

| Tests | Ideal SNR @ 1% of ACK miss | Impairment SNR @ 1% of ACK miss |
| --- | --- | --- |
| 1 CS pair | 7.82 | 9.32 |

Conclusion

In this paper, we give our simulation results for PSSCH, PSCCH, PSFCH and PSBCH.

Reference

- R4-2012758 WF on work scope and general assumptions for NR V2X demodulation performance, LG Electronics.

- R4-2012757 Simulation assumptions for NR V2X test cases, Mediatek.

Appendix

Option 1:

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 |
| Synchronization source |  | GNSS | GNSS |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| Propagation condition |  | [TDLA30-2700] | [TDLA30-180] |
| Channel bandwidth | MHz | 20 | 20 |
| Allocated resource blocks |  | [20] | [20] |
| Subcarrier spacing | kHz | 30 | 30 |
| CP-OFDM symbols per slot (Note 1) |  | [9] | [9] |
| DMRS position in time domain (symbol index) |  | [#1,#4,#7] | [#1,#4,#7] |
| Modulation order |  | QPSK | 64QAM |
| MCS index |  | [4] | [17] |
| The number of REs for 2nd stage SCI |  | [240] | [174] |
| Transport Block Size | Bits | [808] | [3624] |
| Transport block CRC | Bits | 24 | 24 |
| Maximum number of HARQ retransmissions |  | 0 | 0 |
| Binary Channel Bits (Note 2) | Bits | [2760] | [8640] |
| Antenna configuration |  | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@[10]%BLER |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on [every slot]. Note 5: 15RB allocation is listed here, but 10RB is another option for simulation results alignment. |  |  |  |

- Option 2

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test2 |
| Synchronization source |  | GNSS | GNSS |
| Propagation condition |  | TDLA-30-1400 | TDLA-30-2700 |
| Channel bandwidth | MHz | 20MHz | 20MHz |
| Allocated resource blocks | RB | [20] | [20] |
| Subcarrier spacing | kHz | 30 | 30 |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| CP-OFDM symbols per slot (Note 1) |  | [12] for slots without PSFCH 9 for slots with PSFCH | [12] for slots without PSFCH 9 for slots with PSFCH |
| The number of DMRS symbols |  | [3] for slot without PSFCH;[2] for slot with PSFCH | [4] for slot without PSFCH;[3] for slot with PSFCH |
| Modulation order |  | [4] | [2] |
| MCS index |  | [11](4,378/1024) | [4](2,308/1024) |
| The number of REs for 2nd stage SCI (SCI format 2-A) |  | [288](βoffset=7,α=1) | [300] (βoffset=3,α=1) |
| Transport Block Size | Bits | [3104] for slots without PSFCH [2024] for slots with PSFCH | [1192] for slots without PSFCH [736] for slots with PSFCH |
| Transport block CRC | bits | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 |
| Binary Channel Bits (Note 2) | Bits | 8256 for slots without PSFCH 5616 for slots with PSFCH | 3840 for slots without PSFCH 2640 for slots with PSFCH |
| periodPSFCHresource |  | 4 | 4 |
| Antenna configuration |  | 1x2 Low | 1x2 Low |
| Performance metric(Note 3) |  | SNRPSSCH@10%BLER |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: 10 RBs and 2 symbols are allocated for PSCCH, and 2nd stage SCI is also allocated. Note 3: The performance metric is used for PSSCH simulation evaluation. Note 4: PSFCH is transmitted on every [4] slot. |  |  |  |