3GPP TSG-RAN WG4 Meeting #97-e	R4-2015654

# Electronic Meeting, 2 - 13 Nov, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | <CR#> | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for 38.101-4: Introduction of PDSCH requirement with Multi-DCI based transmission scheme |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 agree to introduce PDSCH requirements of Multi-DCI based transmission scheme and the aligned requirements need to be added into the specfication |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PDSCH requirements of Multi-DCI based transmission scheme |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH requirements of Multi-DCI based transmission scheme are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.6, 5.2.2.2.6, 5.2.3.1.6, 5.2.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of First change>

5.2.2.1.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 5.2.2.1.6-3, with the addition of test parameters in Table 5.2.2.1.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.6-1.

Table 5.2.2.1.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Multi-DCI based transmission scheme of Multi-TRP | 1-1 |

Table 5.2.2.1.6-2: Test parameters

| Parameters | Simulation cases 1-1 |
| --- | --- |
| Duplex mode | FDD |
| TDD UL/DL configuration | N/A |
| Timing offset of the second TRP from the first TRP (Note 2) | Positive offset: 2 μs Negative offset: -0.5 μs |
| Frequency offset between two TRPs (Note 2) | 200 Hz |
|  | Common serving cell parameters |
| Physical Cell ID | 0 |
| SSB position in burst | First SSB in Slot #0 |
| SSB periodicity | 20ms |
| Transmit TRP of SSB | Only from TRP1 |
|  | TRS#1 (configuration for TCI state #1) |
| Transmit TRP | TRP#1 |
| CSI-RS periodicity | [20] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 |
| QCL Info | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |
| Transmit TRP | TRP#2 |
| CSI-RS periodicity | [20] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 |
|  | TCI State configuration #0 |
| Type 1 QCL information | SSB index #0, QCL type C |
| Type 2 QCL information | N/A |
|  | TCI State configuration #1 |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |
| Type 2 QCL information | N/A |
|  | TCI State configuration #2 |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |
| Type 2 QCL information | N/A |
|  | PDCCH configuration |
| CORESETPoolIndex | 0, 1 |
| Symbols for PDCCH | 0, 1 |
| Number of PRB | Half of the channel bandwidth with contiguous RB allocation and non-interleaved CCE-to-REG mapping |
| K0 | 0 |
| AL | 8 (576REs) |
|  | PDSCH configuration |
| PDSCH resource mapping type | Type A |
| Resource allocation type | Type 1 |
| DMRS configuration | Type 1, 1+1 |
| Antenna ports index ​ | {1000,1001} for TCI#1 and {1002,1003} for TCI#2 |
| Start symbol (S) | 2 |
| Time duration (L) | 12 |
| PRB allocation | Non-overlapping case PDSCH#1: PRB#0 to RPB#25 PDSCH#2: PRB#26 to RPB#51 |
| Note 1 Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: FFS whether PDSCH is scheduled in slots where TRSs are transmitted. |  |

Table 5.2.2.1.6-3: Minimum performance

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | TBD |

<end of First change>

<Start of Second change>

5.2.2.2.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 5.2.2.2.6-3, with the addition of test parameters in Table 5.2.2.2.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.6-1.

Table 5.2.2.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Multi-DCI based transmission scheme of Multi-TRP | 2-1 |

Table 5.2.2.2.6-2: Test parameters

| Parameters | Simulation cases 2-1 |  |
| --- | --- | --- |
| Duplex mode | TDD |  |
| TDD UL/DL configuration | [FR1.30-1 (7DS2U, 6D+4G+4U)] FFS whether PDSCH is scheduled in the special slots |  |
| Timing offset of the second TRP from the first TRP (Note 2) | Positive offset: 1μs Negative offset: -0.25μs |  |
| Frequency offset between two TRPs (Note 2) | 300 Hz |  |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | [40] slots |  |
| CSI-RS offset | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |  |
| Density | 3 |  |
| First subcarrier index in the PRB used for CSI-RS | k0=0 |  |
| QCL Info | TCI state #0 |  |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | [40] slots |  |
| CSI-RS offset | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |  |
| Density | 3 |  |
| First subcarrier index in the PRB used for CSI-RS | k0=1 |  |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  |
| QCL Info | TCI state #0 |  |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0, 1 |  |
| Symbols for PDCCH | 0, 1 |  |
| Number of PRB | Half of the channel bandwidth with contiguous RB allocation and non-interleaved CCE-to-REG mapping |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| DMRS configuration | Type 1, 1+1 |  |
| Antenna ports index ​ | {1000,1001} for TCI#1 and {1002,1003} for TCI#2 |  |
| Start symbol (S) | 2 |  |
| Time duration (L) | 12 |  |
| PRB allocation | Non-overlapping case PDSCH#1: PRB#0 to RPB#52 PDSCH#2: PRB#53 to RPB#105 |  |
| Note 1 Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: FFS whether PDSCH is scheduled in slots where TRSs are transmitted. |  |

Table 5.2.2.2.6-3: Minimum performance

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | TBD |

<end of Second change>

<Start of Third change>

5.2.3.1.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 5.2.3.1.6-3, with the addition of test parameters in Table 5.2.3.1.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.6-1.

Table 5.2.3.1.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Multi-DCI based transmission scheme of Multi-TRP | 1-2 |

Table 5.2.3.1.6-2: Test parameters

| Parameters | Simulation cases 1-2 |
| --- | --- |
| Duplex mode | FDD |
| TDD UL/DL configuration | N/A |
| Timing offset of the second TRP from the first TRP (Note 2) | Positive offset: 2 μs Negative offset: -0.5 μs |
| Frequency offset between two TRPs (Note 2) | 200 Hz |
|  | Common serving cell parameters |
| Physical Cell ID | 0 |
| SSB position in burst | First SSB in Slot #0 |
| SSB periodicity | 20ms |
| Transmit TRP of SSB | Only from TRP1 |
|  | TRS#1 (configuration for TCI state #1) |
| Transmit TRP | TRP#1 |
| CSI-RS periodicity | [20] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 |
| QCL Info | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |
| Transmit TRP | TRP#2 |
| CSI-RS periodicity | [20] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 |
|  | TCI State configuration #0 |
| Type 1 QCL information | SSB index #0, QCL type C |
| Type 2 QCL information | N/A |
|  | TCI State configuration #1 |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |
| Type 2 QCL information | N/A |
|  | TCI State configuration #2 |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |
| Type 2 QCL information | N/A |
|  | PDCCH configuration |
| CORESETPoolIndex | 0, 1 |
| Symbols for PDCCH | 0, 1 |
| Number of PRB | Half of the channel bandwidth with contiguous RB allocation and non-interleaved CCE-to-REG mapping |
| K0 | 0 |
| AL | 8 (576REs) |
|  | PDSCH configuration |
| PDSCH resource mapping type | Type A |
| Resource allocation type | Type 1 |
| DMRS configuration | Type 1, 1+1 |
| Antenna ports index ​ | {1000,1001} for TCI#1 and {1002,1003} for TCI#2 |
| Start symbol (S) | 2 |
| Time duration (L) | 12 |
| PRB allocation | Non-overlapping case PDSCH#1: PRB#0 to RPB#25 PDSCH#2: PRB#26 to RPB#51 |
| Note 1 Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: FFS whether PDSCH is scheduled in slots where TRSs are transmitted. |  |

Table 5.2.3.1.6-3: Minimum performance

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | TBD |

<end of third change>

<Start of Forth change>

5.2.3.2.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 5.2.3.2.6-3, with the addition of test parameters in Table 5.2.3.2.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.6-1.

Table 5.2.3.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme of Multi-TRP | 2-2 |

Table 5.2.3.2.6-2: Test parameters

| Parameters | Simulation cases 2-2 |
| --- | --- |
| Duplex mode | TDD |
| TDD UL/DL configuration | [FR1.30-1 (7DS2U, 6D+4G+4U)] FFS whether PDSCH is scheduled in the special slots |
| Timing offset of the second TRP from the first TRP (Note 2) | Positive offset: 1μs Negative offset: -0.25μs |
| Frequency offset between two TRPs (Note 2) | 300 Hz |
|  | Common serving cell parameters |
| Physical Cell ID | 0 |
| SSB position in burst | First SSB in Slot #0 |
| SSB periodicity | 20ms |
| Transmit TRP of SSB | Only from TRP1 |
|  | TRS#1 (configuration for TCI state #1) |
| Transmit TRP | TRP#1 |
| CSI-RS periodicity | [40] slots |
| CSI-RS offset | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 |
| QCL Info | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |
| Transmit TRP | TRP#2 |
| CSI-RS periodicity | [40] slots |
| CSI-RS offset | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 |
|  | TCI State configuration #0 |
| Type 1 QCL information | SSB index #0, QCL type C |
| Type 2 QCL information | N/A |
|  | TCI State configuration #1 |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |
| Type 2 QCL information | N/A |
|  | TCI State configuration #2 |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |
| Type 2 QCL information | N/A |
|  | PDCCH configuration |
| CORESETPoolIndex | 0, 1 |
| Symbols for PDCCH | 0, 1 |
| Number of PRB | Half of the channel bandwidth with contiguous RB allocation and non-interleaved CCE-to-REG mapping |
| K0 | 0 |
| AL | 8 (576REs) |
|  | PDSCH configuration |
| PDSCH resource mapping type | Type A |
| Resource allocation type | Type 1 |
| DMRS configuration | Type 1, 1+1 |
| Antenna ports index ​ | {1000,1001} for TCI#1 and {1002,1003} for TCI#2 |
| Start symbol (S) | 2 |
| Time duration (L) | 12 |
| PRB allocation | Non-overlapping case PDSCH#1: PRB#0 to RPB#52 PDSCH#2: PRB#53 to RPB#105 |
| Note 1 Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: FFS whether PDSCH is scheduled in slots where TRSs are transmitted. |  |

Table 5.2.3.2.6-3: Minimum performance

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | TBD |

<end of forth change>