3GPP TSG-RAN WG4 Meeting #97-e								          R4-2015695

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Huawei, HiSilicon

Title: 	On remaining issues for BS TX

Agenda Item:	7.1.4.2

Document for:	Approval

Introduction

In RAN#89 meeting, the CR for NR-U TS 38.104 was approved. There are still some remaining opens issues for BS RF requirements. In this contribution, we discuss these issues and provide proposals for the corresponding issues.

Discussion

- ΔfOBUE

The maximum offset of OBUE outside the downlink operating band defines the boundary of OBUE and spurious emissions. 40 MHz ΔfOBUE was approved for n46. For band n96 it still TBD.

Table 2.1-1: Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-H | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz FDL,high – FDL,low 900 MHz | 40 |
|  | 900 MHz < FDL,high – FDL,low 1200 MHz | [TBD] |
| BS type 1-C | FDL,high – FDL,low 200 MHz | 10 |
|  | 200 MHz < FDL,high – FDL,low 900 MHz | 40 |
|  | 900 MHz < FDL,high – FDL,low 1200 MHz | [TBD] |

It should be noted that when we discuss the boundary between in-band and out of band for FR1bands, band 46 is not in the scope [1].  And it is clearly stated that if wider band(s) than current maximum bandwidth of Band is specified, the values of the boundary should be discussed. Hence we think the conclusion for NR-U should not be extended to other bands without further discussion.

Proposal 1: It is proposed to define the boundary between OBUE and spurious emission in a separate Table for NR-U n46 and n96.

As evaluated in [2], it is challenge to provide the needed attenuation for a BS covering the entire band. The emission limits outside the U-NII-5 and U-NII-8 bands is -27dBm/MHz EIRP. The first 20 MHz at band edges is used as guard band. Hence it is worth discussing whether to remove the 20 MHz channels from band n96.

Table 2.1-2: Maximum offset of OBUE outside the downlink operating band for band n46 and n96

| Operating band | ΔfOBUE (MHz) |
| --- | --- |
| n46 | 40 |
| n96 | TBD |

Conclusions

In this contribution, we provide the discussion on ΔfOBUE,

Proposal 1: It is proposed to define the boundary between OBUE and spurious emission in a separate Table for NR-U n46 and n96.

References

R4-1711779, WF on in-band and out-of-band boundary, NTT DOCOMO, INC., Nokia, Ericsson, Huawei, ZTE

R4-2010495, Consideration on 6GHz band definition, Huawei