3GPP TSG-RAN WG4 Meeting #97-e								          R4-2015697

Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, HiSilicon

Title: 	A-MPR evaluation for NS_54

Agenda Item:	7.1.2.1

Document for:	Approval

Introduction

In RAN#89 meeting, the CR for NR-U 38.101-1 was approved. The values on A-MPR specified in Table 6.2F.3.7-1for NS_54 are put in square bracket for further study. In this contribution, we provide the A-MPR simulation results and corresponding proposal for NS_54.

Discussion

Additional requirements for NS_54

The in-band PSD limits of 17 dBm/MHz have no impact to the A-MPR. For out of band emission, when NS_54 is indicated in the cell, the additional requirements in Table 2-1 shall apply.

Table 2-1: Additional requirements (NS_54)

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

A-MPR evaluation

The deviation between guard =10 MHz and guard =20 MHz is shown as below, using the assumptions agreed in [1].

| Full allocation (15kHz) |  |  | 40M | 60M | 80M |  |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | CP-OFDM |  | -0.7 |  |  |  |
|  | DFT-S |  | -0.5 |  |  |  |
| Full allocation (30kHz) |  |  | 40M | 60M | 80M |  |
| QPSK | CP-OFDM |  | -0.7 | -0.5 | -0.3 |  |
|  | DFT-S |  | -0.5 | -0.2 | -0.3 |  |
| Full allocation (15kHz) |  |  | 40M | 60M | 80M |  |
| 16QAM | CP-OFDM |  | -0.7 |  |  |  |
|  | DFT-S |  | -0.6 |  |  |  |
| Full allocation (30kHz) |  |  | 40M | 60M | 80M |  |
| 16QAM | CP-OFDM |  | -0.7 | -0.4 | -0.3 |  |
|  | DFT-S |  | -0.6 | -0.3 | -0.2 |  |
| Full allocation (15kHz) |  |  | 40M | 60M | 80M |  |
| 64QAM | CP-OFDM |  | 0.1 |  |  |  |
|  | DFT-S |  | 0 |  |  |  |
| Full allocation (30kHz) |  |  | 40M | 60M | 80M |  |
| 64QAM | CP-OFDM |  | 0.1 | 0.1 | 0.1 |  |
|  | DFT-S |  | 0 | 0 | 0 |  |
| Full allocation (15kHz) |  |  | 40M | 60M | 80M |  |
| 256QAM | CP-OFDM |  | -0.1 |  |  |  |
|  | DFT-S |  | 0.1 |  |  |  |
| Full allocation (30kHz) |  |  | 40M | 60M | 80M |  |
| 256QAM | CP-OFDM |  | 0 | 0 | -0.1 |  |
|  | DFT-S |  | 0.1 | 0 | 0 |  |
| Interlace 0 (15kHz) |  | 40M |  | 60M | 80M |  |
| QPSK | CP-OFDM | -0.8 |  |  |  |  |
|  | DFT-S | -0.5 |  |  |  |  |
| Interlace 0 (30kHz) |  | 40M |  | 60M | 80M |  |
| QPSK | CP-OFDM | -0.9 |  | -0.4 | -0.3 |  |
|  | DFT-S | -0.7 |  | -0.3 | -0.3 |  |
| Interlace 0 (15kHz) |  | 40M |  | 60M | 80M |  |
| 16QAM | CP-OFDM | -0.8 |  |  |  |  |
|  | DFT-S | -0.7 |  |  |  |  |
| Interlace 0 (30kHz) |  | 40M |  | 60M | 80M |  |
| 16QAM | CP-OFDM | -0.9 |  | -0.4 | -0.2 |  |
|  | DFT-S | -0.8 |  | -0.4 | -0.2 |  |
| Interlace 0 (15kHz) |  | 40M |  | 60M | 80M |  |
| 64QAM | CP-OFDM | -0.3 |  |  |  |  |
|  | DFT-S | -0.7 |  |  |  |  |
| Interlace 0 (30kHz) |  | 40M |  | 60M | 80M |  |
| 64QAM | CP-OFDM | 0 |  | -0.4 | -0.2 |  |
|  | DFT-S | -0.5 |  | -0.4 | -0.3 |  |
| Interlace 0 (15kHz) |  | 40M |  | 60M | 80M |  |
| 256QAM | CP-OFDM | 0 |  |  |  |  |
|  | DFT-S | 0.1 |  |  |  |  |
| Interlace 0 (30kHz) |  | 40M |  | 60M | 80M |  |
| 256QAM | CP-OFDM | 0.1 |  | 0 | -0.1 |  |
|  | DFT-S | 0 |  | 0 | 0 |  |

Based on the evaluation above, it is found that A-MPR could be reduced by about 0.5 dB for QPSK and 16QAM due to the increase of the guardband size. We propose to update the A-MPR table as below.

Table 2-2: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ [2] | ≤ [4.5] |
|  | 16 QAM |  | ≤ [2.5] | ≤ [4.5] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.0] | ≤ [5.5] |
|  | 16 QAM |  | ≤ [4.0] | ≤ [5.5] |
|  | 64 QAM |  | ≤ [5.5] | ≤ [6.0] |
|  | 256 QAM |  | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. |  |  |  |  |

Conclusions

In this contribution, we provide the A-MPR simulation results and corresponding proposal for NS_54:

Proposal 1: A-MPR for NS_54 is defined in Table 2-2.

References

R4-2005221 WF on NR-U UE Tx requirements

R4-2008436, WF on NR-U MPR, MediaTek Inc., Skyworks Solution Inc.

RP-201890, Introduction of NR-based access to unlicensed spectrum, Qualcomm Incorporated