3GPP TSG-RAN WG4 Meeting #97-e	R4-2015767

Electronic Meeting, 2 – 13 November, 2020

Title: 	Discussion on the scope gNB requirements for NR positioning

Source: 	Huawei, HiSilicon, CMCC

Agenda item:	7.7.3.3

Document for:	Discussion

Introduction

In RAN4#96-e, gNB positioning measurement requirements were discussed and the outcomes are captured in the agreed WF [1].

In particular, regarding the scope of the requirements, following agreements are reached.

- FFS: gNB measurements for which accuracy shall be defined. - Candidate options: - Option 1: - Define accuracy for SRS-RSRP and gNB Rx-Tx time difference - Option 2: - Define accuracy for SRS-RSRP, gNB Rx-Tx time difference and UL RTOA

In this paper, we will provide our views on whether to define accuracy requirements for UL-RTOA.

Discussion

  - Generic principle

gNB measurements for NR positioning are listed in [2].

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

The gNB measurements are used to facilitate

- UL-based positioning techniques: UL-TDOA and UL-AoA

- UL-and-DL-based positioning techniques: multi-RTT

The positioning performance of these techniques depends on the measurement performance of gNB, so in this sense the gNB requirements are as necessary as UE requirements, in order to guarantee the minimum performance when these positioning techniques are deployed.

Observation 1: gNB measurement requirements are necessary to guarantee the minimum performance of UL-based positioning techniques and UL-and-DL-based positioning techniques.

As different gNBs may implement different positioning techniques, and some gNB may not support positioning, the gNB measurement requirements are optional and the applicability depends on gNB declaration. In this sense, defining requirements for a particular measurement type does not mean every gNB needs to implement the measurement. However, on the contrary, not defining requirements for a measurement type means the performance cannot be tested even it is implemented. For same reason, in gNB demodulation discussions, requirements for a feature are defined if it is requested by some network vendor or some operator. Similar principle should be followed.

Observation 2: Defining requirements for a measurement type does not mandate every gNB to implement the measurement; whereas not defining requirements for a measurement type means the measurement performance cannot be tested even the measurement is implemented.

UL-RTOA measurement is used in UL-TDOA, which is an UL-based positioning technique, meaning network can locate the UE without UE positioning measurement. This is useful and beneficial for both network and UE since some UEs in the network may not support positioning measurement, and this UE can still get the location based services, both emergency and commercial.

Therefore, we see a clear motivation to define performance requirements for UL-RTOA measurement. This is particularly important, as requirements for AoA/ZoA measurement, which is for another UL-based positioning method (UL-AoA), is already excluded as a compromise considering the WI timeline.

It should be noted that neither Rx-Rx time difference or SRS-RSRP can enable UL-based positioning, so only defining performance requirements for these two measurement types means the whole use case of UL-based positioning is not supported by RAN4 requirements.

Observation 3: UL-based positioning is an important use case, and it will not be supported by RAN4 requirements if RAN4 only defines gNB requirements for Rx-Tx time difference and SRS-RSRP.

  - UL-RTOA

UL-RTOA measurement is defined in 38.215 as below.

[The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in positioning node j, relative to the configurable reference time.] Multiple SRS resources for positioning can be used to determine the beginning of one subframe containing SRS received at a positioning node.

The definition is same as in LTE, and in LTE the measurement requirements for UL-RTOA measurement are specified, in 36.111. The reason why the requirements are captured in a separate specification than 36.133 was because in LTE the UL-RTOA measurements are assumed to be taken by LMU, which may or may not be a separate node than eNB. In NR there is no such differentiation, and according to 38.215 UL-RTOA is taken by “positioning node” or gNB, which is same as Rx-Tx time difference measurement.

Observation 4: The measurement requirements for UL-RTOA are already defined in LTE.

Observation 5: In NR, UL-ROTA measurement is taken by gNB, same as Rx-Tx time difference and SRS-RSRP measurement.

In RAN4#93 and RAN4#94-e discussion, the main concern to introduce gNB requirements for UL-RTOA was the work load and timeline for completing the WI. However, we do not see any issue here because from gNB measurement performance point of view, UL-RTOA is quite similar as Rx-Tx time difference.

The definition of Rx-Tx time difference measurement is copied below from 38.215.

The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the positioning node received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the positioning node transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS.

- UL-RTOA is the difference between the measured receive timing and a configured reference time. The reference time is also determined based on gNB’s own timing which may e.g. be from GPS.

- Rx-Tx time difference is the difference between the measured receive timing and the transmit timing. The transmit timing is also based on gNB’s own timing which may e.g. be from GPS.

Therefore, the fundamental measurement in both UL-RTOA and Rx-Tx time difference is the receive timing measurement based on UE SRS transmission. The RAN4 work for this part, e.g. determining the SRS configurations and the SINR side conditions, running the simulations, can be used for both requirements. We do not see any major extra work for RAN4 to introduce requirements for UL-RTOA if requirements are anyway introduced for Rx-Tx time difference.

Observation 6: From gNB measurement performance point of view, UL-RTOA is quite similar as Rx-Tx time difference, so RAN4 effort to introduce requirements for UL-RTOA is not an issue.

Based on above observations, we propose

Proposal 1: RAN4 to define gNB measurement accuracy requirements for UL-RTOA in the Perf part of the WI. The requirements and side conditions for gNB Rx-Tx time difference are re-used.

Conclusions

In this paper we provided our views on whether to define accuracy requirements for UL-RTOA.

Generic principles

Observation 1: gNB measurement requirements are necessary to guarantee the minimum performance of UL-based positioning techniques and UL-and-DL-based positioning techniques.

Observation 2: Defining requirements for a measurement type does not mandate every gNB to implement the measurement; whereas not defining requirements for a measurement type means the measurement performance cannot be tested even the measurement is implemented.

Observation 3: UL-based positioning is an important use case, and it will not be supported by RAN4 requirements if RAN4 only defines gNB requirements for Rx-Tx time difference and SRS-RSRP.

UL-RTOA related

Observation 4: The measurement requirements for UL-RTOA are already defined in LTE.

Observation 5: In NR, UL-ROTA measurement is taken by gNB, same as Rx-Tx time difference and SRS-RSRP measurement.

Observation 6: From gNB measurement performance point of view, UL-RTOA is quite similar as Rx-Tx time difference, so RAN4 effort to introduce requirements for UL-RTOA is not an issue.

Proposal 1: RAN4 to define gNB measurement accuracy requirements for UL-RTOA in the Perf part of the WI. The requirements and side conditions for gNB Rx-Tx time difference are re-used.

Reference

- R4-2012302, WF on gNB requirements for NR positioning, Ericsson