3GPP TSG-RAN Working Group 4 (Radio) meeting #97-e	R4-2015798

# Online, 2nd Nov – 13th Nov, 2020

Source:	LG Electronics

Title:	On NRU operation modes and capabilities

Agenda item:	8	Rel-16 UE feature list

Document for:	Approval

# Introduction

During the RAN1#101 meeting and RAN4#96 meeting NRU UE capabilities have been discussed in [1], [2], [3] and [4].

This contribution further discusses this topic and proposes a way forward.

# Discussion

Based on the prior discussions, operational modes of wideband operation can be identified for both DL and UL as summarized below.

DL:

- DL-1:		Mode 1 (DL case 4) - Single wideband carrier operation with no adaptation to LBT due to ‘all or nothing’

- DL-2:		Mode 2/3 (DL case 2a/b) – Utilize multiple LBT sub-bands without scheduling of the intra-cell guard band(s).

- DL-3:		Mode 2/3 (DL case 3) – Utilize multiple LBT sub-bands with scheduling of the intra-cell guard band(s).

UL:

- UL-1:		Mode 2 (UL case 1) - Scheduling one of the multiple LBT sub-bands

- UL-2:		Mode 1 (UL case 3) - Single wideband carrier operation with no adaptation to LBT due to ‘all or nothing’

- UL-3:		Mode 2 (UL case 2) - Scheduling multiple LBT sub-bands without scheduling of the intra-cell guard band(s).

- UL-4:		Mode 2 (UL case 2) - Scheduling multiple LBT sub-bands with scheduling of the intra-cell guard band(s).

Based on the earlier discussions following proposal is made:

DL:

Baseline/default operation mode:

- DL-1: Single wideband carrier operation

  - This is no different than normal NR and there is no adaptation to transmission/reception constrains due to LBT.

Additional capability for DL:

- FG 4-1

  - DL-2 (DL case 2a/2b): Supported if FG 4-1 is not indicated

    - Utilize multiple LBT sub-bands without scheduling of the intra-cell guard band(s).

  - DL-3 (DL case 3): Supported if FG 4-1 is indicated.

    - Utilize multiple LBT sub-bands with scheduling of the intra-cell guard band(s)

UL:

Baseline/default operation mode:

- UL-1: Single LBT sub-band operation

  - This is no different than what has been defined for eLAA and it only requires single CCA to be performed by the UE.

- UL-2: Even if it requires multiple CCAs to be supported, should be considered as part of baseline.

- UL-3 and UL-4: As clarifier in R4-2011931, there is no restriction in scheduling within the intra-cell GB between two scheduled adjacent RB-sets.

Additional capability for UL:

- Not needed

Following update on FG 4-1 (as in R4-2011930) is proposed:

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
|  | 4-1 | Reception in intra-carrier guardband | Capability of reception in the intra-cell guardband between contiguous subbands in DL wideband carrier operation mode 2 or mode 3 wider than 20MHz | [10-19b] or [10-19c] |

# Conclusion

A way forward, as described in chapter 2, to finalize the NR-U capabilities is proposed.

# References

[1]		R4-2011931, LS reply to RAN1on UE capability on wideband carrier operation for NR-U, Source: RAN WG4, To: RAN WG1, RAN WG2, RAN4 #92bis, RAN4 #96e, August 2020

[2]		R4-2011930	RAN4 UE features list for Rel-16, CMCC, RAN4 #96e, August 2020

[3]		R4-2011447, NR-U – On intra-band CA and wideband operation modes, Nokia, RAN4 #96e, August 2020

[4]		R1-2004965, LS on UE capability on wideband carrier operation for NR-U, Source: RAN WG 1, To: RAN WG 4, RAN WG2, RAN1 #101e, June 2020