3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2015807

# Electronic Meeting, 2-13 Nov., 2020

Source:	Sony

Title:	Test frequencies for NB-IOT UE in standalone operation

Agenda item:	5.2

Document for:	Discussion

- Background

Band-edge emission at stand-alone and guard-band NB-IoT conditions is regulated by FCC OOB emission requirements. How to test the emission according to the FCC requirement was debated in RAN4 #96-e [1]. It was decided to seek guidance from the FCC on some specific questions which resulted in an LS sent to FCC [2].  No response has been received so far.

Below we further discuss the test requirement.

- Discussion

The issue is related to band-edge emission at stand-alone and guard-band NB-IoT conditions that is regulated by FCC OOB emission requirements.

According to 3GPP TS 36.508 [1] the NB-IoT channel in stand-alone and guard-band operating modes may be allocated just at 100kHz offset from the band edges; see for example band 12.

Table 8.1.3.1.1.12-1: NB-IoT standalone Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23179 | 0 | 715.9 | 5179 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3: NB-IoT guard-band Test frequencies for operating band 12

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23179 | 0 | 715.9000 | 5179 | -2 | 745.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

The tables refer to a NB-IoT channel of 180kHz, so the first NB-IoT tone (15kHz or 3.75kHz) will be only about 10kHz from the band edges where the FCC regulation applies.

There was a question in last meeting whether the rules apply to the 3GPP band 12, the FFC band 12 or only A block. That is, does the “100 kHz” refer to 698.9-699MHz and 716-716.1MHz or 697.9-698 MHz and 716-716.1MHz or 697.9-698 MHz and 704.704.1MHz? If the answer is that it refers to the 3GPP band 12 there is a problem. This is the way it is tested today.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation may conflict with FCC band-edge spectrum emission requirements.

A proposed way to avoid the violation of the FCC regulation would be to omit operation at the first and last NB-IoT EARFCNs in stand-alone and guard-band modes (in the above example NUL=23011 and NUL=23179).

Observation 2:		100 kHz offset for NB-IoT network deployments may solve the violation of the FCC regulation.

We, therefore, propose an LS should be sent to RAN5 to exclude first and last EARFCNs

Proposal 1:	Send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

The change of test frequencies, of course, must be accompanied with a deployment regime which require 100 kHz offset for NB-IoT network deployments as discussed above.

- Conclusion

In this contribution, we have discussed testing of NB-IOT. The following observations and proposals are made.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation may conflict with FCC band-edge spectrum emission requirements.

Observation 2:		100 kHz offset for NB-IoT network deployments may solve the violation of the FCC regulation.

Proposal 1:	Send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

- References

R4-2011845	“Email discussion summary for [96e][105] LTE UE RF maintenance”, Skyworks Solution Inc.

R4-2011913	“LS on NB-IoT testing issues”, RAN WG4

TS 36.508 	“Common test environments for User Equipment (UE) conformance testing”, Section 8.1.3

TS 36.104	 “Base Station (BS) radio transmission and reception”, Section 5.6