3GPP TSG-RAN WG4 Meeting #97-e	R4-2015813

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.15.3.1.2

Source:	Ericsson

Title:	Simulation results of PDSCH with HST-SFN

Document for:	Information

# 1	Introduction

RAN4#96-e agreed with the way forward on NR UE demodulation requirements for HST scenario [1]. This contribution provides our simulation results for HST-SFN according to the simulation assumption in [1].

# 2	HST-SFN

## 2.1	Simulation parameters

Table 1	Simulation parameters for MPDCCH performance improvements.

Table 1 is the simulation parameters for HST-SFN according to [1].

| Parameter​ | Value​ |  |
| --- | --- | --- |
|  | FDD 15KHz SCS​ | TDD 30KHz SCS​ |
| Antenna configuration​ | 2x2; 2x4​ |  |
| DMRS type​ | type 1​ |  |
| Number of DMRS symbols​ | DMRS 1+1+1​ |  |
| TDD pattern​ | 7D1S2U, S: 6D 4G 4U​ |  |
| MCS​ | MCS 13 based on 64QAM table​ |  |
| Propagation condition​ | HST-SFN​ |  |
| TRS periodicity​ | 10ms, 2slot pattern​ |  |
| PDSCH mapping​ | Type A, Start symbol 2, Duration 12​ |  |
| Ds and Dmin​ | Ds=700m, Dmin=150m​ |  |
| Rank​ | Rank = 2​ |  |
| BW​ | 10MHz​ | 40MHz​ |
| Maximum Doppler shift​ | 870Hz​ | 1667Hz​ |
| Testing metric​ | SNR @70% of maximum throughput​ |  |

## 2.2	Simulation results

Table 2 shows our simulation results for HST-SFN. We propose to consider the simulation results to define the NR PDSCH demodulation requirements for HST-SFN scenario.

Table 2	Simulation results of NR PDSCH demodulation for HST-SFN (SNR @ 70% of maximum throughput).

|  | FDD SCS=15kHz |  | TDD SCS=30kHz |  |
| --- | --- | --- | --- | --- |
|  | 2Rx | 4Rx | 2Rx | 4Rx |
| Alignment results | 10.4 | 7.8 | 12.0 | 10.1 |
| Impairment results | 12.4 | 9.8 | 14.0 | 12.1 |

# 3	References

- R4-2012668, “WF on UE demodulation for NR HST”, CMCC.