3GPP TSG-RAN WG4 Meeting #97-e	R4-2015841

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	6.1.3.2

Source:	Ericsson

Title:	Test cases of RLM for MPDCCH performance improvement

Document for:	Discussion

# 1	Introduction

RAN4#96-e agreed with the way forward on test cases for RLM with enhanced MPDCCH in Rel-16 eMTC [1]

| Type of test | RRC state | Coverage level | Test conf. | Duplex mode | Assigned company |
| --- | --- | --- | --- | --- | --- |
| MPDCCH improvement | CONNECTED | CEModeA and CEModeB | Verify test improved MPDCCH when out-of-sync is triggered, In non-DRX scenario | FDD, HD-FDD, TDD | Ericsson |

This contribution discusses the test setup of RLM with enhanced MPDDCH according to the way forward.

# 2	RLM test cases for MPDCCH performance improvement

## 2.1	Test cases

RAN4 has specified RLM test cases for eMTC from Rel-13, and it covers many scenarios such as FDD/TDD, CEModeA/CEModeB. We think it is straightforward to reuse the test setup for Rel-16 out-of-synch test with MPDCCH performance improvement to reduce the workload in RAN4. Table 1 summarizes the out-of-sync/early-out-of-synch test cases with no DRX condition specified in the TS36.133 V16.7.0. As it is observed from the summary, we can introduce new RLM out-of-synch test cases for CE Mode A based on A.7.3.48, A.7.3.52, and A.7.3.56 for full-duplex FDD (FD-FDD), half-duplex FDD (HD-FDD) and TDD, respectively.

On the other hand, it is observed RAN4 has not defined RLM out-of-synch test cases for CE Mode B. In order to define out-of-synch requirements for CE Mode B according to the WF, one possibility is to define the early-out-of-synch test cases by reusing A.7.3.82, A.7.3.84, and A.7.3.86 for full-duplex FDD (FD-FDD), half-duplex FDD (HD-FDD) and TDD, respectively.

Table 1 Existing out-of-synch test cases in non-DRX in TS36.133.

| Existing test case in TS 38.133 | Duplex mode | Out-of-synch or early out-of-synch | CE Mode | BL UE / Non-BL UE |
| --- | --- | --- | --- | --- |
| A.7.3.48 | FD-FDD | Out-of-synch | Mode A | BL UE |
| A.7.3.52 | HD-FDD | Out-of-synch | Mode A | BL UE |
| A.7.3.56 | TDD | Out-of-synch | Mode A | BL UE |
| A.7.3.68 | FD-FDD | Early out-of-synch | Mode A | BL UE |
| A.7.3.69 | HD-FDD | Early out-of-synch | Mode A | BL UE |
| A.7.3.70 | TDD | Early out-of-synch | Mode A | BL UE |
| A.7.3.74 | FD-FDD | Out-of-synch | Mode A | non-BL UE |
| A.7.3.78 | TDD | Out-of-synch | Mode A | non-BL UE |
| A.7.3.82 | FD-FDD | Early out-of-synch | Mode B | BL UE |
| A.7.3.84 | HD-FDD | Early out-of-synch | Mode B | BL UE |
| A.7.3.86 | TDD | Early out-of-synch | Mode B | BL UE |

Proposal 1: Introduce new Out-of-synch test cases for MPDDCH performance improvement with FD-FDD/HD-FDD/TDD for BL UE CE Mode A.

Proposal 2: Introduce new Early out-of-synch test cases for MPDDCH performance improvement with FD-FDD/HD-FDD/TDD for BL UE CE Mode B.

## 2.2	Test points for Out-of-synch tests with MPDCCH performance improvement

For RLM OOS/IS test cases, RAN4 sets SNR levels based on the Qin/Qout as follows:

- SNR2 = Qout + margin1 dB,

- SNR3 = Qout – margin1 dB,

- SNR4 = Qin – margin2 dB,

- SNR5 = Qin + margin2 dB,

- SNR1 = SNR5,

where margin1/margin2 depend on the scenario like fading condition.

Since RAN4 agreed to define test cases only for out-of-synch for MPDCCH performance improvement, we focus on SNR2/SNR3, derived from Qout. According to our simulation results in [2], Qout for out-of-synch is improved by more than 1.0dB. We therefore propose to reuse the existing test cases but set SNR2/SNR3 1dB lower than the existing SNR levels, as shown in Figure 1.

Proposal 3: Set SNR2/SNR3 1dB lower compared with the existing out-of-synch/early out-of-synch test cases.

Figure 1	SNR2/SNR2 setting for RLM for MPDDCH performance improvement.

# 3		Summary

Proposal 1: Introduce new Out-of-synch test cases for MPDDCH performance improvement with FD-FDD/HD-FDD/TDD for BL UE CE Mode A.

Proposal 2: Introduce new Early out-of-synch test cases for MPDDCH performance improvement with FD-FDD/HD-FDD/TDD for BL UE CE Mode B.

Proposal 3: Set SNR2/SNR3 1dB lower compared with the existing out-of-synch/early out-of-synch test cases.

The detailed test case is given in [3].

# References

- R4-2012192, “Way forward on RRM performance requirements for MTC”, Ericsson.

- R4-2007367, “RLM for enhanced MPDCCH”, Ericsson.

- R4-2015842, “Draft CR: Test cases of RLM for MPDCCH performance improvement”, Ericsson.