3GPP TSG-RAN WG4 Meeting #97-e	R4-2015847

Electronic meeting, 2nd – 13th November, 2020

Source:	Ericsson

Title:	Discussion on HST UL TA remain issues.

Agenda item:	7.15.3.2.3

Document for:	Approval

# Introduction

In the last meeting, RAN4 got some agreements on organization, test metric and configuration, but there are still some issues can’t get agreement in WF [1].

- Issue 1: SCS/CBW combinations

Option 1: Have requirements for

15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

Option 2: Have requirements for

15kHz: 5MHz; 30kHz: 10MHz.

Option 3: Have requirements for all Rel-15 non-HST PUSCH bandwidths.

Other options not precluded.

- Issue 2: Applicability rules concerning SCS/CBW

Option 1: Re-use non-HST PUSCH applicability rules.

Option 2: Re-use HST PUSCH applicability rules.

Other options not precluded.

This contribution will keep on discussing remain issues.

# Discussion

Issue 1: SCS/CBW combinations

In the WF R4-2012676, the channel model for scenario X only mentioned TDLC300-400, but the definition for Δω is 15kHz: 0.04s-1 and 30kHz: 0.08s-1 which is not aligned with each other. The channel model parameter should be defined for SCS separately, and TDLC300-800 for 30kHz need to be added.

Proposal 1: Modify parameter table for scenario X as follows:

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 for 15kHz SCS TDLC300-800 for 30kHz SCS |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 us 30 kHz: 5 us |
| Δω | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

According to our simulation results [2], the performance difference between 5MHz and 10MHz BW for 15kHz SCS is about 1dB. The similar difference also happens between 10MHz and 40MHz BW for 30kHz SCS cases. The difference is not large and considering reducing test effort, so we think requirements for minimum bandwidth could be enough. We prefer Option 2.

Proposal 2: Agree with Option 2 to have requirements for 15kHz SCS 5MHz BW and 30kHz SCS 10MHzBW.

Issue 2: Applicability rules concerning SCS/CBW

We don’t see any special applicability rules for HST or non-HST. We can directly reuse current PUSCH applicability rule for UL TA requirements no matter which option will be agreed in Issue 1.

Proposal 3: Re-use current applicability rules for scenario X requirements.

# Conclusion

Issue 1: SCS/CBW combinations

Proposal 1: Modify parameter table for scenario X as follows:

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 for 15kHz SCS TDLC300-800 for 30kHz SCS |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 us 30 kHz: 5 us |
| Δω | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Proposal 2: Agree with Option 2 to have requirements for 15kHz SCS 5MHz BW and 30kHz SCS 10MHz BW.

Issue 2: Applicability rules concerning SCS/CBW

Proposal 3: Re-use current applicability rules for scenario X requirements.

# References

[1]	R4-2012676, “WF on Rel-16 NR HST PUSCH BS demodulation requirements”, RAN4#96-e, Nokia, Nokia Shanghai Bell, …

[2]	R4-2015848, “additional simulation results for NR PUSCH UL timing adjustments”, RAN4#97-e, Ericsson