3GPP TSG-RAN WG4 Meeting #97-e	R4-2015849

Electronic meeting, 2nd – 13th November, 2020

Source:	Ericsson

Title:	Simulation results for HST PRACH format in fading channel

Agenda item:	7.15.3.2.2

Document for:	Discussion

# Introduction

In the last meeting, RAN4 got a agreements to introduce requirements for PRACH restricted set A/B under fading channel [1]. In this contribution we deliver our simulation results.

- Assumptions

  - TDLC300-100 FO= 0Hz for PRACH with restricted set type A and B

  - Carrier frequency for velocity at 350km/h

    - Restricted set A: 2.1GHz

    - Restricted set B: 3.6GHz

  - Sub-carrier spacing: 1.25kHz

  - Number of transmitter antennas: 1 Tx

  - Total number of PRACH preambles: 64

# Discussion

The simulation results as listed in following table.

Table 2-1 ideal simulation results for PRACH format 0 restricted set type A

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 0 Hz | -8.9 |
|  | 4 | TDLC300-100 Low | 0 Hz | -14.3 |
|  | 8 | TDLC300-100 Low | 0 Hz | -18.4 |

Table 2-2 ideal simulation results for PRACH format 0 restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 0 Hz | -8.8 |
|  | 4 | TDLC300-100 Low | 0 Hz | -14.1 |
|  | 8 | TDLC300-100 Low | 0 Hz | -18.2 |

Table 2-3 impairment simulation results for PRACH format 0 restricted set type A

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 0 Hz | -6.4 |
|  | 4 | TDLC300-100 Low | 0 Hz | -11.8 |
|  | 8 | TDLC300-100 Low | 0 Hz | -15.9 |

Table 2-4 impairment simulation results for PRACH format 0 restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 |
| 1 | 2 | TDLC300-100 Low | 0 Hz | -6.3 |
|  | 4 | TDLC300-100 Low | 0 Hz | -11.6 |
|  | 8 | TDLC300-100 Low | 0 Hz | -15.7 |

# Conclusion

Simulation results for PRACH format 0 restricted set A and B under fading channel are listed in section 2.

# References

- [1]	R4-2012676, “WF on Rel-16 NR HST PUSCH BS demodulation requirements”, RAN4#96-e, Nokia, Nokia Shanghai Bell, …