3GPP TSG-RAN WG4 Meeting #97-e	R4-2015854

Electronic meeting, 2nd – 13th November, 2020

Source:	Ericsson

Title:	Discussion on NR-U PRACH demodulation assumptions

Agenda item:	7.1.8.4.3

Document for:	Discussion

# Introduction

In RAN4#96-e meeting, companies got some agreements on NR-U demodulation assumptions and also left some open issues [1]. This contribution will keep on discussing remain issues and deliver our opinions on NR-U PRACH simulation assumptions.

# Discussion

One of Rel-16 PRACH enhancements for NR-U is to support larger bandwidth PRACH occasions with Ax, Bx, Cx formats. Compared with Rel-15, PRACH occasions bandwidth is extended to almost 20MHz. In the last meeting, companies agreed to introduce new requirements for this case.

For the simulation assumptions, we think it can be a start to reuse most of Rel-15 assumptions on unlicensed band.

We propose simulation assumptions for further discussion can be as follows:

- Sequence format:

Considering the requirements are for FR1 and reducing test effort, it’s no need to introduce requirements for all Rel-15 short formats. We can only test A2,  B4, C2 or less.

- SCS

As agreed in the last meeting, define requirements for both 15kHz and 30kHz.

- LRA=1151for 15kHz SCS571for 30kHz SCS

- Channel model:

If we only use one multipath fading channel for PRACH requirements as Rel-15, we think TDLA30-10 might be more suitable according to NR-U deployment, but we are open for TDLB100 and TDLC300 [2].

Doppler shift value can also be further discussed.

- Timing error tolerance:

In Rel-15 PRACH assumptions, timing error tolerance for AWGN is 0.52us and 0.26us. Considering enhanced PRACH format is extend to 17.28MHz which is 8 and 4 times of normal PRACH bandwidth in 15kHz and 30kHz SCS separately. A simple way to define timing error tolerance is directly divide Rel-15 assumptions by 8 or 4 times, such as 0.52us/8 = 0.26us/4 = 0.065us for both 15kHz and 30kHz SCS.

It should be noted that the time resolution of PRACH is ~0.058us for enhanced PRACH bandwidth 17.28MHz.

Table 2-1: Time error tolerance for AWGN and TDLA30-10

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLA30-10 |
| A2, B4, C2 | 15 | [0.065us] | FFS |
|  | 30 |  | FFS |

- Frequency offset:

0Hz for AWGN and FFS for TDLA30-10

Proposal 1: Simulation assumptions for further discussion

- Sequence format: A2, B4, C2

- SCS: 15kHz and 30kHz for n46 band

- LRA=1151for 15kHz SCS571for 30kHz SCS

- Channel model: AWGN and TDLA30-10

- Timing error tolerance:

Table 2-1: Time error tolerance for AWGN and TDLA30-10

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLA30-10 |
| A2, B4, C2 | 15 | [0.065us] | FFS |
|  | 30 |  | FFS |

- Frequency offset: 0Hz for AWGN and FFS for TDLA30-10

# Conclusion

Proposal 1: Simulation assumptions for  enhanced PRACH format

- Sequence format: A2, B4, C2

- SCS: 15kHz and 30kHz for n46 band

- LRA=1151for 15kHz SCS571for 30kHz SCS

- Channel model: AWGN and TDLA30-10

- Timing error tolerance:

Table 2-1: Time error tolerance for AWGN and TDLA30-10

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLA30-10 |
| A2, B4, C2 | 15 | [0.065us] | FFS |
|  | 30 |  | FFS |

- Frequency offset: 0Hz for AWGN and FFS for TDLA30-10

# References

[1]	R4-2012611, “WF on NR-U BS demodulation requirements”, RAN4#96-e, Ericsson

[2]	R4-2015851, “General issue on NR-U demodulation requirements”, RAN4#97-e, Ericsson