3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2015855

# Electronic Meeting, 2-13 Nov., 2020

Source:	Sony, Ericsson

Title:	Link budget for PC3 for n262

Agenda item:	10.28.1

Document for:	Approval

- Background

The work item introducing the new FR2 47GHz band was approved in RAN #88-e [1]. It was discussed during RAN4 #96-e [2] and a WF was approved [3] where companies are encouraged to provide the link budget parameters for Tx and Rx. Another WF on UE RF requirements was also approved [4], not discussed in this document.

In this contribution we provide our inputs to PC3 link budget for n262.

- Transmitter, PC 3

Power class is defined as maximum peak EIRP, minimum peak EIRP, spherical coverage EIRP and maximum TRP. The n262 band (47.2-48.2GHz) is part of FR2 (24.25GHz – 52.6GHz) it is, therefore, proposed to reuse maximum peak EIRP and maximum TRP from other FR2 bands. For PC3 that is: maximum peak EIRP = 43dBm and maximum TRP = 23dBm.

Proposal 1	For PC3: n262 reuse maximum peak EIRP and maximum TRP from PC3: n260

## RF Architecture

For PC3 in bands n257, n258, n260 and n261, the peak EIRP was derived assuming an HW architecture where each antenna element is individually fed via a power amplifier on each polarization. To our understanding, this is the reason there was an assumption of 2-3 dB polarization gain when deriving the peak EIRP for bands n257, n258, n259, n260 and n261.  The same HW architecture has been assumed in the WF [3] where companies are encouraged to fill in values according to this HW architecture. Other possible RF architectures could be a single PA with splitters for each antenna element (e.g., Butler matrix). Even if other possible HW architectures could be applicable, they may not end up with very different TRP/EIRP values in the end.

Observation 1 	A dual-polarized antenna structure with PA for each polarization was assumed when defining the peak EIRP for bands n257  n261.

Proposal 2	Companies shall provide the reference RF architecture they assumed when deriving the peak EIRP link budget.

## Peak EIRP PC3

Calculation of peak EIRP for band n262 is shown in Table 1

| Parameter | Unit | Nominal value | Contribution to tolerance |
| --- | --- | --- | --- |
| Frequency range | GHz | 47.2 48.2 GHz |  |
| Pout per element | dBm | 10 |  |
| # of antennas in an array |  | 4 |  |
| Total conducted power per polarization | dBm | 16 | 0.5 |
| Avg antenna element gain | dBi | 5.5 |  |
| Antenna roll off loss versus frequency | dB | 0.5 |  |
| Realized antenna array gain | dBi | 10.5 | 1 |
| Polarization gain | dB | 2.5 |  |
| Mismatch and transmission line loss including load pull | dB | 1 | 1 |
| Beam forming loss (phase shifter and amplitude error) | dB | 1 | 0.5 |
| Finite beam table | dB | 0 |  |
| Beam forming loss (one beam table fits all) | dB | 0 |  |
| Form factor integration losses | dB | 4 | 2 |
| Total implementation loss (nominal) | dB | 6 |  |
| Total implementation loss (worst case) | dB | 9.5 |  |
| Peak EIRP (Nominal) | dBm | 23 |  |
| Tolerance (+/-) | dB | 5 |  |
| Peak EIRP (Minimum) | dBm | 18 |  |
| Peak EIRP (Maximum) | dBm |  |  |

Table 1 Estimation on Peak EIRP for PC3: n262

Observation 2	According to our estimate (minimum) peak EIRP is 18 dBm for PC3: n262

## Spherical coverage PC3

The spherical coverage of an antenna in this frequency range depends on many factors, including, but not limited to, the surface current distribution, the material of the back cover and its separation distance to the antennas, other components around the antenna panel etc. During the discussion of spherical coverage for the previously defined FR2 frequency bands, it was observed that impact from all the factors does not monotonously change with frequency but rather changes periodicity (see [6]). Therefore, it cannot be concluded that the n262 band must be worse than e.g., n259 band in terms of spherical coverage. The total impact (peak and spherical coverage) of the back cover and components around the antenna panel, on the other hand, will increase with frequency.

Observation 3	The spherical coverage performance (delta between peak and 50% EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n259 in terms of spherical coverage.

We therefore propose:

Proposal 3	Further study the spherical coverage requirement (delta between peak and 50% EIRP) of n262.

## Multi-band relaxation for PC3

Multi-band relaxation was agreed as a trade-off between link budget and implementation constraints. For example, in [6] it was shown that there is a clear trade-off between antenna volume and multi-band performance. Band n262 (47.2-48.2GHz) is quite narrow (2.1%) compared to other bands and multiband relaxation can, therefore, follow the multiband relaxation of n259 and n260 which is MBP,n = 0.5dB and MBS,n = 0.4dB.

Observation 4	Multi-band relaxation for specification (Table 6.2.2.3-4 in TS 38.101-2 ) for n262 shall be MBP,n=0.5dB and MBS,n=0.4dB.

- Receiver PC3

## Reference sensitivity

The reference sensitivity can be described as below [5]:

Sensitivity= -174dBm+10*logBW+NF-Array gain-diversity gain+SNR+IM

SNR refers to achieving 95% BER. IM is the baseband implementation margin (i.e., uncertainty in the detector implementation) not to confuse with the uncertainty in the NF (i.e., front end HW). The implementation margin is not present in the WF [3], and we therefore bundle it in the SNR requirement, which is then estimated to 0 dB (SNR itself is -1 dB). HW uncertainty (margin) is usually bundled into the NF, and the NF is estimated to 12.5 dB, which includes the implementation margin and margin for ETC case. In order to follow the WF document [3] losses due to beamformer and form factor are separate and not included in the NF. (Alternatively, they could be bundled into the NF since we have the OTA approach for FR2). The “Total Insertion Loss” then consists of losses before the front end which are form factor integration losses and transmission line mismatches (worst case). The BW is the occupied BW (~channel BW) here chosen to 100MHz, but the REFSENS could be scaled accordingly. Diversity gain was decided to be 0 dB for FR2 [5] due to the OTA verification procedure. The “array gain” in the formula refers to the total antenna gain (element gain + array gain - efficiency loss).

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Band number |  | n262 |  |
| Frequency range | GHz | 47.2 – 48.2 |  |
| Modulation |  | QPSK |  |
| SNR requirement | dB | 0 | Including 1dB demodulator IM |
| Bandwidth | MHz | 100 |  |
| Thermal noise | dBm/Hz | -174 |  |
| Noise Figure | dB | 12.5 | Worst case NF including HW "implementation margin" and ETC case |
| Number of antennas in an array |  | 4 |  |
| Array gain | dB | 5.5 |  |
| Element gain | dBi | 4.5 | 5.5 ± 1 |
| Diversity gain | dB | 0 | Due to the measurement method |
| Antenna gain roll-off over frequency | dB | 0.5 | Relative band is “only” 2.1% |
| Beamforming loss | dB | 1 | 1.2 ± 0.5 |
| Total insertion loss | dB | 8 | Including form factor integration losses (worst case) |
| REFSENS | dBm | -81.5 |  |

Table 2 Estimation of REFSENS for PC3: n262

Observation 5	According to our estimate REFSENS for PC3: n262 is -81.5 dBm

- Conclusion

In this contribution we have discussed the requirement for PC3: n262. The following observations and proposal are made:

Observation 1 	A dual-polarized antenna structure with PA for each polarization was assumed when defining the peak EIRP for bands n257  n261.

Observation 2	According to our estimate (minimum) peak EIRP is 18 dBm for PC3: n2

Observation 3	The spherical coverage performance (delta between peak and 50% EIRP) depends on many factors, and it cannot be concluded that the n262 band must be worse than, e.g., n259 in terms of spherical coverage.

Observation 4	Multi-band relaxation for specification (Table 6.2.2.3-4 in TS 38.101-2 ) for n262 shall be MBP,n=0.5dB and MBS,n=0.4dB.

Observation 5	According to our estimate REFSENS for PC3: n262 is -81.5 dBm

Proposal 1	For PC3: n262 reuse maximum peak EIRP and maximum TRP from PC3: n260

Proposal 2	Companies shall provide the reference RF architecture they assumed when deriving the peak EIRP link budget.

Proposal 3	Further study the spherical coverage requirement (delta between peak and 50% EIRP) of n262.

- References

- RP-201232, 	“New WID: introduction of NR 47GHz band, RAN #88-e, June 2020

- R4-2011873,	“Email discussion summary for [96e][135] NR_47GHz_Band”, Nokia

- R4-2011817, 	“WF on link budget parameters for Tx/Rx of n262”, Huawei, HiSilicon, Apple

- R4-2011818,	“WF on UE RF requirement for 47 GHz band”, Qualcomm Incorporated

- R4-1709951,	“Way forward on NR REFSENS SNR simulation assumptions”, Huawei, Intel

- R4-1815845,	“UE Spherical EIRP for the 28GHz and 39GHz bands”, Sony