3GPP TSG-RAN WG4 #97-e	R4-2015858

Electronic meeting. 2nd – 13th November 2020

Source: 	Ericsson

Title:  	Simulation results for 2-step RACH

Agenda Item:	7.18.3

Document for:	Information

# Introduction

During RAN4#96-e. a WF [1] was agreed for 2-step RACH in which a number of parameters were agreed. In this contribution. we presented our simulation results based on the agreed simulation assumptions and open options.

# Simulation assumptions and simulation results

In this section. simulation assumptions used in the simulation are presented in Table 1 and the corresponding simulation (i.e. ideal) results and results with impairments are summarized in Table 2.

Table 1: Simulation assumptions.

| Parameter | Values for FR1 | Values for FR2 |
| --- | --- | --- |
| Preamble format | Free choice |  |
| Waveform | CP-OFDM |  |
| Power offset between preamble and MsgA | Free choice |  |
| Subcarrier spacing for PUSCH | 15kHz. 30kHz | 60kHz. 120 kHz |
| PUSCH Mapping Type | Type A and Type B | Type B |
| MCS level | 1 | 2 |
| Number of symbols | 14 | 10 |
| Number of PRBs | 2 | 2 |
| TBS | 72 bits |  |
| DMRS | Option 1: 1+1+1 Option 2: 1+1 | 1+1 |
| Antenna configuration | 1T2R | 1T2R |
| Propagation channel | TDLC300-100 | TDLA30-300 |
| TO values | Medium level cycling values: 15k SCS: [0:0.4:2]. 30k SCS: [0:0.2:1] [High level cycling values 15k SCS: [0:0.1:3.8]. 30k SCS: [0:0.1:3.8]] | Medium level cycling values: 60k SCS: [0:0.1.0.5]. 120 SCS: [0:0.05.0.25] [High level cycling values: 60k SCS: [0:0.1.0.6]. 120 SCS: [0:0.1.0.6]] |
| Test metric (BLER of MsgA when preambles are correctly detected) | Baseline: 0.01 | Baseline: 0.01 |
| Re-transmission | No retransmission considered |  |

Table 2: Ideal simulation results (with T0 compensation on) and results with impairments.

| [SCS. PUSCH mapping type. DM-RS configuration] | T0 values | Ideal results |  | Results with impairments |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR @ BLER = 0.1 | SNR @ BLER = 00.1 | SNR @ BLER = 0.1 | SNR @ BLER = 00.1 |
| FR1 |  |  |  |  |  |
| 15 kHz. mapping type A (1+1) DM-RS symbols | Medium | -0.5 | 4.9 | 1.7 | 7.3 |
| 15 kHz. mapping type A (1+1+1) DM-RS symbols |  | -0.6 | 4.8 | 1.8 | 7.1 |
| 15 kHz. mapping type B (1+1) DM-RS symbols |  | -0.5 | 4.9 | 1.5 | 6.9 |
| 15 kHz. mapping type B (1+1+1) DM-RS symbols |  | -0.6 | 4.8 | 1.8 | 6.8 |
| 15 kHz. mapping type A (1+1) DM-RS symbols | High | -0.5 | 4.8 | 1.9 | 7.1 |
| 15 kHz. mapping type A (1+1+1) DM-RS symbols |  | -0.6 | 4.7 | 1.9 | 7.1 |
| 15 kHz. mapping type B (1+1) DM-RS symbols |  | -0.5 | 4.9 | 2 | 7.1 |
| 15 kHz. mapping type B (1+1+1) DM-RS symbols |  | -0.6 | 4.8 | 1.7 | 6.8 |
| 30 kHz. mapping type A (1+1) DM-RS symbols | Medium | -0.4 | 4.5 | 1.7 | 6.7 |
| 30 kHz. mapping type A (1+1+1) DM-RS symbols |  | -0.5 | 4.4 | 1.9 | 6.6 |
| 30 kHz. mapping type B (1+1) DM-RS symbols |  | -0.4 | 4.5 | 2 | 6.6 |
| 30 kHz. mapping type B (1+1+1) DM-RS symbols |  | 0.1 | 5.1 | 2.6 | 7.6 |
| 30 kHz. mapping type A (1+1) DM-RS symbols | High | -0.3 | 4.7 | 2.2 | 6.8 |
| 30 kHz. mapping type A (1+1+1) DM-RS symbols |  | -0.4 | 4.6 | 2 | 6.9 |
| 30 kHz. mapping type B (1+1) DM-RS symbols |  | -0.3 | 4.6 | 2.1 | 7 |
| 30 kHz. mapping type B (1+1+1) DM-RS symbols |  | 0.2 | 5.2 | 2.5 | 7.5 |
| FR2 |  |  |  |  |  |
| 60 kHz. mapping type B (1+1) DM-RS symbols | Medium | 0.6 | 6.3 | 3 | 8.3 |
|  | High | 0.6 | 6.3 | 3 | 8.8 |
| 120 kHz. mapping type B (1+1) DM-RS symbols | Medium | 0.6 | 6 | 2.8 | 8.2 |
|  | High | 0.6 | 6 | 2.7 | 8.4 |

Observation 1: The difference in PUSCH performance is insignificant between (1+1) and (1+1+1) DM-RS symbols.

Observation 2: T0 could be compensated satisfactory in the presence of medium or high level T0.

In Table 3, we compare the difference between uncompensated and compensated simulation results.

Table 3: Comparison between uncompensated and compensated simulation results.

| [SCS. PUSCH mapping type. DM-RS configuration] | T0 values | Uncompensated SNR @ BLER = 0.01 | Compensated SNR @ BLER = 0.01 |
| --- | --- | --- | --- |
| 15 kHz. mapping type A (1+1+1) DM-RS symbols | Medium | 6.8 | 4.8 |
| 15 kHz. mapping type A (1+1+1) DM-RS symbols | High | 8.5 | 4.7 |
| 30 kHz. mapping type A (1+1+1) DM-RS symbols | Medium | 6.8 | 4.4 |
| 30 kHz. mapping type A (1+1+1) DM-RS symbols | High | NA | 4.6 |
| 60 kHz. mapping type B (1+1) DM-RS symbols | Medium | 8 | 6.3 |
| 60 kHz. mapping type B (1+1) DM-RS symbols | High | 8.4 | 6.3 |
| 120 kHz. mapping type B (1+1) DM-RS symbols | Medium | 7.5 | 6 |
| 120 kHz. mapping type B (1+1) DM-RS symbols | High | 10 | 6 |

Observation 3: Difference between compensated and uncompensated results at medium T0 level is ~2dB which could be averaged out by results from different companies and may not necessary be able to test the performance of the T0 compensation algorithm.

# Conclusion

In this contribution. we present our simulation results and make the following observations:

Observation 1: The difference in PUSCH performance is insignificant between (1+1) and (1+1+1) DM-RS symbols.

Observation 2: T0 could be compensated satisfactory in the presence of medium or high level T0.

Observation 3: Difference between compensated and uncompensated results at medium T0 level is ~2dB which could be averaged out by results from different companies and may not necessary be able to test the performance of the T0 compensation algorithm.

# References

R4-2012705. “WF on BS demodulation requirements for 2-step RACH”. RAN4#96-e. 17th-28th August. 2020.