3GPP TSG-RAN WG4 Meeting #97-e                                                       R4-2015864

Electronic meeting, 2nd – 13th November, 2020

Agenda item:	7.8.1.1.2

Source:	Ericsson

Title:	Simulation results on URLLC UE CQI reporting requirements for CQI table 3

Document for:	Information

# Introduction

In this paper, we present our simulation results on URLLC UE CQI reporting test for CQI table 3.

# Simulation Results

## Simulation configurations

It is proposed that the same parameters are applied for the CQI test as for the FMCS test as far as possible. Therefore, our simulations are based on the already agreed simulation assumptions for UE ultra-low BLER test as presented in WF[1], as well as the test configurations defined for Rel-15 CQI reporting tests given by Table 6.2.2.1.1.1-1, Table 6.2.2.2.1.1-1, Table 6.2.3.1.1.1-1 and Table 6.2.3.2.1.1-1 in 3GPP TS 38.101-4 [2]. The simulation configurations are summarized in table1 below.

Table 1 Simulation configurations for CQI table 3 tests

| Parameter |  |  | Unit | FDD |  | TDD |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |  |
| Subcarrier spacing |  |  | kHz | 15 | 30 |  |
| MCS table |  |  |  | Table 3 |  |  |
| PDSCH starting symbol/length |  |  |  | 2/12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 |  |  |
| PDSCH mapping type |  |  |  | Type A |  |  |
| DMRS type |  |  |  | Type 1 |  |  |
| DMRS duration |  |  |  | single-symbol DMRS |  |  |
| Number of additional DMRS |  |  |  | 1 |  |  |
| Slot pattern |  |  |  | N/A | 7D1S2U, S=6D: 4G: 4U |  |
| Propagation channel |  |  |  | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2, ULA low​ 1x4, ULA low​ |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  | 1A |  |  |
|  | CDM Type |  |  | No CDM |  |  |
|  | Density (ρ) |  |  | 1 |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 2,4 |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |  |
|  | CDM Type |  |  | No CDM |  |  |
|  | Density (ρ) |  |  | 3 |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  |  | Row 1,(0,-) |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |  |
| CQI-table |  |  |  | Table 3 |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |
| Sub-band Size |  |  | RB | 8 | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |
|  |  | RI Restriction |  | 00000001 |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |
| Target BLER |  |  |  | 10^-5 |  |  |
| PBCH |  |  |  | slot#0 per 20ms periodicity |  |  |
| PT-RS |  |  |  | Disabled |  |  |
| PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL |  |  |  |  |  |  |

## Simulation results

### The simulation results for different scenarios are summarized below.

Table 2 CQI reporting results for FDD with 1Tx2Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 0 | 1.6573e-01 | 100 | 1.6573e-01 | 1.6573e-01 | 1.6573e-01 |
| -11 | 0 | 6.7600e-02 | 100 | 6.7600e-02 | 6.7600e-02 | 6.7600e-02 |
| -10 | 0 | 2.3111e-03 | 100 | 2.4481e-03 | 2.4481e-03 | 2.4481e-03 |
| -9 | 1 | 6.0469e-07 | 100 | 0 | 0 | 1.3200e-02 |
| -8 | 2 | 4.8375e-06 | 100 | 0 | 4.6702e-06 | 4.1200e-02 |
| -7 | 3 | 1.9350e-05 | 100 | 0 | 2.2949e-05 | 8.0533e-02 |
| -6 | 4 | 1.3605e-04 | 100 | 0 | 1.2851e-04 | 1.8253e-01 |
| -5 | 4 | 0 | 100 | 0 | 0 | 1.5600e-02 |
| -4 | 5 | 0 | 100 | 0 | 0 | 1.9667e-01 |
| -3 | 5 | 0 | 100 | 0 | 0 | 3.4188e-03 |
| -2 | 6 | 0 | 100 | 0 | 0 | 1.6347e-01 |
| -1 | 6 | 0 | 100 | 0 | 0 | 6.8847e-05 |
| 0 | 7 | 0 | 100 | 0 | 0 | 5.6400e-02 |
| 1 | 8 | 0 | 100 | 0 | 0 | 9.7653e-01 |
| 2 | 8 | 0 | 100 | 0 | 0 | 1.6933e-02 |
| 3 | 9 | 0 | 100 | 0 | 0 | 8.6693e-01 |
| 4 | 9 | 0 | 100 | 0 | 0 | 5.3333e-03 |
| 5 | 10 | 0 | 100 | 0 | 0 | 5.3267e-01 |
| 6 | 10 | 0 | 100 | 0 | 0 | 1.1333e-03 |
| 7 | 11 | 0 | 100 | 0 | 0 | 4.2067e-01 |
| 8 | 11 | 0 | 100 | 0 | 0 | 3.2129e-05 |
| 9 | 12 | 0 | 100 | 0 | 4.5898e-06 | 8.6933e-02 |
| 10 | 12 | 0 | 100 | 0 | 0 | 0 |
| 11 | 13 | 0 | 100 | 0 | 0 | 1.1907e-01 |
| 12 | 13 | 0 | 100 | 0 | 0 | 2.2949e-05 |
| 13 | 14 | 0 | 100 | 0 | 0 | 1.2013e-01 |
| 14 | 14 | 0 | 100 | 0 | 0 | 4.5898e-06 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 3 CQI reporting results for FDD with 1Tx4Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 1 | 4.6296e-04 | 100 | 4.5977e-04 | 4.5977e-04 | 5.4359e-03 |
| -11 | 2 | 1.1828e-03 | 100 | 0 | 1.1240e-03 | 6.7302e-03 |
| -10 | 3 | 2.3750e-03 | 100 | 4.5898e-06 | 2.3169e-03 | 1.2667e-02 |
| -9 | 4 | 3.8857e-03 | 100 | 0 | 3.8519e-03 | 4.8453e-01 |
| -8 | 4 | 1.0884e-05 | 100 | 0 | 4.5898e-06 | 7.7333e-03 |
| -7 | 5 | 5.2130e-04 | 100 | 0 | 5.2107e-04 | 5.9800e-01 |
| -6 | 5 | 0 | 100 | 0 | 0 | 6.1449e-03 |
| -5 | 6 | 0 | 100 | 0 | 9.3403e-06 | 3.4693e-01 |
| -4 | 6 | 0 | 100 | 0 | 0 | 2.2732e-03 |
| -3 | 7 | 0 | 100 | 0 | 0 | 2.8267e-02 |
| -2 | 8 | 0 | 100 | 0 | 3.6718e-05 | 9.9987e-01 |
| -1 | 8 | 0 | 100 | 0 | 0 | 9.0667e-03 |
| 0 | 9 | 0 | 100 | 0 | 0 | 9.9773e-01 |
| 1 | 9 | 0 | 100 | 0 | 0 | 3.9259e-03 |
| 2 | 10 | 0 | 100 | 0 | 0 | 9.5293e-01 |
| 3 | 10 | 0 | 100 | 0 | 0 | 9.1275e-04 |
| 4 | 11 | 0 | 100 | 0 | 0 | 8.2400e-01 |
| 5 | 11 | 0 | 100 | 0 | 0 | 1.5146e-04 |
| 6 | 12 | 0 | 100 | 0 | 0 | 6.4133e-02 |
| 7 | 12 | 0 | 100 | 0 | 0 | 0 |
| 8 | 13 | 0 | 100 | 0 | 0 | 9.2667e-02 |
| 9 | 13 | 0 | 100 | 0 | 0 | 6.8847e-05 |
| 10 | 14 | 0 | 100 | 0 | 0 | 1.4947e-01 |
| 11 | 14 | 0 | 100 | 0 | 0 | 4.5898e-06 |
| 12 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 4 CQI reporting results for TDD with 1Tx2Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 0 | 1.9146e-01 | 100 | 1.9146e-01 | 1.9146e-01 | 1.9146e-01 |
| -11 | 0 | 8.9547e-02 | 100 | 8.9547e-02 | 8.9547e-02 | 8.9547e-02 |
| -10 | 1 | 7.2894e-04 | 100 | 7.5418e-04 | 7.5418e-04 | 1.4686e-01 |
| -9 | 1 | 0 | 100 | 0 | 0 | 1.5505e-02 |
| -8 | 2 | 0 | 100 | 0 | 0 | 3.5366e-02 |
| -7 | 3 | 0 | 100 | 0 | 0 | 1.0679e-01 |
| -6 | 4 | 0 | 100 | 0 | 3.1055e-05 | 2.1167e-01 |
| -5 | 4 | 0 | 100 | 0 | 0 | 2.2300e-02 |
| -4 | 5 | 0 | 100 | 0 | 0 | 2.5348e-01 |
| -3 | 5 | 0 | 100 | 0 | 0 | 6.0074e-03 |
| -2 | 6 | 0 | 100 | 0 | 0 | 2.1620e-01 |
| -1 | 6 | 0 | 100 | 0 | 0 | 1.6148e-04 |
| 0 | 7 | 0 | 100 | 0 | 0 | 7.4564e-02 |
| 1 | 8 | 0 | 100 | 0 | 0 | 9.9965e-01 |
| 2 | 8 | 0 | 100 | 0 | 0 | 1.5505e-02 |
| 3 | 9 | 0 | 100 | 0 | 0 | 9.4861e-01 |
| 4 | 9 | 0 | 100 | 0 | 0 | 4.9361e-03 |
| 5 | 10 | 0 | 100 | 0 | 0 | 9.6533e-01 |
| 6 | 10 | 0 | 100 | 0 | 0 | 4.2762e-03 |
| 7 | 11 | 0 | 100 | 0 | 0 | 2.0488e-01 |
| 8 | 11 | 0 | 100 | 0 | 0 | 6.2109e-06 |
| 9 | 12 | 0 | 100 | 0 | 0 | 1.8397e-01 |
| 10 | 12 | 0 | 100 | 0 | 0 | 1.8633e-05 |
| 11 | 13 | 0 | 100 | 0 | 0 | 7.3171e-02 |
| 12 | 13 | 0 | 100 | 0 | 0 | 0 |
| 13 | 14 | 0 | 100 | 0 | 0 | 1.4251e-01 |
| 14 | 14 | 0 | 100 | 0 | 0 | 6.2109e-06 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 5 CQI reporting results for TDD with 1Tx4Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 1 | 5.1473e-04 | 100 | 5.1090e-04 | 5.1090e-04 | 1.3240e-02 |
| -11 | 2 | 2.3229e-03 | 100 | 0 | 2.3543e-03 | 1.4634e-02 |
| -10 | 3 | 3.2907e-03 | 100 | 0 | 3.2871e-03 | 1.4460e-02 |
| -9 | 4 | 8.1748e-03 | 100 | 0 | 8.1748e-03 | 6.9530e-01 |
| -8 | 4 | 9.3039e-06 | 100 | 0 | 1.2422e-05 | 1.0801e-02 |
| -7 | 5 | 8.0836e-04 | 100 | 0 | 7.9943e-04 | 8.7578e-01 |
| -6 | 5 | 0 | 100 | 0 | 0 | 1.2195e-02 |
| -5 | 6 | 0 | 100 | 0 | 1.8633e-05 | 7.1202e-01 |
| -4 | 6 | 0 | 100 | 0 | 0 | 9.0592e-03 |
| -3 | 7 | 0 | 100 | 0 | 0 | 3.7108e-02 |
| -2 | 8 | 0 | 100 | 0 | 1.0374e-04 | 1.0000e+00 |
| -1 | 8 | 0 | 100 | 0 | 0 | 1.4111e-02 |
| 0 | 9 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 1 | 9 | 0 | 100 | 0 | 0 | 1.0627e-02 |
| 2 | 10 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 3 | 10 | 0 | 100 | 0 | 0 | 5.7709e-03 |
| 4 | 11 | 0 | 100 | 0 | 0 | 3.9965e-01 |
| 5 | 11 | 0 | 100 | 0 | 0 | 6.2109e-06 |
| 6 | 12 | 0 | 100 | 0 | 0 | 1.4512e-01 |
| 7 | 12 | 0 | 100 | 0 | 0 | 1.2422e-04 |
| 8 | 13 | 0 | 100 | 0 | 0 | 2.0209e-02 |
| 9 | 13 | 0 | 100 | 0 | 0 | 0 |
| 10 | 14 | 0 | 100 | 0 | 0 | 1.0871e-01 |
| 11 | 14 | 0 | 100 | 0 | 0 | 6.2109e-06 |
| 12 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

# Conclusion

The paper presents simulation results for URLLC UE CQI reporting tests for CQI table 3.

# References

- R4-2012646, “Way Forward on ultra-low BLER requirements”, Ericsson

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception, Part 4: Performance requirements (Release 15)