3GPP TSG-RAN WG4 Meeting #97-e	R4-2015904

# Electronic meeting, November 2nd – 13th, 2020

Source:		Ericsson

Title:			BS RF Requirements and System parameters - TP to TR 38.847

Agenda item:		10.28.2

Document for:		Approval

# Introduction

In RAN#88e, a WID [1] to introduce the 47 GHz band was approved. The purpose of the WID is to specify NR operating band covering the entire 47.2-48.2 GHz in order to accommodate operations in any part of this range.

In last RAN4#96-e meeting, the impacts on BS RF requirements and system parameters ([2], [3], [4] and [5]) were agreed, re-using existing requirements from band n259/n260 as basis. This contribution is proposing a text proposal to TR 38.847 to capture those agreements.

# References

- RP-201232, New WID on introduction of NR 47 GHz band, T-Mobile USA

- R4-2011412, BS RF requirements for 47 GHz band, Nokia

- R4-2010447, 47GHz band - Regulatory overview – Band plan - System parameters, Ericsson

- R4-2010522, Band number and System parameters of 47 GHz band, Nokia

- R4-2011873, Email discussion summary for [96e][135] NR_47GHz_Band

# Text Proposal

<Start of the change>

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Global frequency raster granularity

ΔFRaster	Channel raster granularity

FREF-Offs	Offset used for calculating FREF

Fstep,X	Frequency steps for the OTA transmitter spurious emissions

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

BW	Bandwidth

EIRP	Effective Isotropic Radiated Power

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

ITU-R	Radiocommunication Sector of the International Telecommunication Union

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OTA	Over The Air

RF	Radio Frequency

RX	Receiver

SCS	Sub-Carrier Spacing

TDD	Time division Duplex

# 4	Background

## 4.1	ITU-R

As part of facilitating the development of 5G mobile networks, WRC-19 identified the 47 GHz frequency band (47.2-48.2 GHz) for International Mobile Telecommunications (IMT). Table 4.1-1 is the extract of the Radio Regulations (XX) table of allocations providing the services allocated in the 47.2-48.2 GHz frequency range

Table 4.1-1: Allocation information in the 47.2-48.2 GHz frequency range

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47.2-47.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B 5.552A |  |  |
| 47.5-47.9 FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 (space-to-Earth) 5.516B 5.554A MOBILE 5.553B | 47.5-47.9 FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B |  |
| 47.9-48.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.553B 5.552A |  |  |

With the footnote 5.553B: In Region 2 and Algeria, Angola, Saudi Arabia, Australia, Bahrain, Benin, Botswana, Burkina Faso, Burundi, Cameroon, Central African Rep., Comoros, Congo (Rep. of the), Korea (Rep. of), Côte d’Ivoire, Djibouti, Egypt, United Arab Emirates, Eswatini, Ethiopia, Gabon, Gambia, Ghana, Guinea, Guinea-Bissau, Equatorial Guinea, India, Iran (Islamic Republic of), Iraq, Japan, Jordan, Kenya, Kuwait, Lesotho, Liberia, Libya, Lithuania, Madagascar, Malaysia, Malawi, Mali, Morocco, Mauritius, Mauritania, Mozambique, Namibia, Niger, Nigeria, Oman, Uganda, Qatar, the Syrian Arab Republic, the Dem. Rep. of the Congo, Rwanda, Sao Tome and Principe, Senegal, Seychelles, Sierra Leone, Singapore, Slovenia, Somalia, Sudan, South Sudan, South Africa, Sweden, Tanzania, Chad, Togo, Tunisia, Zambia and Zimbabwe, the frequency band 47.2-48.2 GHz is identified for use by administrations wishing to implement International Mobile Telecommunications (IMT). This  identification does not preclude the use of this frequency band by any application of the services to which it is allocated, and does not establish any priority in the Radio  Regulations. Resolution 243 (WRC-19) applies. (WRC-19)

## 4.2	FCC

The US FCC auctioned the 47 GHz band beginning in December 2019 as part of Auction 103. The 47 GHz band (47.2-48.2 GHz) was auctioned in 10 blocks of 100 megahertz in each PEA (Partial Economic Area) license. The auction was concluded on 3/5/2020.

The channel arrangement, power limits and unwanted emissions are specified in CFR Title 47 Part 30 (XX) and captured in Table 4.2-1.

Table 4.2-1: FCC requirements

| Channel arrangement | 47.2-47.3 GHz; 47.3-47.4 GHz; 47.4-47.5 GHz; 47.5-47.6 GHz; 47.6-47.7 GHz; 47.7-47.8 GHz; 47.8-47.9 GHz; 47.9-48.0 GHz; 48.0-48.1 GHz; and 48.1-48.2 GHz |
| --- | --- |
| Power limit (EIRP) | Base station: +75 dBm/100MHz Mobile station: +43 dBm Transportable station: +55 dBm |
| Unwanted emissions | +5 dBm/MHz (within 10% of channel bandwidth separation) +13 dBm/MHz (outside more than 10% of channel bandwidth apart) |

# 5	NR Frequency band definition

A summary of the new NR band n262 is provided in Table 5-1.

Table 5-1: New NR band in FR2

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n262 | 47.2 – 48.2 GHz | 47.2 – 48.2 GHz | TDD |

# 6	Channel numbering and channel bandwidth

BS channel bandwidth for FR2 has been defined in TS 38.104. They are captured to support at least 50 MHz as a minimum channel bandwidth and up to 400 MHz as the maximum channel bandwidth. In order to apply the same requirement for Band n262, the same bandwidths as other FR2 bands are proposed as shown in table 6-1.

Table 6-1: NR channel bandwidth in the frequency range between 47.2-48.2 GHz

| NR band |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band number | data SCS(kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

NR-ARFCN parameters for the global frequency raster are presented in TS 38.104, table 6-2:

Table 6-2: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

Using information above and the equation FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs), The NR-ARFCN that are applicable in the frequency range 47.2-48.2 GHz can be calculated as in table 6-3:

Table 6-3: Applicable NR-ARFCN in the frequency range between 47.2-48.2 GHz

| NR Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n262 | 60 | 2399166 – <1> – 2415832 |
|  | 120 | 2399167 – <2> – 2415831 |

The synchronization raster in the frequency range between 47.2-48.2 GHz is given in Table 6-4. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 6-4 with the step size interval of 17.28 MHz.

Table 7-4: Applicable SS raster entries in the frequency range between 39.5-43.5 GHz

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  | 240 kHz | Case E | 23588 – <2> – 23640 |
| NOTE: SS Block pattern is defined in subclause 4.1 in TS 38.213. |  |  |  |

<End of the change>

<Start of the change>

## 8.2	BS specific

### 8.2.1	Band agnostic requirements

The BS RF requirements summarized in Table 8.2.1-2 is band agnostic RF requirements for FR2 which are applicable for band n262 as well.

Table 8.2.1-2: Summary on band agnostic of BS RF requirements for FR2

| BS TX side capture in TS 38.104 | BS RX side capture in TS 38.104 |
| --- | --- |
| 9.2 Radiated transmit power | 10.3 OTA reference sensitivity level |
| 9.3 OTA Base station output power | 10.5 OTA In-band selectivity and blocking |
| 9.4 OTA Output power dynamics | 10.7 OTA Receiver spurious emissions |
| 9.5 OTA Transmit ON/OFF power | 10.9 OTA In-channel selectivity |
| 9.6 OTA Transmitted signal quality |  |
| 9.7.2 OTA Occupied bandwidth |  |
| 9.7.5 OTA Transmitter spurious emissions |  |

### 8.2.2	Transmitter characteristics

#### 8.2.2.1	Adjacent Channel Leakage Ratio (ACLR)

The BS OTA ACLR limit for spectrum range 37 – 52.6 GHz has been defined in TS 38.104. This is also applicable for Band n262.

#### 8.2.2.2	OTA operating band unwanted emissions

The BS OTA operating band unwanted emission for spectrum range 37 – 52.6 GHz has been specified in TS 38.104, section 9.7. Those limits are applicable for Band n262.

#### 8.2.2.3	Step frequencies for Tx spurious emission

The band n262 shall be added to Table 9.7.5.3.2.3-2 in TS 38.104 as proposed below.

Table 9.7.5.3.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |

#### 8.2.2.4  Measurement uncertainty and test tolerance

8.2.3 Receiver characteristics

#### 8.2.3.1  Step frequencies for Rx spurious emission

The band n262 shall be added to Table 10.7.3-2 in TS 38.104 as proposed below.

Table 10.7.3-2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23,5 | 35,5 | 38 | 45 | 47,5 | 59,5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |

<End of the change>