3GPP TSG-RAN WG4 Meeting #97-e	R4-2016028

# e-Meeting, 2nd - 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.809 | CR | 6028 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TR38.809: IAB RRM general |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR IAB-Core |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding general descriptions is to summarize the meeting agreements as Rel-16 RAN4 conclusions and the reference for future release IAB RRM requirement standardization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces general description on IAB-MT RRM requirement in TR38.809. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirement related part for Rel-16 IAB is missing in the IAB TR 38.809. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 11.2 11.3 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

11	   IAB RRM requirements

11.1	General

In Rel-16, the WI on IAB is considered for physically fixed deployment only. In light of the characteristic of Rel-16 IAB (e.g. fixed operation) and the differences between UE and IAB-MT (e.g. power saving consideration and seeking for higher throughput), selected RRM requirements in TS 38.133 are considered fundamentals and defined for Rel-16 IAB. Based on this background the RRM requirements defined for Rel-16 IAB are agreed as table 11-1.

Table 11-1: Summary on RRM requirement applicability for IAB-MT

| Requirement | Items | Comments | Applicability |
| --- | --- | --- | --- |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | To make sure IAB can remain RRC connection after link failure with original parent node. | Apply for both Local Area and Wide Area IAB-MT. |
| Radio Link Monitoring | - Requirements for radio link monitoring | Feature to maintain PCell’s MT link with parent node. | Apply for Local Area IAB-MT only. Requirements are defined for no-DRX only. |
| Link Recovery Procedures | - Requirements for beam failure detection - Requirements for candidate beam detection | Feature to maintain PCell’s MT beam management with parent node. | Apply for Local Area IAB-MT only. Requirements are defined for no-DRX only. |
| MT Timing related requirements | - MT transmit timing - Timing Advance | The same as UE related requirement which is reused for IAB-MT | Apply for both Local Area and Wide Area IAB-MT. |
| DU Timing related requirements | - Cell phase synchronization accuracy | Reuse the requirement 3 µs in TS38.133 | Apply for IAB-DU. |

For other UE or BS RRM requirements in TS38.133, including handover, interruptions, (de)activation delay, addition/release delay, active BWP switching, measurement related requirements and so forth, are not defined or not decided for IAB in Rel-16 due to various reasons such as fixed IAB operation where the link remains comparatively stable for longer periods of time. The requirements which are not defined not necessarily indicate the corresponding features are not supported by IAB.

11.2 RRC_CONNECTED state mobility for IAB-MTs

Detailed structure of the subclause is TBD.

11.3 Timing

Detailed structure of the subclause is TBD.

11.4 Signalling Characteristics for IAB-MTs

Detailed structure of the subclause is TBD.

<End of Change>