3GPP TSG-RAN WG4 Meeting #97-e		R4-2016094

Electronic Meeting, 2nd – 13th November 2020

Agenda item:	7.10.1.3

Source:	Ericsson

Title:	Simulation results for FR2 256QAM UE CQI performance requirements

Document for:	Information

# Introduction

In this paper, we present our simulation results on UE CQI requirement tests for FR2 DL 256QAM, based on the WF from RAN4#96-e [1].

# Discussion

## Simulation assumptions

The simulations are based on the assumptions found in WF.

Table 1 Simulation assumptions

- Whether to define FR2 CQI reporting requirements for CQI table 2 - In the next meeting, decide whether to introduce the requirements for AWGN and/or fading conditions based on the simulation results under 50MHz CBW: - For AWGN condition, companies are encouraged to simulate the required SNR for achieving median CQI of [11, 12 and 13] in CQI table 2. - For fading condition, companies are encouraged to simulate the required SNR where CQI indices corresponding to 256QAM (i.e., 12 and higher) in CQI table 2 can be reported with at least [10%] probability. - If it is agreed to define FR2 CQI reporting test for CQI table 2, use channel bandwidth of 50MHz. - If it is agreed to define FR2 CQI reporting for CQI table 2, consider the following test applicability: - If UE passes the test with CQI Table 2, then it can skip the corresponding test with CQI Table 1 - Note: Similar to the existing CQI table 1 test, the test for CQI Table 2 will cover both lower SNR and higher SNR.

## Simulation results

Simulation results for AWGN and TDLA channel conditions for 50MHz CBW are given below based on the above simulation assumptions and 38.101-4Table 8.2.2.2.1.1-1 and Table 8.2.2.2.2.1-1.

### Simulation results with AWGN channel condition

Simulation results with AWGN channel condition are summarized in Table 1 below.

Table 1 CQI reporting results with AWGN channel condition

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -10 | 0 | 1.0000 | 100 | 1.0000 | 1.0000 | 1.0000 |
| -9 | 0 | 0.9993 | 100 | 0.9993 | 0.9993 | 0.9993 |
| -8 | 0 | 0.4385 | 100 | 0.4385 | 0.4385 | 0.4385 |
| -7 | 0 | 0.0586 | 100 | 0.0586 | 0.0586 | 0.0586 |
| -6 | 1 | 0.0001 | 100 | 0.0001 | 0.0001 | 0.1714 |
| -5 | 1 | 0 | 100 | 0 | 0 | 0.0084 |
| -4 | 2 | 0 | 100 | 0 | 0 | 0.9976 |
| -3 | 2 | 0 | 100 | 0 | 0 | 0.9974 |
| -2 | 2 | 0 | 100 | 0 | 0 | 0.1124 |
| -1 | 3 | 0 | 100 | 0 | 0 | 0.9976 |
| 0 | 3 | 0 | 100 | 0 | 0 | 0.9976 |
| 1 | 3 | 0 | 100 | 0 | 0 | 0.6711 |
| 2 | 3 | 0 | 100 | 0 | 0 | 0.0011 |
| 3 | 4 | 0 | 100 | 0 | 0 | 0.3911 |
| 4 | 5 | 0.0001 | 100 | 0 | 0.0001 | 0.9976 |
| 5 | 5 | 0 | 100 | 0 | 0 | 0.9921 |
| 6 | 6 | 0.0052 | 100 | 0 | 0.0052 | 0.9976 |
| 7 | 6 | 0 | 100 | 0 | 0 | 0.0395 |
| 8 | 7 | 0 | 100 | 0 | 0 | 0.9976 |
| 9 | 7 | 0 | 100 | 0 | 0 | 0.9643 |
| 10 | 8 | 0.0024 | 100 | 0 | 0.0024 | 0.9963 |
| 11 | 8 | 0 | 100 | 0 | 0 | 0.0108 |
| 12 | 9 | 0 | 100 | 0 | 0 | 0.9976 |
| 13 | 9 | 0 | 100 | 0 | 0 | 0.2381 |
| 14 | 10 | 0 | 100 | 0 | 0 | 0.9976 |
| 15 | 10 | 0 | 100 | 0 | 0 | 0.6422 |
| 16 | 11 | 0.0001 | 100 | 0 | 0.0001 | 0.9976 |
| 17 | 12 | 0.1079 | 100 | 0 | 0.1079 | 0.9976 |
| 18 | 12 | 0 | 100 | 0 | 0 | 0.9976 |
| 19 | 13 | 0.0960 | 100 | 0 | 0.0960 | 0.9976 |
| 20 | 13 | 0 | 100 | 0 | 0 | 0.9972 |
| 21 | 13 | 0 | 100 | 0 | 0 | 0.0178 |
| 22 | 14 | 0 | 100 | 0 | 0 | 0.0027 |
| 23 | 14 | 0 | 100 | 0 | 0 | 0 |
| 24 | 15 | 0 | 100 | 0 | 0 | 0 |
| 25 | 15 | 0 | 100 | 0 | 0 | 0 |
| 26 | 15 | 0 | 100 | 0 | 0 | 0 |
| 27 | 15 | 0 | 100 | 0 | 0 | 0 |
| 28 | 15 | 0 | 100 | 0 | 0 | 0 |
| 29 | 15 | 0 | 100 | 0 | 0 | 0 |
| 30 | 15 | 0 | 100 | 0 | 0 | 0 |

### Simulation results with fading channel condition

Simulation results with TDLA30-35 channel condition are summarized in Table 2 below.

Table 2 CQI reporting results with TDLA30-35 channel condition

| SNR [dB] | Median CQI Index | Percentage of Reported CQI Index not in {median CQI -1, median CQ1 +1} (%) | BLER with following CQI | BLER with Median CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- | --- | --- |
| -10 | 0 | 12.9445 | 0.5443 | 0.5346 | 1.1420 |
| -9 | 0 | 22.4930 | 0.4037 | 0.3904 | 1.2074 |
| -8 | 1 | 1.5581 | 0.2763 | 0.2540 | 1.2652 |
| -7 | 1 | 4.8342 | 0.1855 | 0.1574 | 1.3801 |
| -6 | 2 | 18.5378 | 0.1228 | 0.2428 | 1.1734 |
| -5 | 2 | 11.6260 | 0.0904 | 0.1521 | 1.2780 |
| -4 | 2 | 9.7483 | 0.0706 | 0.0819 | 1.4037 |
| -3 | 2 | 13.1842 | 0.0689 | 0.0430 | 1.5897 |
| -2 | 3 | 9.2289 | 0.0754 | 0.3215 | 1.1678 |
| -1 | 3 | 13.7036 | 0.0782 | 0.2034 | 1.1993 |
| 0 | 3 | 19.9760 | 0.0951 | 0.1207 | 1.2676 |
| 1 | 4 | 21.7739 | 0.1081 | 0.3618 | 1.1850 |
| 2 | 4 | 24.1310 | 0.1154 | 0.2305 | 1.1466 |
| 3 | 5 | 34.6784 | 0.1307 | 0.3909 | 1.2668 |
| 4 | 5 | 34.9181 | 0.1321 | 0.2544 | 1.1685 |
| 5 | 5 | 38.5937 | 0.1578 | 0.1581 | 1.1340 |
| 6 | 6 | 34.3188 | 0.1662 | 0.3611 | 1.2407 |
| 7 | 7 | 41.7099 | 0.1706 | 0.3416 | 1.2443 |
| 8 | 7 | 40.6312 | 0.1706 | 0.2153 | 1.1434 |
| 9 | 8 | 31.5621 | 0.1847 | 0.4568 | 1.3471 |
| 10 | 8 | 31.5222 | 0.1867 | 0.3288 | 1.1819 |
| 11 | 9 | 32.0416 | 0.2001 | 0.3656 | 1.2161 |
| 12 | 9 | 30.9229 | 0.1999 | 0.2373 | 1.0867 |
| 13 | 9 | 33.5997 | 0.2172 | 0.1486 | 1.0149 |
| 14 | 10 | 36.1566 | 0.2167 | 0.3614 | 1.2011 |
| 15 | 10 | 40.8710 | 0.2331 | 0.2446 | 1.0607 |
| 16 | 11 | 39.9121 | 0.2336 | 0.3266 | 1.1301 |
| 17 | 12 | 41.9497 | 0.2248 | 0.4378 | 1.3099 |
| 18 | 12 | 39.8322 | 0.2245 | 0.3161 | 1.1389 |
| 19 | 13 | 26.2885 | 0.2132 | 0.4649 | 1.3875 |
| 20 | 13 | 23.6117 | 0.1934 | 0.3481 | 1.2164 |
| 21 | 13 | 24.6504 | 0.1723 | 0.2329 | 1.0997 |
| 22 | 14 | 12.3851 | 0.1500 | 0.3769 | 1.3112 |
| 23 | 14 | 6.6320 | 0.1239 | 0.2686 | 1.1764 |
| 24 | 14 | 3.7954 | 0.0918 | 0.1727 | 1.0940 |
| 25 | 15 | 11.4662 | 0.0717 | 0.1812 | 1.1079 |
| 26 | 15 | 6.0328 | 0.0460 | 0.1132 | 1.0596 |
| 27 | 15 | 3.5557 | 0.0277 | 0.0644 | 1.0290 |
| 28 | 15 | 1.5182 | 0.0174 | 0.0358 | 1.0136 |
| 29 | 15 | 0.3995 | 0.0106 | 0.0179 | 1.0045 |
| 30 | 15 | 0.1199 | 0.0050 | 0.0064 | 0.9998 |

# Conclusion

In this paper we’ve provided our simulation results for FR2 256QAM CQI requirements based on agreed WF.

# References

- R4-2012666, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom