3GPP TSG-RAN WG4 Meeting #97-e	    	                          R4-2016101

Electronic Meeting, November 2nd – 13th, 2020

Agenda item:	7.9.4.3

Source:	Ericsson

Title:	Evaluations of Rel-16 Type II PMI testing

Document for:	Information

# Introduction

In this paper we provide simulation results for CSI – PMI testing for Rel-16 Type II codebook.

# Results

## SU-MIMO FDD 10MHz/15kHz

| Codebook | Antenna configuration | correlation | Subband Size | SNR [dB 90% max TP] | Gain [TypeII/TypeI] | Gain [Follow/Random] | SNR [dB 70% max TP] | Gain [TypeII/TypeI] | Gain [Follow/Random] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-16 Type II | 16Tx2Rx | Medium | 4 | 11.32 | 1.12 | 2.04 | 5.65 | 1.32 | 3.81 |
|  |  |  | 8 | 11.79 | 1.08 | 1.97 | 5.55 | 1.32 | 4.36 |
|  |  | High | 4 | 9.97 | 1.06 | 2.68 | 6.33 | 1.06 | 5.78 |
|  |  |  | 8 | 10.05 | 1.06 | 2.67 | 6.27 | 1.07 | 6.3 |
|  |  | Custom | 4 | 11 | 1.33 | 2.06 | 6.51 | 1.57 | 3.07 |
|  |  |  | 8 | 10.68 | 1.37 | 2.01 | 6.24 | 1.62 | 2.97 |
|  | 16Tx4Rx | Medium | 4 | 5.24 | 1.16 | 2.67 | 2.91 | 1.26 | 3.94 |
|  |  |  | 8 | 5.41 | 1.12 | 2.4 | 2.92 | 1.24 | 3.37 |
|  |  | High | 4 | 5.82 | 0.98 | 4.62 | 3.51 | 0.99 | 7.6 |
|  |  |  | 8 | 5.61 | 1.01 | 4.06 | 3.07 | 1.04 | 5.4 |
|  |  | Custom | 4 | 7.68 | 1.19 | 2.34 | 3.83 | 1.4 | 3.59 |
|  |  |  | 8 | 7.88 | 1.16 | 2.1 | 3.7 | 1.4 | 4.39 |

## SU-MIMO TDD 40MHz/30kHz

| Codebook | Antenna configuration | correlation | Subband Size | SNR [dB 90% max TP] | Gain [TypeII/TypeI] | Gain [Follow/Random] | SNR [dB 70% max TP] | Gain [TypeII/TypeI] | Gain [Follow/Random] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-16 Type II | 16Tx2Rx | Medium | 8 | 8.03 | 1.51 | 2.9 | 4.9 | 1.51 | 3.7 |
|  |  |  | 16 | 8.8 | 1.38 | 2.49 | 5.26 | 1.48 | 4.17 |
|  |  | High | 8 | 10.61 | 1.14 | 2.54 | 7.87 | 1.13 | 2.88 |
|  |  |  | 16 | 10.51 | 1.16 | 2.81 | 7.63 | 1.15 | 4.23 |
|  |  | Custom | 8 | 7.6 | 1.88 | 2.64 | 4.96 | 1.61 | 2.65 |
|  |  |  | 16 | 7.85 | 1.87 | 2.77 | 5.56 | 1.54 | 2.81 |
|  | 16Tx4Rx | Medium | 8 | 3.49 | 1.45 | 2.58 | 1.72 | 1.4 | 2.49 |
|  |  |  | 16 | 3.72 | 1.41 | 2.61 | 2.05 | 1.39 | 2.74 |
|  |  | High | 8 | 3.59 | 1.12 | 4.35 | 1.64 | 1.11 | 7.31 |
|  |  |  | 16 | 3.37 | 1.15 | 4.08 | 1.37 | 1.14 | 5.39 |
|  |  | Custom | 8 | 5.27 | 1.61 | 2.35 | 2.27 | 1.49 | 2.52 |
|  |  |  | 16 | 5.3 | 1.6 | 2.32 | 2.29 | 1.49 | 2.43 |

## MU-MIMO FDD 10MHz/15kHz

### 16Tx

| Codebook | Antenna configuration | correlation | Subband Size | SNR [dB 90% max TP] | Gain [TypeII/TypeI] |
| --- | --- | --- | --- | --- | --- |
| Rel-16 Type II | 16Tx2Rx | Medium | 4 | -2.81 | 1.17 |
|  |  | Custom | 4 | -2 | 1.31 |
|  | 16Tx4Rx | Medium | 4 | -6.07 | 1.02 |
|  |  | Custom | 4 | -3.95 | 1.34 |

## MU-MIMO TDD 40MHz/30kHz

### 16Tx ports

| Codebook | Antenna configuration | correlation | Subband Size | SNR [dB 90% max TP] | Gain [TypeII/TypeI] |
| --- | --- | --- | --- | --- | --- |
| Rel-16 Type II | 16Tx2Rx | Medium | 8 | -2.94 | 1.01 |
|  |  | Custom | 8 | -1.1 | 1.14 |
|  | 16Tx4Rx | Medium | 8 | -5.58 | 0.98 |
|  |  | Custom | 8 | -4.18 | 1.08 |

# Conclusions

In this contribution we’ve extensively simulated with the SU-MIMO setup, and our novel MU-MIMO test setup for Type II PMI testing and provide our simulation results

# References

- R4-2012761 “Way forward on PMI reporting requirements for eMIMO”, Qualcomm, Ericsson, RAN4 #96e, August 2020.