3GPP TSG-RAN WG4 Meeting #97-e	 R4-2016104

Electronic Meeting, 2nd – 13th November, 2020

Agenda item:	7.8.1.2.1

Source:	Ericsson

Title:	Simulation results on UE URLLC demodulation performance requirements with higher BLER

Document for:	Information

# Introduction

In the previous RAN4 e-meetings, it’s agreed to define URLLC UE demodulation performance test cases with higher BLER requirements for PDSCH slot aggregation for both FR1 and FR2, PDSCH mapping type B and processing capability 2 for FR1,  PDSCH mapping type B for FR2, and PDSCH pre-emption for FR1. In this paper we present our simulation results for these functionality tests respectively.

# Simulation results

## Slot aggregation

### Simulation assumptions and results for FR1

Simulation results on FR1 PDSCH slot aggregation are presented below for MCS13/16/19, based on the following simulation assumptions agreed in [1] and [2]:

- TDD pattern: 7D1S2U, S=6D: 4G: 4U for 30 kHz SCS.

- Propagation condition: TDLA30-10

- SCS & CBW:

  - FDD: 15 kHz & 10 MHz

  - TDD: 30 kHz & 40 MHz

- PDSCH configuration: Mapping type A, symbol length 12, starting symbol 2.

- Antenna configuration: 2x2 and 2x4, ULA low

- Target BLER: 1%

- Target Confidence level: 99%

- BLER is calculated after all transmission

- Max number of HARQ transmissions: 4

- PDSCH aggregation level

  - 2 for FDD and TDD

- Scheduling for PDSCH:

  - FDD: No scheduling in slots 0 and 1 (or 19) within 20ms.

  - TDD: No scheduling in D slots i, where mod(i, 10) = 0, and S slots

- TRS configuration: 20 ms periodicity, 2 slots, Offset 10 ms

- SSB configuration: Periodicity 20 ms, Allocated in first slot within 20ms

- PRB bundling: 2

- Precoding model: Random Precoding, per slot, PRB bundling granularity (codebook configuration Single panel Type 1)

- Receiver type: MMSE-IRC

- Methodology for MCS selection​: Higher or equal to -4 dB for final 4 Rx requirement definition (average ideal SNR alignment result + IM)​

- BLER calculation method​: BLER = NpacketFail/NpacketTx, where NpacketFail is the number of packets with CRC fail after all transmissions (initial and retransmissions), NpacketTx is the total number of packets transmitted during the test.

Figure 1 SNR - BLER curve for FR1 FDD

Figure 2 SNR - BLER curve for FR1 TDD

Table 1 SNR at 1% BLER target for FR1 FDD

| MCS | SNR @ 1% BLER mark [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2Rx (ideal) | 2Rx (impairment) | 4Rx (ideal) | 4Rx (impairment) |
| 13 | -3.66 | -2.66 | -8.34 | -7.34 |
| 16 | -1.37 | -0.37 | -6.24 | -5.24 |
| 19 | 0.17 | 1.17 | -4.47 | -3.47 |

Table 2 SNR at 1% BLER target for FR1 TDD

| MCS | SNR @ 1% BLER mark [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2Rx (ideal) | 2Rx (impairment) | 4Rx (ideal) | 4Rx (impairment) |
| 13 | -3.84 | -2.84 | -8.41 | -7.41 |
| 16 | -1.36 | -0.36 | -6.21 | -5.21 |
| 19 | 0.41 | 1.41 | -4.34 | -3.34 |

### Simulation assumptions and results for FR2

We have simulated with the following simulation assumptions for FR2 PDSCH slot aggregation.

- TDD pattern: DDSU

- AL = 2

- Propagation condition: TDLA30-75, TDLA30-300

- SCS & CBW

  - 120kHz & 100MHz

- MCS: {13, 16, 19} from table 3

- PDSCH configuration: Mapping type A, symbol length 13, starting symbol 1.

- DMRS configuration: Type 1, 1 additional DMRS, Single symbol

- Antenna configuration 2x2, ULA low

- Target BLER 1%

- Scheduling for PDSCH:

  - No scheduling in D slot i, where mod(i,160) = 0 and mod(i, 160) = 1, and S slots

- PTRS on

- Overhead for TBS determination: 6

Figure 3 SNR - BLER curve for FR2 TDD

Table 3 SNR at 1% BLER target for FR2 TDD

| Channel Model | MCS | SNR @ 1% BLER mark [dB] |  |
| --- | --- | --- | --- |
|  |  | (ideal) | (impairment) |
| TDLA30-75 | 13 | -5.44 | -4.44 |
|  | 16 | -3.27 | -2.27 |
|  | 19 | -1.62 | -0.62 |
| TDLA30-300 | 13 | -6.63 | -5.63 |
|  | 16 | -4.4 | -3.4 |
|  | 19 | -3.01 | -2.01 |

## PDSCH mapping type B and processing capability 2

### Simulation assumptions and results for FR1

Our simulation results are based on the following simulation assumptions agreed in [1] and [2]:

- Slots to be scheduled:

  - FDD: All DL slots with K1=0

  - TDD: S slots with K1=0

- Starting symbol: 2

- Symbol length: 2

- Slot aggregation level: 1

- Max number of HARQ transmissions: 1

- Verify PDSCH processing capability 2 and type B mapping together

- SCS/CBW:

  - FDD: 15 KHz/10 MHz

  - TDD: 30 KHz/ 40 MHz

- TDD pattern (30KHz SCS): DDDSU, S=10:2:2

- Number of HARQ process: 2 for FDD and TDD

- Channel model: TDLA30-10

- Antenna configuration: 2x2 and 2x4, ULA Low.

- MCS: MCS 4

- Number of RBs: Full bandwidth

- PDSCH symbol length for FDD and TDD: 2os

- Test metrics: 70% throughput

- SSB configuration: Periodicity 20 ms, Allocated in first slot within 20ms

- PRB bundling: 2

- Precoding model: Random Precoding, per slot, PRB bundling granularity (codebook configuration Single panel Type 1)

- Receiver type: MMSE-I

Figure 4 Throughput curve for FR1 FDD

Figure 5 Throughput curve for FR1 TDD

Table 4 70% Throughput results for FR1 FDD

| MCS | SNR @ 70% TP mark [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2Rx (ideal) | 2Rx (impairment) | 4Rx (ideal) | 4Rx (impairment) |
| 4 | -0.82 | 0.18 | -4.11 | -3.11 |

Table 5 70% Throughput results for FR1 TDD

| MCS | SNR @ 70% TP mark [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2Rx (ideal) | 2Rx (impairment) | 4Rx (ideal) | 4Rx (impairment) |
| 4 | -1.02 | -0.02 | -4.5 | -3.5 |

### Simulation assumptions and results for FR2

Our simulation is based on the following simulation assumptions agreed in [1]:

- SCS/CBW: 120 kHz/100 MHz

- TDD pattern: DDDSU with S = 10D:2G:2U

- Scheduling: No PDSCH in slot 0 within 20 ms

- MCS: {MCS4} from table 1.

- Max number of HARQ transmissions: 1

- Number of HARQ process: 8

- Antenna configuration: 2x2, ULA low

- Channel model:

  - Option 1: TDLA30-75

- PTRS on

- Overhead for TBS determination: 6

- Test metrics: 70% throughput

- PDSCH Configuration: Start symbol 1, Symbol length 7

- DMRS configuration: Type 1, 1 additional DMRS, Single symbol

Furthermore, for the PTRS configurations, we propose to use the same RAN4 PDSCH FR2 common test parameters:

- PTRS configuration: Frequency density (KPT-RS) 2, Time density (LPT-RS) 1, resource element offset 2

Figure 6 Throughput curve for FR2 TDD

Table 6 70% Throughput results for FR2 TDD

|  | SNR @ 70% TP mark [dB] |  |
| --- | --- | --- |
|  | ideal | impairment |
| MCS4 TDLA30-75 | -3.54 | -2.54 |

## PDSCH pre-emption

### Simulation assumptions and results for FR1

URLLC PDSCH pre-emption performance test is only defined for FR1. The simulations are based on the following simulation assumptions provided in [1] and [2]:

- SCS/CBW: 15kHz/10MHz for FDD and 30kHz/40Mhz for TDD

- TDD UL-DL pattern: 7D1S2U with S=6D: 4G: 4U

- Time frequency set: 14x1

- Number of symbols to be pre-empted: 2

- Starting symbol to be pre-empted: 3

- Aggregation level: 1

- Maximum number of HARQ transmissions: 4

- Maximum number of HARQ process: 4 for FDD and 8 for TDD

- Antenna Configuration: 2x2 and 2x4, ULA low

- DMRS configuration: Type 1, 1 additional DMRS, Single symbol

- Pre-emption probability

  - Option 1: 10% within 1 radio frame

  - Option 2: 20% within 1 radio frame

- Pre-emption sheduling: Fixed scheduling

- Number of pre-empted symbols: Only 2os

- eMBB MCS

  - Option 1: MCS13 in Table 1

  - Option 2: MCS4 in Table 1

- Test metric: 70% of max T-put

- TRS configuration: 20 ms periodicity, 2 slots, Offset 10 ms

- SSB configuration: Periodicity 20 ms, Allocated in first slot within 20ms

- PRB bundling: 2

- Precoding model: Random Precoding, per slot, PRB bundling granularity (codebook configuration Single panel Type 1)

- Receiver type: MMSE-IRC

- The assumption of UE behaviours for buffer flushing and decoding​: If UE cannot decode the PDCSH correctly, UE feeds back NACK to gNB. Then UE flushes the buffer and waits for the next re-transmission for LLR combing to decode the PDSCH. ​

- URLLC interference modelling​:

  - SNR: Same as for eMBB transmission​

  - Structure: Some random data

Figure 7 Throughput curve for FDD with no PI detection and HARQ buffer flushing

Figure 8 Throughput curve for FDD with HARQ buffer flushing

Table 7 SNR at 70% Throughput target for FDD

| MCS | Pre-emption probability | Number of Rx | SNR @ 70% throughput mark [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | No HARQ buffer Flushing | With HARQ buffer Flushing |
| 4 | 10% | 2Rx (ideal) | -2.28 | -2.27 |
|  |  | 2Rx (impairment) | -1.28 | -1.27 |
|  |  | 4Rx (ideal) | -6.22 | -6.22 |
|  |  | 4Rx (impairment) | -5.22 | -5.22 |
|  | 20% | 2Rx (ideal) | -1.84 | -1.86 |
|  |  | 2Rx (impairment) | -0.84 | -0.86 |
|  |  | 4Rx (ideal) | -5.81 | -5.82 |
|  |  | 4Rx (impairment) | -4.81 | -4.82 |
| 13 | 10% | 2Rx (ideal) | 4.72 | 4.43 |
|  |  | 2Rx (impairment) | 5.72 | 5.43 |
|  |  | 4Rx (ideal) | 0.92 | 0.48 |
|  |  | 4Rx (impairment) | 1.92 | 1.48 |
|  | 20% | 2Rx (ideal) | 4.96 | 4.69 |
|  |  | 2Rx (impairment) | 5.96 | 5.69 |
|  |  | 4Rx (ideal) | 1.13 | 0.74 |
|  |  | 4Rx (impairment) | 2.13 | 1.74 |

Figure 9 Throughput curve for TDD with no PI detection and HARQ buffer flushing

Figure 10 Throughput curve for TDD with HARQ buffer flushing

Table 8 SNR at 70% Throughput target for TDD

| MCS | Pre-emption probability | Number of Rx | SNR @ 70% throughput mark [dB] |  |
| --- | --- | --- | --- | --- |
|  |  |  | No HARQ buffer Flushing | With HARQ buffer Flushing |
| 4 | 10% | 2Rx (ideal) | -2.14 | -2.05 |
|  |  | 2Rx (impairment) | -1.14 | -1.05 |
|  |  | 4Rx (ideal) | -6.26 | -6.22 |
|  |  | 4Rx (impairment) | -5.26 | -5.22 |
|  | 20% | 2Rx (ideal) | -2.07 | -2.06 |
|  |  | 2Rx (impairment) | -1.07 | -1.06 |
|  |  | 4Rx (ideal) | -6.2 | -6.18 |
|  |  | 4Rx (impairment) | -5.2 | -5.18 |
| 13 | 10% | 2Rx (ideal) | 5.54 | 5.64 |
|  |  | 2Rx (impairment) | 6.54 | 6.64 |
|  |  | 4Rx (ideal) | 1.15 | 1.18 |
|  |  | 4Rx (impairment) | 2.15 | 2.18 |
|  | 20% | 2Rx (ideal) | 5.99 | 5.74 |
|  |  | 2Rx (impairment) | 6.99 | 6.74 |
|  |  | 4Rx (ideal) | 1.81 | 1.25 |
|  |  | 4Rx (impairment) | 2.81 | 2.25 |

# Conclusion

In this contribution, we’ve summarized simulation results for different UE URLLC features defined for FR1 and FR2.

# References

- R4-2012648, “Way forward for NR UE URLLC performance requirements”, Intel

- R4-2008808, “Simulation assumptions for NR URLLC UE performance requirements test cases”, Intel