3GPP TSG-RAN WG4 Meeting #97-e	R4-2016117

# E-meeting, 2nd Nov, 2020 – 13th Nov, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2: Introduction of 35MHz and 45MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_35MHz_45MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BS conformance testing requirements for 35MHz and 45MHz is missing in the existing TS 38.141-2, therefore corresponding RF requirements should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Summary the change: Tx: output dynamics , ACLR requirements Rx: REFSENS, dynamic range, ACS, IBB, narrowband blocking, receiver intermodulation, ICS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BS conformance testing requirements for 35MHz and 45MHz is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.5, 6.7.3.5,, 6.6.3.5, 7.2.5, 7.3.5, 7.4.5, 7.5.1.5, 7.5.2.5, 7.8.5, 7.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.104, 37.104, 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1, 37.145-1/2, TS 37.141 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6	Radiated transmitter characteristics

6.1	General

General test conditions for transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in clause 4.5.

If beams have been declared equivalent and parallel (D.13, D.14), only a representative beam is necessary to be tested to demonstrate conformance.

6.4	OTA output power dynamics

6.4.1	General

The requirements in clause 6.4 apply during the transmitter ON period. Transmit signal quality (as specified in clause 6.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are single direction requirements and apply to the beam peak directions over the OTA peak directions set.

6.4.3	OTA total power dynamic range

6.4.3.1	Definition and applicability

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE:	The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbols shall carry PDSCH and not contain PDCCH, RS or SSB.

6.4.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 9.4.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], clause 9.4.3.3.

6.4.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

6.4.3.4	Method of test

6.4.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Beams to be tested:	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

Directions to be tested: The OTA peak directions set reference beam direction pair (D.8).

6.4.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-FR1-TM3.1a in TS 38.141-1 [3] clause 4.9.2.2.6 if 256QAM is supported by BS without power back off;

-	or NR-FR1-TM3.1 in TS 38.141-1 [3] clause 4.9.2.2.5 if 256QAM is not supported by BS;

-	or NR-FR1-TM3.1 in TS 38.141-1 [3] clause 4.9.2.2.5 if 256QAM is supported by BS with power back off;

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model:

-	NR-FR2-TM3.1a if 256QAM is supported by BS without power back off, or

-	NR-FR2-TM3.1 if 256QAM is supported by BS with power back off, or 256QAM is not supported by BS; with 64QAM signals if 64QAM is supported by BS without power back off, or;

-	NR-FR2-TM3.1 with highest modulation order supported without power back off if 64QAM is not supported by BS, or;

-	NR-FR2-TM3.1with highest modulation order supported without power back off if 64QAM is supported by BS with power back off;

6)	Measure the OFDM symbol TX power as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-FR1-TM2a in TS 38.141-1 [3] clause 4.9.2.2.4 if 256QAM is supported by BS;

-	or NR-FR1-TM2 in TS 38.141-1 [3] clause 4.9.2.2.3 if 256QAM is not supported by BS;

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-FR2-TM2a if 256QAM is supported by BS, or;

-	NR-FR2-TM2 with highest modulation order supported if 256QAM is not supported by BS;

8)	Measure the OFDM symbol TX power (OSTP) as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS or SSB.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.4.3.5	Test requirement

6.4.3.5.1	BS type 1-O

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.3.5.1-1.

Table 6.4.3.5.1-1: Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 22.9 | 19.8 | 16.6 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

NOTE:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in clause 6.6.

6.7	OTA unwanted emissions

6.7.1	General

OTA unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [5]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the BS channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The OTA out-of-band emissions requirement for the BS type 1-O and BS type 2-O transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). The OTA Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. OTA Unwanted emissions outside of this frequency range are limited by an OTA spurious emissions requirement.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 6.7.1-1 for BS type 1-O and BS type 2-O for the NR operating bands.

Table 6.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low ≤ 4000 MHz | 1500 |

The OTA unwanted emission requirements are applied per cell for all the configurations. Requirements for OTA unwanted emissions are captured using TRP, directional requirements or co-location requirements as described per requirement.

There is in addition a requirement for OTA occupied bandwidth.

6.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

6.7.3.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

For both BS type 1-O and BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the OTA ACLR requirements in clause 6.7.3.2 apply to BS channel bandwidths of the outermost carrier.

For BS type 1-O, for a RIB operating in non-contiguous spectrum, the OTA ACLR requirements in clause 6.7.3.2 shall apply inside sub-block gaps for the frequency ranges defined in table 6.7.3.5.1-2a, while the CACLR requirement in clause 6.7.3.2 shall apply in sub block gaps for the frequency ranges defined in table 6.7.3.5.1-3. In addition, for a multi-band RIB, the ACLR requirement in clause 6.7.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1-2a, while the CACLR requirement in clause 6.7.3.2 shall apply in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1-3.

For BS type 2-O, for a RIB operating in non-contiguous spectrum, the OTA ACLR requirements in clause 6.7.3.2 shall apply inside any sub-block gap for the frequency ranges defined in table 6.7.3.5.2-3, while the CACLR requirement in clause 6.6.3.2 shall apply in sub block gaps for the frequency ranges defined in table 6.7.3.5.2-3.

The requirement shall be applied per RIB during the transmitter ON period.

6.7.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 9.7.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], clause 9.7.3.3.

6.7.3.3	Test purpose

To verify that the OTA adjacent channel leakage ratio requirement shall be met as specified by the minimum requirement.

6.7.3.4	Method of test

6.7.3.4.1	Initial conditions

Test environment:	normal; see annex B.2.

RF channels to be tested for single carrier:	B and T; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operaton, see clause 4.9.1.

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

6.7.3.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annexI. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 6, 8, 9, 10, 11, 12 and 13.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	The measurement devices characteristics shall be:

- measurement filter bandwidth: defined in clause 6.7.3.5.

- detection mode: true RMS voltage or true power averaging.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2 at manufacturers declared rated carrier output power (Prated,c,TRP).

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Measure the absolute power of the assigned channel frequency and the (adjacent channel frequency).

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel using the measurements made in Step 7.

9)	Calculate relative ACLR estimate.

NOTE 1:	ACLR is calculated by the ratio of the absolute TRP of the assigned channel frequency and the absolute TRP of the adjacent frequency channel.

NOTE 2:	For FR1 the measurement uncertainty of the reverberation chamber for the relative ACLR is higher than the measurement uncertainty in clause 4.1.2 the test requirements in table 6.7.3.5.1-1 shall be tightened following the procedure in clause 4.1.3.

10)	Measure OTA ACLR for the frequency offsets both side of channel frequency as specified in table 6.7.3.5.1-1 for BS type 1-O or table 6.7.3.5.2-1for BS type 2-O respectively. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

11)	For the OTA ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure OTA ACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

b)	Measure OTA CACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

12)	Repeat the test with the channel set-up using NR- FR1-TM1.2 defined in clause 4.9.2 in TS 38.141-1 [3] for BS type 1-O.

In addition, for multi-band RIB, the following steps shall apply:

13)	For BS type 1-O and multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.7.3.5	Test requirements

6.7.3.5.1	BS type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/2a or the OTA ACLR absolute limits in table 6.7.3.5.1-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.1-3 or the OTA CACLR absolute limits in table 6.7.3.5.1-3a shall apply, whichever is less stringent.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-3 and the filters on the assigned channels are defined in table 6.7.3.5.1-4.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-1.

Table 6.7.3.5.1-1: BS type 1-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1-2.

Table 6.7.3.5.1-2: BS type 1-O ACLR absolute limit

| BS category / BS class | OTA ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |
| NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void |  |

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-2a.

Table 6.7.3.5.1-2a: BS type 1-O ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3GHz) | OTA ACLR limit (3-6GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

The OTA CACLR measurement result shall not less than the OTA CACLR limit specified in table 6.7.3.5.1-3.

Table 6.7.3.5.1-3: BS type 1-O CACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit (0-3 GHz) | OTA CACLR limit (3-6 GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 | 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.1-3a.

Table 6.7.3.5.1-3a: BS type 1-O CACLR absolute limit

| BS category / BS class | OTA CACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |
| NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void |  |

Table 6.7.3.5.1-4: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

6.7.3.5.2	BS type 2-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.2-1/3 or the OTA ACLR absolute limits in table 6.7.3.5.2-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.2-4 or the OTA CACLR absolute limits in table 6.7.3.5.2-4a shall apply, whichever is less stringent.

The CACLR in a sub-block gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.2-4 and the filters on the assigned channels are defined in table 6.7.3.5.2-5.

The OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-1.

Table 6.7.3.5.2-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.2-2

Table 6.7.3.5.2-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide-area BS | -10.3dBm/MHz |
| Medium-range BS | -17.3 dBm/MHz |
| Local-area BS | -17.3 dBm/MHz |

For operation in non-contiguous spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.2-3.

Table 6.7.3.5.2-3: BS type 2-O ACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted (MHz) | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (MHz) |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | Wgap ≥ 100 (Note 5) Wgap ≥ 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| 200, 400 | Wgap ≥ 400 (Note 6) Wgap ≥ 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |

For operation in non-contiguous spectrum, the CACLR for carriers located on either side of the sub-block gap shall be less than the value specified in table 6.7.3.5.2-4.

Table 6.7.3.5.2-4: BS type 2-O CACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted (MHz) | Sub-block gap size (Wgap) where the limit applies (MHz) | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA CACLR limit (dB) |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | 50 ≤ Wgap < 100 (Note 5) 50 ≤ Wgap < 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| 200, 400 | 200 ≤ Wgap < 400 (Note 6) 200 ≤ Wgap < 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. |  |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.2-4a.

Table 6.7.3.5.2-4a: BS type 2-O CACLR absolute limit

| BS class | CACLR absolute limit |
| --- | --- |
| Wide area BS | -10.3 dBm/MHz |
| Medium range BS | -17.3 dBm/MHz |
| Local area BS | -17.3 dBm/MHz |

Table 6.7.3.5.2-5: Filter parameters for the assigned channel

| RAT of the carrier adjacent to the sub-block gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| NR | NR of same BW with SCS that provides largest transmission bandwidth configuration |

7	Radiated receiver characteristics

7.1	General

General test conditions for receiver tests are given in clause 4, including interpretation of measurement results and configurations for testing. BS configurations for the tests are defined in clause 4.5.

Unless otherwise stated, the following arrangements apply for radiated receiver characteristics requirements in clause 7:

-	Requirements apply during the BS receive period.

-	Requirements shall be met for any transmitter setting.

-	For FDD operation the requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements defined for the radiated receiver characteristics do not assume HARQ retransmissions.

-	When BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS, blocking and intermodulation characteristics, the negative offsets of the interfering signal apply relative to the lower Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap, and the positive offsets of the interfering signal apply relative to the upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap.

NOTE 1:	In normal operating condition the BS in FDD operation is configured to transmit and receive at the same time.

NOTE 2:	In normal operating condition the BS in TDD operation is configured to TX OFF power during receive period.

Each requirement, except OTA receiver spurious emissions, shall be met over the RoAoA specified.

For FR1 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for the reference direction.

And

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) (dB) for all other directions.

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

For FR2 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔFR2_REFSENS = -3 dB for the reference direction

and

ΔFR2_REFSENS = 0 dB for all other directions

7.2	OTA sensitivity

7.2.1	Definition and applicability

The OTA sensitivity requirement is based upon the declaration of one or more OTA sensitivity direction declarations (OSDD), related to a BS type 1-H and BS type 1-O receiver.

The BS type 1-H and BS type 1-O receiver may optionally be capable of redirecting/changing the receiver target by means of adjusting BS settings resulting in multiple sensitivity RoAoA. The sensitivity RoAoA resulting from the current BS settings is the active sensitivity RoAoA.

If the BS is capable of redirecting the receiver target related to the OSDD then the OSDD shall include:

-	BS channel bandwidth and declared minimum EIS level applicable to any active sensitivity RoAoA inside the receiver target redirection range in the OSDD.

-	A declared receiver target redirection range, describing all the angles of arrival that can be addressed for the OSDD through alternative settings in the BS.

-	Five declared sensitivity RoAoA comprising the conformance testing directions as detailed in TR 37. 941 [29].

-	The receiver target reference direction.

NOTE 1:	Some of the declared sensitivity RoAoA may coincide depending on the redirection capability.

NOTE 2:	In addition to the declared sensitivity RoAoA, several sensitivity RoAoA may be implicitly defined by the receiver target redirection range without being explicitly declared in the OSDD.

If the BS is not capable of redirecting the receiver target related to the OSDD, then the OSDD includes only:

-	BS channel bandwidth and declared minimum EIS level applicable to the sensitivity RoAoA in the OSDD.

-	One declared active sensitivity RoAoA.

-	The receiver target reference direction.

NOTE 3:	For BS without target redirection capability, the declared (fixed) sensitivity RoAoA is always the active sensitivity RoAoA.

The OTA sensitivity EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

7.2.2	Minimum requirement

For a received signal whose AoA of the incident wave is within the active sensitivity RoAoA of an OSDD, the error rate criterion as described in TS 38.104 [2] clause 7.2.2 shall be met when the level of the arriving signal is equal to the minimum EIS level in the respective declared set of EIS level and BS channel bandwidth.

7.2.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EIS level and the range of angles of arrival declared in the OSDD.

7.2.4	Method of test

7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Directions to be tested:

-	receiver target reference direction (D.31),

-	conformance test directions (D.33).

7.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 7.2.5.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in clause 7.2.5.

9)	Measure the throughput according to annex A.1 for each supported polarization.

10)	Repeat steps 3 to 9 for all OSDD(s) declared for the BS (D.23), and supported polarizations.

For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

7.2.5	Test requirements

7.2.5.1	General

The minimum EIS level is a declared figure (D.27, D.28) for each OSDD (D.23). The test requirement is calculated from the declared value offset by the EIS Test Tolerance specified in clause 4.1.

7.2.5.2	Test requirements for BS type 1-H and BS type 1-O

For each measured carrier, the throughput measured in step 9 of clause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5.2-1.

Table 7.2.5.2-1: EIS levels

| BS channel | Sub-carrier | Reference | OTA sensitivity level, EIS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 |  |  |  |
| 10, 15 | 30 | G-FR1-A1-2 |  |  |  |
| 10, 15 | 60 | G-FR1-A1-3 | Declared | Declared | Declared |
| 20, 25, 30, 35,40, 45,50 | 15 | G-FR1-A1-4 | minimum EIS | minimum EIS | minimum EIS |
| 20, 25, 30, 35,40, 45,50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | + 1.3 | + 1.4 | + 1.6 |
| 20, 25, 30, 35,40, 45,50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 |  |  |  |
| NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

7.2.5.3	Test requirements for BS type 2-O

There is no OTA sensitivity requirement for FR2, the OTA sensitivity is the same as the OTA reference sensitivity in clause 7.3.

7.3	OTA reference sensitivity level

7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

The OTA REFSENS EIS level declaration shall apply to each supported polarization, under the assumption of polarization match.

7.3.2	Minimum requirement

For BS type 1-O the minimum requirement is in TS 38.104 [2], clause 10.3.2.

For BS type 2-O the minimum requirement is in TS 38.104 [2], clause 10.3.3.

7.3.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

7.3.4	Method of test

7.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	B, M and T; see clause 4.9.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.54),

-	OTA REFSENS conformance test directions (D.55).

7.3.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 7.3.5.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in clause 7.3.5.

9)	Measure the throughput according to annex A.1 for each supported polarization.

10)	Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the BS (D.55), and supported polarizations.

For multi-band capable FR1 BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

7.3.5	Test requirements

7.3.5.1	General

The FR1 EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in clause 4.1.

7.3.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 9 of clause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in tables 7.3.5.2-1 to 7.3.5.2-3.

Table 7.3.5.2-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | OTA reference sensitivity level, EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS | -100.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS | -100.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS | -97.3 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS | -93.7 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS | -94 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS | -94.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS | -95.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS | -95.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS | -88.7 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS | -89 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS | -89.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-3: Local Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | OTA reference sensitivity level, EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS | -92.1 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS | -92.2 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS | -85.7 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS | -86 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS | -86.1 – ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

7.3.5.3	Test requirements for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz BS channel bandwidth it does not imply that BS has to support 50 MHz BS channel bandwidth.

For Wide Area BS, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Medium Range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For Local Area BS, EISREFSENS_50M is an integer value in the range -86 to -109 dBm. The specific value is declared by the vendor.

Table 7.3.5.3-1 FR2 OTA reference sensitivity requirement

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (annex A.1) | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 2.4 + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + 2.4 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in table 10.3.3-2. |  |  |  |

7.4	OTA dynamic range

7.4.1	Definition and applicability

The OTA dynamic range is a measure of the capability of the receiver unit to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

7.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 10.4.2.

7.4.3	Test purpose

The test purpose is to verify that at the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

7.4.4	Method of test

7.4.4.1	Initial conditions

Test environment: Normal: see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Directions to be tested: OTA REFSENS receiver target reference direction (D.54).

7.4.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.2.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

b)	Set the signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.4.5.2-1 to 7.4.5.2-3.

8)	Measure the throughput according to annex A.2 for each supported polarization.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

7.4.5	Test requirement

7.4.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA dynamic range Test Tolerance specified in clause 4.1.

7.4.5.2	Test requirements for BS type 1-O

For each measured carrier, the throughput measured in step 6 of clause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in tables 7.4.5.2-1 to 7.4.5.2-3.

Table 7.4.5.2-1: Wide Area BS dynamic range

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.7– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.6– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-2: Medium Range BS dynamic range

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.7– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.6– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -66.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.2-3: Local Area BS dynamic range

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.7 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -63.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

7.5	OTA in-band selectivity and blocking

7.5.1	OTA adjacent channel selectivity

7.5.1.1	Definition and applicability

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

7.5.1.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 10.5.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], clause 10.5.1.3.

7.5.1.3	Test purpose

The test purpose is to verify the ability of the BS receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

7.5.1.4	Method of test

7.5.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.9.1.

Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31),

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

7.5.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.1.4.1-1 for BS type 1-O and table 7.5.1.4.2-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 7.5.1.4.1-1 for BS type 1-O and table 7.5.1.4.2-1 for BS type 2-O.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclauses 4.7.2 and 4.8.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

7.5.1.5	Test requirement

7.5.1.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA ACS Test Tolerance specified in clause 4.1.

7.5.1.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 7.5.1.5.2-1 and table 7.5.1.5.2-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.2 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA ACS requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.5.1.5.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.5.1.5.2-1: OTA ACS requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) |  |  | Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | EISminSENS + 6 dB |  |  | Wide Area BS: -52 – ΔminSENS Medium Range BS: -47– ΔminSENS Local Area BS: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. |  |  |  |  |

Table 7.5.1.5.2-2: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

7.5.1.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 7.5.1.5.3-1 and table 7.5.1.5.3-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement shall apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.5.1.5.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.1.5.3-1: OTA ACS requirement for BS type 2-O

| BS channel bandwidth of the | Wanted signal mean power (dBm) |  | Interfering signal mean |
| --- | --- | --- | --- |
| lowest/highest carrier received (MHz) | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz | power (dBm) |
| 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 3) | EISREFSENS + 6 dB (Note 3) | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS (Note 1) EISREFSENS_50M + 26.7 + ΔFR2_REFSENS (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 3: EISREFSENS is specified in TS 38.104 [2], clause 10.3.3. |  |  |  |

Table 7.5.1.5.3-2: OTA ACS interferer frequency offset for BS type 2-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 | 50 MHz DFT-s-OFDM NR |
| 100 | ±24.31 | signal, 60 kHz SCS, 64 RBs |
| 200 | ±24.29 |  |
| 400 | ±24.31 |  |

7.5.2	OTA in-band blocking

7.5.2.1	Definition and applicability

The OTA in-band blocking characteristics is a measure of the receiver's ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one RB for narrowband blocking.

7.5.2.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], clause 10.5.2.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], clause 10.5.2.3.

7.5.2.3	Test purpose

The test purpose is to verify the ability of the BS receiver to withstand high-levels of in-band interference from unwanted signals at specified frequency offsets without undue degradation of its sensitivity.

7.5.2.4	Method of test

7.5.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Base Station RF Bandwidth edge position to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

Directions to be tested:

For BS type 1-O:

-	receiver target reference direction for the minSENS OSDD (D.31),

-	OTA REFSENS conformance test directions (D.55),

For BS type 2-O:

-	OTA REFSENS receiver target reference direction (D.54),

-	OTA REFSENS conformance test directions (D.55).

7.5.2.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the BS Antenna Array coordinate system reference point is as follows:

For general OTA blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in table 7.5.2.5.2-1 for BS type 1-O and table 7.5.2.5.3-1 for BS type 2-O. The interfering signal shall be swept with a step size of 1 MHz for BS type 1-O or as indicated in Table 7.5.2.4.2-1 for BS type 2-O starting from the minimum offset to the channel edges of the wanted signals.

Table 7.5.2.4.2-1: FR2 Interferer signal step size

| Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- |
| 50 | 15 |
| 100 | 30 |
| 200 | 60 |
| 400 | 60 |

For OTA narrowband blocking:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.5.2.5.2-2 for BS type 1-O.

b)	Set the signal generator for the interfering signal at the specified frequency offset from the wanted signal to transmit as specified in tables 7.5.2.5.2-2 and 7.5.2.5.2-3 for BS type 1-O. Set-up and sweep the interfering RB centre frequency offset to the channel edge of the wanted signal according to table 7.5.2.5.2-3.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in subclauses 4.7.2 and 4.8.

9)	Repeat steps 3 to 8 for all the specified measurement directions.

For multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

7.5.2.5	Test requirement

7.5.2.5.1	General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA in-band blocking Test Tolerance specified in clause 4.1.

7.5.2.5.2	Test requirements for BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted and OTA interfering signal specified in tables 7.5.2.5.2-1, table 7.5.2.5.2-2 and table 7.5.2.5.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.2 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-O the OTA in-band blocking requirement shall apply in the in-band blocking frequency range, which is defined within frequency range from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the FDD operating band, where the ΔfOOB for BS type 1-O is defined in table 7.5.2.5.2-0.

Table 7.5.2.5.2-0: ΔfOOB offset for NR operating bands in FR1

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 1-O | FUL_high – FUL_low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the OTA in-band blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.5.2.5.2-1 and 7.5.2.5.2-3.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA narrowband blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a multi-band RIBs, the OTA narrowband blocking requirements apply in the narrowband blocking frequency ranges for each supported operating band. The requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the interfering signal minimum offset in table 7.5.2.5.2-3.

Table 7.5.2.5.2-1: General OTA blocking requirement for BS type 1-O

| BS channel bandwidth of the | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Interfering signal centre frequency minimum offset | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- |
| lowest/highest carrier received (MHz) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | signal |
| 5, 10, 15, 20 | EISREFSENS + 6 dB (NOTE 2) |  |  | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS (NOTE 2) | ±7.5 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
|  | EISminSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -43 - ΔminSENS Medium Range BS: -38 - ΔminSENS Local Area BS: -35 - ΔminSENS (NOTE 3) |  |  |
| 25 ,30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB (NOTE 2) |  |  | Wide Area BS: -43 - ΔOTAREFSENS Medium Range BS: -38 - ΔOTAREFSENS Local Area BS: -35 - ΔOTAREFSENS (NOTE 2) | ±30 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
|  | EISminSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -43 - ΔminSENS Medium Range BS: -38 - ΔminSENS Local Area BS: -35 - ΔminSENS (NOTE 3) |  |  |
| NOTE 1: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |  |  |

Table 7.5.2.5.2-2: OTA narrowband blocking requirement for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA Wanted signal mean power (dBm) |  |  | OTA Interfering signal mean power (dBm) |
| --- | --- | --- | --- | --- |
|  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |  |
| 5, 10, 15, 20 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area BS: -49 - ΔminSENS Medium Range BS: -44 - ΔminSENS Local Area BS: -41 - ΔminSENS |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB (NOTE 3) |  |  | Wide Area BS: -49 - ΔOTAREFSENS Medium Range BS: -44 - ΔOTAREFSENS Local Area BS: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB (NOTE 4) |  |  | Wide Area BS: -49 - ΔminSENS Medium Range BS: -44 - ΔminSENS Local Area BS: -41 - ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. NOTE 3: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 4: This test requirement is only applied in the OTA minSENS receiver target reference direction. NOTE 5: 7.5 kHz shift is not applied to the wanted signal. |  |  |  |  |

Table 7.5.2.5.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350 + m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565 + m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block is positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

7.5.2.5.3	Test requirements for BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS type 2-O, the OTA wanted and OTA interfering signals are provided at RIB using the parameters in table 7.5.2.5.3-1 for general OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in clause 7.3.5.3 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.104 [2] annex D.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For BS type 2-O the OTA blocking requirement shall apply in the in-band blocking frequency range, which is defined within frequency range from FUL_low - ΔfOOB to FUL_high + ΔfOOB,where the ΔfOOB for BS type 2-O is defined in table 7.5.2.5.3-0.

Table 7.5.2.5.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 2-O | FUL_high – FUL_low ≤ 4000 MHz | 1500 |

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 7.5.2.5.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

Table 7.5.2.5.3-1: General OTA blocking requirement for BS type 2-O

| BS channel bandwidth of the | OTA wanted signal mean power (dBm) |  | OTA interfering signal mean | OTA interfering signal centre | Type of OTA interfering |
| --- | --- | --- | --- | --- | --- |
| lowest/highest carrier received (MHz) | 24.24 GHz < f ≤ 33.4 GHz | 37 GHz < f ≤ 52.6 GHz | power (dBm) | frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | signal |
| 50, 100, 200, 400 | EISREFSENS + 6 dB | EISREFSENS + 6 dB | EISREFSENS_50M + 33 + ΔFR2_REFSENS dB | ±75 | 50 MHz DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| NOTE: EISREFSENS and EISREFSENS_50M are given in TS 38.104 [2], clause 10.3.3. |  |  |  |  |  |

7.6	OTA out-of-band blocking

7.6.1	Definition and applicability

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

For the general OTA out-of-band blocking the requirement applies to the wanted signal for each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched for in-band frequencies and the polarization maintained for out-of-band frequencies.

7.6.2	Minimum requirement

The minimum requirement for BS type 1-O is defined in TS 38.104 [2], clause 10.6.2.

The minimum requirement for BS type 2-O is defined in TS 38.104 [2], clause 10.6.3.

7.6.3	Test purpose

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

7.6.4	Method of test

7.6.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW in single-band RIB, see clause 4.9.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band RIB, see clause 4.9.1.

In addition, for multi-band RIB:

-	For BRFBW_T'RFBW, blocking testing above the highest operating band may be omitted.

-	For B'RFBW_TRFBW, blocking testing below the lowest operating band may be omitted.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

7.6.4.2	Procedure

7.6.4.2.1	BS type 1-O procedure for out-of-band blocking

1)	Place BS and the test antenna(s) according to annex E.2.4.1.

2)	Align the BS and test antenna(s) according to the directions to be tested.

3)	Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

4)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

5)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

6)	Generate the wanted signal in receiver target reference direction, according to the applicable test configuration (see clause 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

7)	Configure the beam peak direction for the transmitter units associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

8)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

9)	The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range specified in clause 7.6.5.1.1.

10)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the clause 7.6.5, for the relevant carriers specified by the test configuration in clause 4.7 and 4.8.

11)	Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

12)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.6.4.2.2	BS type 1-O procedure for co-location blocking

1)	Place NR BS and CLTA as specified in clause 4.12.2.3.

2)	Several CLTA are required to cover the whole co-location blocking frequency ranges. The CLTA shall be selected according to clause 4.12.2.2.

3)	Align the NR BS and test antenna(s) according to the directions to be tested.

4)	Connect test antenna and CLTA to the measurement equipment as depicted in annex E.2.4.2.

5)	The NR BS receives the wanted signal in all supported polarizations, in the receiver target reference direction from the test antenna.

6)	The OTA co-location blocking interferer is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization.

7)	Generate the wanted signal in receiver target reference direction, all supported polarizations, from the test antenna, according to the applicable test configuration (see clause 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

8)	Configure the beam peak direction for the transmitter units associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

9)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1 and, when applicable, for co-location test requirements in table 7.6.5.1.2-1.

10)	The CW interfering signal shall be swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands releated to co-located systems (according to declaration D.43).

11)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the clause 7.6.5, for the relevant carriers specified by the test configuration in clause 4.7 and 4.8.

In addition, for multi-band RIB, the following steps shall apply:

12)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.6.4.2.3	BS type 2-O procedure for out-of-band blocking

1)	Place BS and the test antenna(s) according to annex E.2.4.1.

2)	Align the BS and test antenna(s) according to the directions to be tested.

3)	Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

4)	The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

5)	The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

6)	Generate the wanted signal, according to the applicable test configuration (see clause 4.7 and 4.8) using applicable reference measurement channel to the RIB, according to annex A.1.

7)	Configure the beam peak direction for the transmitter unit associated with the RIB under test according to the declared reference beam direction pair for the appropriate beam identifier with the carrier set-up and power allocation according to the applicable test configuration(s) (see clause 4.7 and 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

8)	Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.2.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

9)	The interfering signal shall be swept within the frequency range specified in table 7.6.5.2.1-1 with the step size specified in table 7.6.4.2.3-1.

10)	Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the clause 7.6.5, for the relevant carriers specified by the test configuration in clause 4.7 and 4.8.

Table 7.6.4.2.3-1: Interferer signal step size

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

11)	Repeat for all supported polarizations.

7.6.5	Test requirements

7.6.5.1	Requirement for BS type 1-O

The test requirement consists of general and co-location requirements.

7.6.5.1.1	General

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 7.6.5.1.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.2 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1.

For a multi-band RIB, the OTA out-of-band requirement shall apply for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to clause 7.4.2.2 in TS 38.104 [2] shall be excluded from the OTA out-of-band blocking requirement.

For BS type 1-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-O is defined in table 7.5.2.5.2-0.

Table 7.6.5.1.1-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (Note 1) | 0.36 V/m | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m; for example, 0.36 V/m is equivalent to 36 dBm at fixed distance of 30 m. |  |  |

7.6.5.1.2	Co-location requirement

This additional OTA out-of-band blocking requirement may be applied for the protection of BS receivers when NR, E-UTRA BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with a BS.

The interferer power level is specified at the CLTA conducted input(s) as the signal power per supported polarization.

For OTA wanted and OTA interfering signal provided at the RIB using the parameters in table 7.6.5.1.2-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.2 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1.

For BS type 1-O the OTA blocking requirement for co-location with BS in other frequency bands is applied for all operating bands for which co-location protection is provided.

Table 7.6.5.1.2-1: OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.2.1. NOTE 2: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOBimmediately outside any of the supported uplink operating band(s). NOTE 3: The specified interferer signal power level is applied to all supported CLTA input ports. |  |  |  |  |  |

7.6.5.2	Requirement for BS type 2-O

The test requirement consists of general requirements.

7.6.5.2.1	General requirement

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 7.6.5.2.1-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.3 in TS 38.104 [2] for each BS channel bandwidth and further specified in annex A.1.

For BS type 2-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL_low – ΔfOOB and from FUL_high + ΔfOOB up to min(2nd harmonic of the upper frequency edge of the operating band, 60 GHz). The ΔfOOB for BS type 2-O is defined in table 7.5.2.5.3-0.

Table 7.6.5.2.1-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW carrier |
| 12750 to FUL_low – ΔfOOB |  | 0.1 |  |
| FUL_high + ΔfOOB to min(2nd harmonic of the upper frequency edge of the operating band, 60000) |  | 0.1 |  |
| NOTE: EISREFSENS is given in TS 38.104 [2], clause 10.3.3. |  |  |  |

7.7	OTA receiver spurious emissions

7.7.1	Definition and applicability

The OTA RX spurious emission is the power of the emissions radiated from the antenna array from a receiver unit.

Unless otherwise stated, all requirements are measured as mean power.

The OTA receiver spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in clause 6.7.1. For some operating bands, the upper limit of the spurious range might be higher than 12.75 GHz in order to comply with the 5th harmonic limit of the uplink operating band, as specified in ITU-R recommendation SM.329 [5].

For multi-band RIB the above exclusion applies for each supported operating band.

The OTA receiver spurious emission limits for FR2 shall apply from 30 MHz to 2nd harmonic of the upper frequency edge of the uplink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in clause 6.7.1.

For a BS operating in FDD, OTA RX spurious emissions requirement do not apply as they are superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain.

For a BS operating in TDD, the OTA RX spurious emissions requirement shall apply during the transmitter OFF period only.

The metric used to capture OTA receiver spurious emissions for BS type 1-O and BS type 2-O is total radiated power (TRP), with the requirement defined at the RIB.

7.7.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], clause 10.7.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], clause 10.7.3.

7.7.3	Test purpose

The test purpose is to verify if the receiver radiated spurious emissions from the BS at the RIB are within the specified minimum requirements.

7.7.4	Method of test

7.7.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see clause 4.9.1:

-	For FR1:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in single-band operation, see clause 4.9.1:

-	For FR1:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz)

RF bandwidth positions to be tested in multi-band operation, see clause 4.9.1:

-	For FR1:

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

7.7.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 7.7.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the TDD BS to receive only.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-9 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

NOTE 1:	The TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For BS type 1-O and multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.7.5	Test requirement

7.7.5.1	Test requirement for BS type 1-O

For RX only multi-band RIB, the OTA receiver spurious emissions requirements are subject to exclusion zones in each supported operating band.

The power of any spurious emission shall not exceed the levels in table 7.7.5.1-1:

Table 7.7.5.1-1: General OTA BS receiver spurious emission limits for BS type 1-O

| Spurious frequency range | Test limits (Note 6, Note 8) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -36 + X dBm | 100 kHz | Note 1, Note 6 |
| 1 GHz – 6 GHz | -30 + X dBm | 1 MHz | Note 1, Note 2, Note 6 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 + X dBm | 1 MHz | Note 1, Note 2, Note 3, Note 6 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.7.1. For multi-band RIBs, the exclusion applies for all supported operating bands. NOTE 5: Void NOTE 6: X = 9 dB, unless stated differently in regional regulation. NOTE 7: Void NOTE 8: Additional limits may apply regionally. |  |  |  |

7.7.5.2	Test requirement for BS type 2-O

The power of any receiver spurious emission shall not exceed the limits in table 7.7.5.2-1.

Table 7.7.5.2-1: Radiated Rx spurious emission limits for BS type 2-O

| Spurious frequency range (Note 4) | Limit (Note 5) | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 min(2nd harmonic of the upper frequency edge of the UL operating band in GHz; 60 GHz) | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1. NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [19], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 7.7.5.2-2. NOTE 5: Additional limits may apply regionally. |  |  |  |

Table 7.7.5.2-2: Step frequencies for defining the radiated Rx spurious emission limits 
for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |

In addition, the following requirement may be applied for protection of EESS for BS operating in frequency range 24.25 – 27.5 GHz.

The power of any receiver spurious emission shall not exceed the limits in Table 7.7.5.2-3.

Table 7.7.5.2-3: Limits for protection of Earth Exploration Satellite Service

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm | 200 MHz | Note 1 |
| 23.6 – 24 GHz | -9 dBm | 200 MHz | Note 2 |
| NOTE 1: This limit applies to BS brought into use on or before 1 September 2027 and enters into force from January 1, 2021. NOTE 2: This limit applies to BS brought into use after 1 September 2027. |  |  |  |

7.8	OTA receiver intermodulation

7.8.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement is defined as a directional requirement at the RIB.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

7.8.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 10.8.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], clause 10.8.3.

7.8.3	Test purpose

To verify that the BS receiver dynamic range, the relative throughput shall fulfil the specified limit.

7.8.4	Method of test

7.8.4.1	Initial conditions

Test environment: Normal, annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW for single-band operation, see clause 4.9.1.

-	BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band operation, see clause 4.9.1.

Directions to be tested:

-	OTA REFSENS receiver target reference direction (D.54).

-	In addition, for BS type 1-O, receiver target reference direction (D.31).

7.8.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.6.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction of the BS according to declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band as the OTA REFSENS RoAoA or OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 7.8.5.1-1 (for general intermodulation) or 7.8.5.1-3 (for narrowband intermodulation) for BS type 1-O, or table 7.8.5.2-1 (for general intermodulation) for BS type 2-O.

b)	Set the signal generator for the interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.8.5.1-1 (for general intermodulation) or 7.8.5.1-3 (for narrowband intermodulation) for BS type 1-O, or table 7.8.5.2-1 (for general intermodulation) for BS type 2-O.

8)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.8.5.1-2 (for general intermodulation) or 7.8.5.1-4 (for narrowband intermodulation) for BS type 1-O, or table 7.8.5.2-2 (for general intermodulation) for BS type 2-O.

9)	Measure the throughput according to annex A.1 for each supported polarization, for multi-carrier and/or CA operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clause 4.7.

10)	Repeat for all the specified measurement directions and supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

11)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.8.5	Test requirement

7.8.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the FR1 OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB with the conditions specified in tables 7.8.5.1-1 and 7.8.5.1-2 for intermodulation performance and in tables 7.8.5.1-3 and 7.8.5.1-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 7.3.5.2-1, table 7.3.5.2-2 and table 7.3.5.2-3 for each BS channel bandwidth and further specified in annex A.1.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth ≤ 20 MHz, for which the subcarrier spacing of the interfering signal should be 30 kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement shall apply in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.

For multi-band RIBs, the narrowband intermodulation requirement shall apply in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 7.8.5.1-2 and 7.8.5.1-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

Table 7.8.5.1-1: General intermodulation requirement

| BS class | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB | -52 - ΔOTAREFSENS | See table 7.8.5.1-2 |
|  | EISminSENS + 6 dB | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB | -44 - ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depend on the BS class and on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. |  |  |  |

Table 7.8.5.1-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 35 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal, (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 45 | ±7.415 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal, (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: For the 15 kHz subcarrier spacing, the number of RB is 25. For the 30 kHz subcarrier spacing, the number of RB is 10. NOTE 2: For the 15 kHz subcarrier spacing, the number of RB is 100. For the 30 kHz subcarrier spacing, the number of RB is 50. For the 60 kHz subcarrier spacing, the number of RB is 24. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. |  |  |

Table 7.8.5.1-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (Note 1) | -52 - ΔOTAREFSENS | See table 7.8.5.1-4 |
|  | EISminSENS + 6 dB (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (Note 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -44 - ΔminSENS |  |
| NOTE: EISREFSENS and EISminSENS depends on the BS channel bandwidth as specified in TS 38.104 [2], clause 10.3.2 and 10.2.1. |  |  |  |

Table 7.8.5.1-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±360 | CW |
|  | ±1420 | 5MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 15 (Note 2) | ±380 | CW |
|  | ±1960 | 5MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 20 (Note 2) | ±390 | CW |
|  | ±2320 | 5MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 25 (Note 2) | ±325 | CW |
|  | ±2350 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 30 (Note 2) | ±335 | CW |
|  | ±2350 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 35 (Note 2) | ±345 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 40 (Note 2) | ±355 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 45 (Note 2) | ±365 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 50 (Note 2) | ±375 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 60 (Note 2) | ±395 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 70 (Note 2) | ±415 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 80 (Note 2) | ±435 | CW |
|  | ±2710 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 90 (Note 2) | ±365 | CW |
|  | ±2530 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 100 (Note 2) | ±385 | CW |
|  | ±2530 | 20MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

7.8.5.2	BS type 2-O

Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted signal at the assigned channel frequency and two OTA interfering signals provided at the RIB using the parameters in tables 7.8.5.2-1 and 7.8.5.2-2. All of the OTA test signals arrive from the same direction, and the requirement is valid if the signals arrive from any direction within the FR2 OTA REFSENS RoAoA. The reference measurement channel for the wanted signal is identified in table 7.3.5.3-1 for each BS channel bandwidth and further specified in annex A.1.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

Table 7.8.5.2-1: General intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Mean power of interfering signals (dBm) | Wanted signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS_50M + 25 + ΔFR2_REFSENS dB | EISREFSENS + 6dB | See table 7.8.5.2-2 |
| NOTE: EISREFSENS and EISREFSENS_50M are given in TS 38.104 [2], clause 10.3.3. |  |  |  |

Table 7.8.5.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | ±7.5 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 100 MHz | ±6.88 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 200 MHz | ±5.64 | CW |
|  | ±40 | 50MHz DFT-s-OFDM NR signal (Note) |
| 400 MHz | ±6.02 | CW |
|  | ±45 | 50MHz DFT-s-OFDM NR signal (Note) |
| NOTE: For the 60 kHz subcarrier spacing, the number of RB is 64. For the 120 kHz subcarrier spacing, the number of RB is 32. |  |  |

7.9	OTA in-channel selectivity

7.9.1	Definition and applicability

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex E in TS 38.141-1 [3] and shall be time aligned with the wanted signal.

7.9.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 10.9.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], clause 10.9.3.

7.9.3	Test purpose

The purpose of this test is to verify the BS receiver ability to suppress the IQ leakage.

7.9.4	Method of test

7.9.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Directions to be tested:

-	For BS type 1-O, receiver target reference direction (D.31),

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

7.9.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.7.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Align the BS with the test antenna in the declared direction to be tested.

4)	Align the BS to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the BS to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA or OSDD being tested according to the appropriate test configuration in subclauses 4.7 and 4.8.

7)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified as follows:

a)	Adjust the signal generator for the wanted signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of Medium Range BS class on one side of the FC.

For BS type 2-O, table 7.9.5.2-1 on one side of the FC.

b)	Adjust the signal generator for the interfering signal as specified in:

For BS type 1-O, table 7.9.5.1-1 for BS of Wide Area BS class, in table 7.9.5.1-2 for BS of Local Area BS class and in table 7.9.5.1-3 for BS of Medium Range BS class at opposite side of the FC and adjacent to the wanted signal.

For BS type 2-O, table 7.9.5.2-1 at opposite side of the FC and adjacent to the wanted signal.

8)	Measure throughput according to annex A.1 for each supported polarization.

9)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

10)	Repeat for all the specified measurement directions and supported polarizations.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.9.5	Test requirement

7.9.5.1	BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For BS type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.9.5.1-1 for Wide Area BS, in table 7.9.5.1-2 for Medium Range BS and in table 7.9.5.1-3 for Local Area BS.

Table 7.9.5.1-1: Wide Area BS in-channel selectivity

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -98.2-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS | -96.3-ΔminSENS | -77.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS | -89.9-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS | -98.9-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS | -96.4-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -71.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS | -95.8-ΔminSENS | -78.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS | -90.3-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-2: Medium Range BS in-channel selectivity

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -93.2-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS | -91.3-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS | -84.9-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS | -93.9-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS | -91.4-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS | -85.2-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS | -90.8-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS | -85.3-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.1-3: Local area BS in-channel selectivity

| BS channel bandwidth | Subcarrier spacing | Reference measurement | Wanted signal mean power (dBm) |  |  | Interfering signal mean | Type of interfering |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) | channel (annex A.1) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | power (dBm) | signal |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -90.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS | -88.3-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS | -81.9-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS | -90.9-ΔminSENS | -73.4 - ΔminSENS | DFT-s- NR signal, 30 kHz SCS, 5 RBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS | -88.4-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS | -82.2-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS | -87.8-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS | -82.3-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

7.9.5.2	BS type 2-O

For BS type 2-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.9.5.2-1.

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

Table 7.9.5.2-1: OTA in-channel selectivity requirement for BS type 2-O

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (annex A.1) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + 3.4 + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RBs |
| 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 6.4 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RBs |
| 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + 3.4 + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RBs |
| 100, 200, 400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 6.4 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [2]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in TS38.104 [2], clause 7.3.3. |  |  |  |  |  |